From: elg@cypress.com (Ed Grivna)					X3T10.1/95a145r0

To: ssa@dt.wdc.com

Date:	95-04-20 15:35:52 EDT

Subj:	transmission line concerns

Hello group,

I have completed my analysis and, after numerous discussions by voice/email I believe I can put our original fears to rest.  At my present level of understanding I agree with Phil Murfet that the SSA transmission lines are indeed differential transmission lines.  The key items here are:

1.	the driver uses a current steering mechanism

2.	the leg without current flow is still at a low source impedance (i.e., this is not an open collector driver)

3.	as the + and - outputs are switched a di/dt exists between them that alternates in direction

Being a true differential transmission (odd mode propagation) means that any signal radiation is reduced by 10-20 dB over that of a single ended transmission line.  This lowers the shielding requirements of the interface and the overall cost of the cables.

IBM has evaluated the emissions of SSA links on their quad cable.  This is a double shielded cable having both a 100% foil shield and an 85% (minimum) coverage copper braid.  Any changes made to the level of shielding may make the interface not pass emissions; i.e., this means that any alternate cable plant should have at least the same level of shielding as that present in the currently documented quad cable.

Evaluations done for Fibre Channel have shown transmission systems with this level of shielding to effectively radiate at 0 dB over ambiant.  Changing the secondary shield to an aluminum braid (or going with only a foil shield) raises the transfer impedance such that the link will no longer pass class-A emissions.

Please also keep in mind that these emissions tests are data rate specific.  As the bit rate is increased, the amount of shielding effectiveness necessary to pass emisssions also increases.

Regards,

Ed Grivna

Principal Engineer

Data Communications Division

Cypress Semiconductor

2401 E. 86th Street

Bloomington, MN 55425

PH (612) 851-5046

FX (612) 851-5087

Email elg@cypress.com