Subj:	SSA Deadlock Notes						X3T10.1/95a132r0

Date:	95-03-15 11:48:05 EST

From:	Ed.Carmona@conner.com

To:	ssa@dt.wdc.com

From: Ed.Carmona@conner.com (Ed Carmona)

Sender: ssa-owner@dt.wdc.com

To: ssa@dt.wdc.com

Accredited Standards Committee

X3, Information Processing Systems

Doc: 	X3T10.1/95-__R0

Date:	March 15, 1995

Project: N/A

Ref Doc: X3T10.1/989D Rev 3d

Reply to: Ed Carmona

To: 	X3T10.1 Membership

From: 	Ed Carmona

Subject: SSA Deadlock Discussion Notes from X3T10.1 Plenary and SSA Deadlock

Working Group Meeting.

BACKGROUND

All the information that follows are notes generated by John Scheible and Ed Carmona during the Plenary meeting of X3T10.1 and SSA Deadlock Working Group Meeting held in March in Newport Beach, CA.. These notes should be used as a basis for further discussion on the SSA deadlock issue. I encourage feedback on this issue. These notes will be used to generate a proposal to resolve the deadlock issue.

The following is a quick summary of the SSA deadlock issue in a 2 initiator/2 target web string.

- Data is paced with Data_ready/Data_request (deadlock not applicable)

- Issue applies only to SMSs.

IA = Initiator A, IB = Initiator B

TA = Target A, TB = Target B

IA ====> TB <====> TA <====IB

IA sends SCSI_commands to TA	IB sends SCSI_commands to TB

TA sends SCSI_status to IA		TB sends SCSI_status to IB

1. TA withholds RR when IA sends more commands that TA can queue (RRs withheld on link between TA & TB).

2. TB withholds RR when IB sends more commands that TB can queue queue (RRs withheld on link between TA & TB).

3. TA cannot send SCSI_status (QUEUE FULL) to IA since TB is withholding RRs.

4. TB cannot send SCSI_status SMS (QUEUE FULL) to IB since TA is withholding RRs.

5. Link between TA & TB is deadlocked.

PLENARY MEETING DISCUSSION NOTES

The following are notes based on a discussion in an attempt to find a solution  to the SSA deadlock issue.

1. Deadlock due to Application frames (possible)

- Target detects deadlock with timer (timeout waiting for RR).

- Target places port in Privileged mode (forces incoming Application frames to be discarded).

- Target generates Async_alert SMS (indicating queue resources exhausted)

- Initiator restarts with a lower threshold of outstanding Application frames

2. Deadlock due to Application/Privileged frames mixture (rare)

- Deadlock due to Application SMSs, but some Privileged SMSs in  pipeline (not discarded by port in Privileged mode).

- Separate queue for Privileged SMSs and/or their responses (less buffer space required for queue responses).

3. Deadlock due to Privileged frames (very rare)

- Target issues Total Reset to neighbor node (TB->TA). This clears queued Privileged SMSs and responses.

- Generate Async_alert

- Initiator restarts with lower threshold of outstanding Privileged frames.

SSA DEADLOCK WORKING GROUP MEETING DISCUSSION

The following are notes based on a discussion in an attempt to find a solution to the SSA deadlock issue. These notes will serve as a basis to generate a proposal to handle the SSA deadlock condition.

1. When the Target detects an inbound SMS and no resources are available to store it internally, the Target discards both Privileged and Application SMSs directed to the node (i.e. frames with path = 0). Note that cut-through frames continue to be routed through by the Target.

2. The Target stops processing all internally queued SMSs (both Application and Privileged). Additional incoming Privileged/Application SMSs will be discarded.

3. The Target generates an Async_alert indicating SMS FRAMES DISCARDED (correct type code still needs to be determined). After sending the Async_alert, the Target will continue to discard all incoming Privileged/Application SMSs with the exception of Async_reply & Quiesce Privileged messages which will be processed normally.

4. Upon receiving the Async_alert from the Target, the Master generates an Async_reply to the Target acknowledging the Async_alert. The Master then sends Master_alert SMSs to all the initiators on the web informing them of the Target's condition. The Master should alert each Initiator individually,  awaiting a Response SMS from an Initiator for the Master_alert before informing  the next initiator.

5. Upon receiving a Master_alert, the Initiator immediately stops initiating new commands to the Target. In addition, the Initiator generates a Quiesce SMS to the affected Target. After receiving a Response from the Queisce, the Initiator generates a Response SMS to the Master for the Master_alert.

6. Once all commands to the Target have been quiesced, the Target must be reconfigured by each initiator to resume SMS processing.

7. The procedure as listed above applies to a Target on a per port basis. Therefore, if SMS are being processed by the Target on each port, the Target will generate two Async_alert SMSs to the Master indicating SMS FRAMES DISCARDED.

Ed Carmona

Conner Peripherals, Inc.

Voice: (408) 456-4082

Email: ed.carmona@conner.com