Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a128R2
Date:	April 26, 1995
Project:	989 and 1121	
Ref Doc.:	X3T10.1 / 989 D Rev 4
	X3T10.1/1121 D rev 1
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Error Handling changes proposal

REVISION 1

Revision 1 is modified as agreed in the Monday April 25 1995 X3T10.1 Working group meeting.  The proposal has also been modified to match revision 4 of SSA-TL1.  Redefinition of some ASYNC ALERT meaning codes has also been added.

REVISION 2

Revision 2 documents the modifications made (and accepted) at the April 26th 1995 X3T10.1 working group.  Also included are some style changes to better match SSA-TL1 (table under 5.9 in SSA-S2P).

BACKGROUND

The current error handling discussions are incomplete, especially in regards to initiator error handling.  The group moved most errors the Asynchronous Alert handing and they are thus reported to the master.

Discussions regarding invalid reserved field handling generating a RESPONSE rather than ASYNC ALERT has been incomplete.  This proposal gives it more complete thought.

DISCUSSION

The following discussion is my thoughts on error handling, with the thought of reporting errors via a RESPONSE message to the offending Initiator if possible to avoid bothering the entire web by using an ASYNC ALERT.

In SSA-TL1, the proposal adds a new set of sections 10.3 and it’s subsections, to go between the 10.2 and 10.3, as well as updating section 10.3.4.3 (SMS Field validation).  It also changes a reserved field in QUERY NODE REPLY to a return code field allow reporting of errors.

In SSA-S2P, the proposal changes the SMS field validation section, and adds a return code field to the SCSI RESPONSE SMS for reporting of transport level errors.

PROPOSAL

The purpose of this proposal is twofold.  First, create a more complete error handling section (including Initiator error handling).  Second, move error reporting away from Asynchronous Alerts that impact the entire web when the error information can be returned to the offending initiator.

Make the following changes to SSA-TL1.

- Add 10.3 and its subsections as shown in the following pages following the existing section 10.2.

- Remove the ASYNC ALERT meaning code of  UNSUPPORTED SMS from Table 14 - Type/subtype/type_info fields.  The UNKNOWN SMS code should be used instead.  Change all references.
This is redundant since the SMS code is sent with the frame data field.  This also allows for easier decode of the SMSs by a device (it decodes it’s supported SMSs, and anything else is UNKNOWN SMS).

- Change INVALID CODE VALUE meaning code in table 14 (03/02/00) to INVALID FIELD, to return it to the value specified in SSA-PH rev 3e.

- Change the first paragraph under 11.1.3 as follows:
Each SMS listed in Table 12 shall perform the following SMS field validations. If an asynchronous alert is generated then the message is then terminated, without any other response

- Change bullet 1 under 11.1.3 as follows:
If the return path or initiator id field is unknown, then generate an ASYNC ALERT SMS with an Unknown Return path or INITIATOR id meaning code, with two exceptions. The QUERY NODE accepts any return path, since it builds the Initiator Table.  The CONFIGURE PORT SMS responds with a RESPONSE SMS with a return code of UNKNOWN RETURN PATH for an unknown return path, since an alert path may not be accepted yet for the ASYNC ALERT SMS.

- Change bullet 2 under 11.1.3 to match the other terminology as follows
If any reserved field is non-zero or a valid field contains a reserved value, then generate an appropriate response as defined in section 10.3.8

- Add a new third bullet to 11.1.3 as follows:
If the tag field is invalid, or valid with an improper response, then the initiator should log the error and ignore the SMS.  If the SMS was a data transfer SMS, the initiator may choose to abort the I/O process using the ULP if the ULP is known.

- Change the first sentence of the first paragraph under 11.2.12 Response 
from:
The response SMS (see table 33) is used to acknowledge the configure port, master alert, and quiesce SMSs.
to:
The RESPONSE SMS (see table 33) is used to acknowledge the CONFIGURE PORT, MASTER ALERT, and QUIESCE SMSs.  In addition, in the case of invalid fields, the RESPONSE SMS is used to acknowledge the QUERY PORT, QUERY PROTOCOL and DATA REPLY SMS.

- Restore the following return code value for RESPONSE SMSs in table 30 as follows:
03h	Invalid return path

- Add the following sentence at the end of the sixth paragraph after table 18: 
If the b quota value is less than the a quota value, then the CONFIGURE PORT SMS shall be ignored and a RESPONSE SMS generated to the initiator issuing the CONFIGURE PORT SMS with the appropriate tag value and a return code of INVALID FIELD.

- Add the following sentence to the end of the first paragraph under 11.2.7 QUERY PORT
The destination node normally returns a QUERY PORT REPLY SMS, however, and invalid field will cause a RESPONSE SMS to be generated with a return code of INVALID FIELD.

- Change the last sentence of the first paragraph under 11.2.9 QUERY PROTOCOL to:
The destination node normally returns a QUERY PROTOCOL REPLY SMS, however, and invalid field will cause a RESPONSE SMS to be generated with a return code of INVALID FIELD.

- Delete table 33 and rewrite the preceding paragraph to  
 In the case of an invalid field, then a the DATA REPLY SMS will be acknowledged with a RESPONSE SMS with a return code of INVALID FIELD.  In the case of an invalid initiator id field value, an unexpected DATA REPLY SMS, or too many outstanding DATA REPLY SMSs, an ASYNC ALERT SMS will be generated with the appropriate meaning codes.

- Rename the following ASYNC ALERT meaning codes from table 14 and all references to them.
04/01/00 from “Unexpected response SMS” to “SMS UNEXPECTED”   
04/03/00 from “Too many outstanding data reply SMSs” to “TOO MANY SMSS OUTSTANDING”

- Remove the ASYNC ALERT meaning code of  UNEXPECTED DATA REPLY SMS from Table 14 - Type/subtype/type_info fields.  The SMS UNEXPECTED SMS code should be used instead.  Change all references.

Make the following changes to SSA-S2P.

- Replace sub-clauses 5.9 through 5.10.3 as attached.

- Change the last sentence to the first paragraph of 5.1 SCSI COMMAND SMS 
from
The target will respond by optionally initiating a data transfer and sending a SCSI STATUS SMS.
to
If the SCSI COMMAND SMS can be successfully parsed, the target will respond by optionally initiating a data transfer and sending a SCSI STATUS SMS.  Refer to section 5.10 for invalid SMS processing which can result in either an ASYNC ALERT with various meaning codes or a SCSI RESPONSE SMS with a return code of INVALID FIELD.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com

- Contents

10. Error Handling

10.1 Link Errors

10.2 Link ERP

10.3 Invalid SMS field handling

10.3.1 Unknown SMS code

10.3.2 Unsupported SMS code

10.3.3 SMS too small

10.3.4 Invalid Return Path

10.3.5 Invalid Initiator ID

10.3.6 Invalid Tag

10.3.7 Invalid context

10.3.8 Non-zero reserved field or reserved code value

10.4 Asynchronous Alert Handling

10.5 Master Alert Handling

10.6 Target not responding

10.7 Communication blocked

# Error Handling

(Same text as in the same section/subsections of the specification)

## Link Errors

(Same text as in the same section/subsections of the specification)

## Link ERP

(Same text as in the same section/subsections of the specification)

## Invalid SMS field handling

The section discusses invalid field handling of data fields within frames.  It is assumed that the frame has been successfully received by the hardware.  This discussion centers on the processing of the data field (i.e. the contents of the SMS).

The order of processing is critical and matches the order of the following subsections.

### Unknown SMS code

An SMS received with an unknown sms code cannot be parsed since it cannot be determined if the SMS has either a Return Path or Initiator ID, and therefore must generate an ASYNC ALERT SMS with a meaning code of UNKNOWN SMS.

### Unsupported SMS code

An SMS received with a known sms code that is either not supported by the node, or has the incorrect frame type must generate an ASYNC ALERT SMS with a meaning code of UNKNOWN SMS.

### SMS too small

If the SMS length is shorter than specified under clause 11, an ASYNC ALERT SMS shall be generated with a meaning code of SMS TOO SHORT.

It is not an error to receive an SMS longer than the specified length (if less  than 33 bytes of data field), even if the extra bytes are non-zero.

### Invalid Return Path

An invalid Return Path is one not contained in the Initiator Table.

The QUERY PORT SMS will always execute, as it is building the Initiator table

The CONFIGURE PORT SMS with an invalid Return Path will generate a RESPONSE SMS with a return code of INVALID FIELD.

Other SMSs with an invalid Return Path will generate an ASYNC ALERT SMS with a meaning code of INVALID PATH OR INITIATOR ID.  These SMSs include MASTER ALERT, QUERY PORT, QUERY PROTOCOL,  and QUIESCE.

### Invalid Initiator ID

An Invalid Initiator ID is an Initiator ID not contained in the Initiator table.  No Return Path can be associated with it and therefore, no response can be sent to the specified initiator.

If the target receives an SMS with an invalid Initiator ID, an ASYNC ALERT SMS with a meaning code of INVALID RETURN PATH OR INITIATOR ID shall be generated.  When the master generates the MASTER ALERT SMS, this should cause all initiators to issue a query node to the associated device to rebuild the table.

### Invalid Tag

An SMS sent to an initiator with an invalid Tag, or a valid Tag with an improper response, no recovery is possible.  The initiator should log the error and ignore the SMS.  In the case of data transfer SMSs with an invalid Tag, the protocol layer could issue the appropriate upper level protocol response to cancel the I/O process if possible (ABORT TAG SMS for SSA-S2P or SSA-S3P).

### Invalid context

When an SMS is received out of context, it shall be discarded and the event shall be logged by an initiator or ignored by a target.  Examples of out of context SMSs include the following.

- DATA REPLY SMS when no associated DATA READY SMS was issued.

- DATA REQUEST SMS was received for an I/O process that did not need to write data.

- DATA READY SMS was received for an I/O process that did not need to read data.

- ASYNC REPLY SMS with a valid tag value without an outstanding ASYNC ALERT SMS.

- QUERY PROTOCOL REPLY SMS with a valid tag value not associated with an outstanding QUERY PROTOCOL SMS.

- QUERY PORT REPLY SMS with a valid tag value not associated with an outstanding QUERY PORT SMS.

### Non-zero reserved field or reserved code value

An otherwise valid SMS with an non-zero reserved field or a reserved code value could be parsed and the Initiator ID or Return Path used to send the appropriate response (excluding Unknown or Invalid SMS codes).  Table 1 shows the response for each transport layer SMS.

Table 1 - Response to non-zero reserved fields or reserved code values

| SMS Name | Resulting SMS | Notes |
| --- | --- | --- |
| QUERY NODE | Normal QUERY NODE REPLY | Ignore reserved field/values |
| QUERY NODE REPLY | None | Ignore reserved field/values |
| CONFIGURE PORT | RESPONSE SMS with return code of INVALID FIELD |  |
| RESPONSE | Discard | System dependent retry |
| ASYNC ALERT | ASYNC REPLY SMS with return code of INVALID FIELD | Generate MASTER ALERT |
| MASTER ALERT | RESPONSE SMS with return code of INVALID FIELD |  |
| QUIESCE | RESPONSE SMS with return code of INVALID FIELD |  |
| ASYNC REPLY | None | No resend needed, clear the async alert data. |
| QUERY PROTOCOL | RESPONSE SMS with return code of INVALID FIELD |  |
| QUERY PROTOCOL REPLY | Discard | System dependent retry |
| QUERY PORT | RESPONSE SMS with return code of INVALID FIELD |  |
| QUERY PORT REPLY | Discard | System dependent retry |
| DATA READY | Discard | Cancel I/O Process |
| DATA REPLY | RESPONSE SMS with return code of INVALID FIELD | Ignore DATA REPLY |
| DATA REQUEST | Discard | Cancel I/O Process |

## Asynchronous Alert Handling

(Same text as in the same section/subsections of X3T10.1/95-127r0 shown as section 10.3 and subsections).

## Master Alert Handling

(Same text as in the same section/subsections of X3T10.1/95-127r0 shown as section 10.4 and subsections).

## Target not responding

(Same text as in the section of the same name in the rev 3d specification)

## Communication blocked

(Same text as in the same section of the specification, or Ed Carmona’s deadlock proposal)

(IN SSA-TL1)

- 11.1.3  SMS field validation

Each SMS listed in Table 12 shall perform the following SMS field validations. If an asynchronous alert is generated then the message is then terminated, without any other response

- If the return path or initiator id field is unknown, then generate an ASYNC ALERT SMS with an Unknown Return path or INITIATOR id meaning code, with two exceptions.  The QUERY NODE accepts any return path, since it builds the Initiator Table.  The CONFIGURE PORT SMS responds with a RESPONSE SMS with a return code of UNKNOWN RETURN PATH for an unknown return path, since an alert path may not be accepted yet for the ASYNC ALERT SMS.

- If any reserved field is non-zero or a valid field contains a reserved value, then generate an appropriate response as defined in section 10.3.8.

- If the tag field is invalid, or valid with an improper response, then the initiator should log the error and ignore the SMS.  If the SMS was a data transfer SMS, the initiator may choose to abort the I/O process using the ULP if the ULP is known.

(IN SSA-S2P)

- 5.9  Summary of the SSA SCSI-2 Protocol SMSs

A summary of all the SSA SCSI-2 Protocol SMSs is shown in Table 2.

Table 2 - Summary of SCSI-2 SMSs

| SMS name | Byte 0 | Byte 1 | SMS frame type | Node type support |  |
| --- | --- | --- | --- | --- | --- |
|  | ulp code | sms code |  | Send | Receive |
| SCSI response | 80h | 03h | APPLICATION | target | initiator |
| SCSI command | 80h | 10h | APPLICATION | initiator | target |
| SCSI status | 80h | 11h | APPLICATION | target | initiator |
| Abort tag | 80h | 30h | APPLICATION | initiator | target |
| Abort | 80h | 31h | APPLICATION | initiator | target |
| Clear queue | 80h | 32h | APPLICATION | initiator | target |
| Device reset | 80h | 33h | APPLICATION | initiator | target |
| Clear ACA condition | 80h | 34h | APPLICATION | initiator | target |

- 5.10  SMS verification

Prior to the SSA SCSI-2 Protocol receiving the SSA SCSI-2 Protocol SMS, the transport layer has validated the ulp code field (byte 0) as matching that of SSA-SCSI-2 (82h).  Each SMS then undergoes three levels of validation as described in the following three sections.

- 5.10.1  SMS code verification

After the transport layer validity checks the SMS, the SCSI-2 Protocol will validity check byte 1, the sms code, in the order  shown in the following list.  If an async alert SMS is generated then the message is then terminated, without any other response.

- If byte 1 of the SMS (sms code) is not a value supported in Table 2, then generate an async alert SMS with an Unknown SMS error code.

- If byte 1 of the SMS is supported in Table 2, but the node types do not match the corresponding entry, then generate an async alert SMS with an SMS UNKNOWN error code.

- If Table 2 matches byte 1 of the SMS, but the frame type field does not have a value of application then generate an async alert SMS with an SMS UNKNOWN error code.

- 5.10.2  SMS length verification

If the SMS length is shorter than that specified, then generate an async alert SMS with an SMS too short error code.  If an async alert SMS is generated then the message is then terminated, without any other response.

- 5.10.3  SMS field verification

Each SMS listed in Table 13 shall perform the following SMS field verifications in the order shown. If an async alert SMS is generated then the message is then terminated, without any other response

- If the initiator id field is unknown, then generate an ASYNC ALERT SMS with an Unknown RETURN PATH OR Initiator ID meaning code.

- If the initiator receives a SCSI RESPONSE or SCSI STATUS SMS with an invalid tag field, then generate an ASYNC ALERT with an SMS UNEXPECTED meaning code.

- If any reserved field is non-zero or valid field contains a reserved code value, then generate a RESPONSE SMS with an Invalid field  return code.  There is one exception to this case, where any improper field values within the command descriptor block, luntar, or luntrn of the SCSI COMMAND SMS shall reply with a SCSI STATUS SMS with Check Condition Status and sense data as defined in SCSI-2.