To:	X3T10.1 Membership						X3T10.1/95a143r0

From:	ianjudd@VNET.IBM.COM

Subj:	New link connected

Date:	95-03-21 12:01:49 EST

BACKGROUND

Currently SSA-PH section 10.3.5, ‘New Link connected’, does not specify any explicit Master_alert messages to indicate that the new ports have been changed to Normal mode.

Although SSA-PH does not specifically say, the action of forwarding the ‘Port now operational’ Master_alert performs two functions.  Firstly it informs other Initiators that there is a new link which should be walked and secondly it informs them that the ports being walked are now in Normal mode.

This means that we are effectively sending a global ‘all ports in Normal mode’ indication to the other Inititators. This can lead to different Initiators having conflicting views of which ports in the web are in Normal mode. In turn ULP messages can be sent via links whose ports are still in Privileged mode.

Consider the following. A port on the web becomes Operational and it informs the Master via a ‘Port Now Operational’ Async_alert. The Master walks the web and finds N nodes, some of which are Initiators. The Master issues Configure_port to all of the new ports to put them in Normal mode. The Master then forwards a Master_alert to each other Initiator which in turn walks the web. In between the Master finishing his walk and the other Initiators starting theirs another port becomes operational so that X more nodes are added to the web.

The Initiators will walk the web and find N+X nodes. Due to the implicit ‘all ports in Normal mode’ indication conveyed by the ‘Port Now Operational’ Master_alert the other Initiators assume, incorrectly, that N+X nodes are in Normal mode. In fact only the first N nodes are in Normal mode.

A ‘ports now normal’ indication is needed but it should be implemented via a separate Master_alert for each link. This would also be consistent with all other forms of link-related alert processing whereby the Master forwards the error to the other Initiators, and, when recovery is complete, informs the Initiators of the fact via a separate ‘Both ports of link are in Normal mode’ Master_alert.

PROPOSAL

Rewrite section 10.3.5 'New Link Connected' as follows:

10.3.5 New Link Connected

When a new link is connected to the web a port on one of the existing nodes becomes operational:

1) The node shall send an Async_alert to the Master specifying 'Port now operational'. The Master shall return an Async_reply.

2) The Master shall walk the new link by issuing Query_node to the new  node(s). Each node shall return a Query_node_reply.

3) The Master shall send a Master_alert specifying 'Port now operational' to each other primary Initiator.

4) Each other Initiator shall walk the new link by issuing Query_node to the new node(s). Each node shall return a Query_node_reply. When an Initiator has completed its walk it shall return a Response to the 'Port now operational' Master_alert.

5) The Master shall issue a Configure_port specifying 'set Normal mode' to those ports which are operational on the new nodes that it has walked. Each port shall return a Response.

6) The Master shall issue a Master_alert to each other Initiator specifying 'Both ports of link are in Normal mode' for each new link whose ports are now in Normal mode.

7) The other Initiators shall return a Response for each Master_alert.