Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/94-015r3
Date:	May 09, 1995
Project:	989-D
Ref Doc.:	X3T10.1/989-D r3
Reply to:	John Scheible

To: 	X3T10.1 Membership
From:   John Scheible, IBM

Subject:Option block Annex proposal

The May 1995 X3T10.1 plenary altered the style and moved information in the LED capable output section to an implementors note.

The November X3T10.1 plenary authorized me to convert X3T10.1/94-015r1 to Word and include it in the SSA-PH document as an informative annex.  That document had three parts.

- SSA port description:
The SSA port write-up should be contained in the new Electrical Interface write-up.

- Option block description:
The following  pages are that converted portion of the document.

- Power section:
the Power section should be contained in the connector write-up section.

The following section only contains the option block section which is to be included as an informative annex to SSA-PH.  The method of controlling the programmable pins is a protocol level discussion and will not be included in SSA-PH, but will appear in the protocol sections when X3T10 approves the method.

Currently a proposal entitled “protocol mode page proposal” is being discussed.  This proposal hopes to identtify a protocol specfici mode page to report the Unique ID, as well as control the programmable option block pins.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com

- Annex A
(Normative)
Unitized Connector Option Bay

- Annex A.1.  Electrical characteristics

All option port signals are TTL level compatible as defined below.  In addition, four signals are capable of sinking sufficient current to drive LED (Light Emitting Diodes).  All electrical characteristics must be met over a 0 to 70 degree centigrade range as measured at the surface of the associated integrated circuit.  All signals are negative active.

- Annex A.1.1.  Input characteristics

Table 1 - TTL input pin parameters

| Characteristic | Minimum | Maximum | Conditions |
| --- | --- | --- | --- |
| Input Leakage | -10 uA | +10 uA |  |
| Input High | +2.0 V | +5.5 V |  |
| Input Low | -0.5 V | +0.8 V |  |

- Annex A.1.2.  Output characteristics

Table 2 - TTL output pin parameters

| Characteristic | Minimum | Maximum | Conditions |
| --- | --- | --- | --- |
| Tri-state Leakage | -10 uA | +10 uA | 0.4 V to 5.5 V |
| Output High | 2.4 V | 5.5 V | Output current = 800 uA |
| Output Low | 0.0 V | 0.4 V | Output current = 4 mA |

- Annex A.1.3.  LED capable Outputs

The LED capable outputs are current limited open collector outputs that would normally have an LED anode connected directly to 5V through a current limiting resister with the LED cathode connected directly to the LED capable output pin.  The LED capable output must be capable of indefinitely sinking the current produced when the pin is shorted directly to 5.5 V.

Implementor’s note: The specifications can be met with an open collector that is capable of indefinitely sinking 37 mA with a 150 ohm current limiting resistor.  However, the normal current draw with a 150 ohm resister would be 28 ma (5.0V - 0.7V led drop divided by 150 ohms).

Table 3 - TTL LED Capable Output pin parameters

| Characteristic | Minimum | Maximum | Conditions |
| --- | --- | --- | --- |
| Output current at 5.5 V |  | 40 mA | Pathological case of "shorted to +5V pin" with no LED |
| 'On' Output current at 3.3 V | 18 mA | 25 mA | Normal on case with 1.7 V drop across LED tied to +5V |
| 'Off' Output current at 3.3 V | 0.0 mA | 1.0 mA | Normal on case with 1.7 V drop across LED tied to +5V |

- Annex A.2.  Signal definitions

Figure 1.  Options connector when looking at the device end.

Insert artwork of option port of unitized connector here

- MTM (Manufacturing Test Mode) (Input)
A low active input pin, that when active (pulled below 0.8 V) makes pins 2, 3, 4, 6, 8, 9, and 10 available to be redefined.  Pins 5 and 7 must remain Ground and +5 V respectively.  One possible purpose of this pin is to allow a manufacturing tester to redefine the option pins to whatever functions it desires, while allowing the shipped product to return to the standard definitions in the customers environment.

- Auto Start (Input)
A low active input pin, that when active (pulled below 0.8 V) causes the device motor to spin up and become ready for media access after power is applied without the need to receive a Start/Stop Unit SCSI command.  When inactive (pulled above 2.0 V), the device motor shall not spin up until after the receipt of a Start/Stop Unit SCSI command.  The signal is to be sampled by the device at Power on and hard or soft reset conditions.

- Sync (Input/Output)
Synchronization is achieved by having one node use this pin as an output to transmit a sync signal.  The other nodes may use the pin as an input and synchronize their revolution speeds to match the Sync signal.  The SSA network provides a sync character over the SSA network, but this option pin allows synchronization across multiple SSA networks, or allow for tighter control of the jitter of the Sync pulse.
The width, period, and tolerance of the negative active Sync pulse is manufacturer dependent, and thus synchronization across different manufacturers or even different product lines of the same manufacturer is not guaranteed.  The Sync pin usage is controlled by mode pages within the Mode Select SCSI command.

- External Fault (Input)
A low active input pin, that when active (pulled below 0.8 V) indicates a fault in the external environment.  This pin may also be used as a write protect.  When the pin is active, any actions that alter customer data on the media shall be inhibited.

- Ground (long)
The device shall be capable of handling 1.0 amp of current through this ground pin.  This pin is longer than any others in the option block to allow for the ground to mate first in a hot-plug situation.

- Device Activity (LED capable output) 
This pin is a low active LED output which shall sink sufficient current to light an LED when the device is active.

- +5 Volts
The device shall supply up to 1.0 Amp of current limited +5 V (+/- 10%), as long as power is supplied to the device.

- Device Fault (LED capable output)
This pin is a low active LED output which shall sink sufficient current to light an LED when the device is in an fault condition.

- Programmable 1 (Input/LED capable output)	
This pin must be capable of either being an input or an LED capable output.  The function of this pin is controlled in a protocol specific manner.  Refer to the appropriate protocol specification for details.

- Programmable 2 (Input/LED capable output)
This pin must be capable of either being an input or an LED capable output.  The function of this pin is controlled in a protocol specific manner.  Refer to the appropriate protocol specification for details.

- Annex A.3.  Programmable features

The programmable pins in the option block are controlled in a protocol specific manner, but the meaning and values are standardized across all protocols.

Table 4 - Programmable pin values.

| Value | Meaning | Direction | Type |
| --- | --- | --- | --- |
| 0 | Node installed | From node | Output |
| 1 | Environmental warning | To node | Input |
| 2 | Environmental failure | To node | Input |
| 3 | Reserved |  |  |
| 4 | Port 1 ready | From node | LED Output |
| 5 | Port 2 ready | From node | LED Output |
| 6 | Serial In | To node | Input |
| 7 | Serial out | From node | Output |
| 8 | Node off-line | From node | LED Output |
| 9-FFh | Reserved |  |  |

Node installed is a low active signal that indicates a node is installed.  This can be accomplished by pulling this signal high on the backplane via a resister then having a device pull the signal to ground.  If the device shorts the signal to ground on the circuit board (not programmable), then a low active signal (below 0.8V) indicates that a device is installed.  If the device uses the programmable nature of the pin and determines it is programmed as "Node installed", then a negative active signal (below 0.8V) indicates the device is installed and the device is (or was) operational.

Environmental warning is a low active signal that indicates a non-critical problem in the environment.  When the device detects a low active signal (below 0.8V), the actions will be protocol dependent.  In the case of SCSI mappings, a deferred error will be generated with a sense key/code/qualifier of T.B.D..  An example of an Environmental warning is the failure of a redundant cooling fan.

Environmental Failure	is a low active signal that indicates a critical problem in the environment.  When the device detects a low active signal (below 0.8V), the device shall create an Async_alert message with a Type field of Environmental failure.

Port 1 Ready is a negative active LED output which shall sink sufficient current to light an LED when port 1 is in the ready mode.

Port 2 Ready is a negative active LED output which shall sink sufficient current to light an LED when port 2 is in the ready mode.

Serial In is an input signal to the device that carries serial data using a T.B.D. protocol.

Serial Out is an output signal from the device that carries serial data using a T.B.D. protocol.

Node Off-line is a low active LED output which shall sink sufficient current to light an LED when no ports to a device are receiving flags or data.  This signal can be used to indicate that the device can be removed with no impact to the system.  This is accomplished by having the system place all ports in wrap mode that are transmitting to the device to be replaced.  If the device to be replaced is functioning enough to detect the loss of signals, then it can light the "Node off-line" LED.