Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a133R0
Date:	April 22, 1995
Project:	1121
Ref Doc.:	X3T10.1/1121 rev 1
	X3T10.1/95a128r0
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: SCSI STATUS SMS enhancement

BACKGROUND

SSA-S2P currently does not allow Contingent Allegiance.  The problem is that commands in the pipeline will reset the Contingent Allegiance condition before the initiator sees the SCSI Status.  However, if we reported sense data with the SCSI Status and the initiator did not care about holding the queue, we could offer contingent allegiance.

This would give the initiator the choice of using Contingent Allegiance for times when he does not need the queue held, and Auto Contingent Allegiance when recovery must be done before the queue is released.

This proposal might also make it easier through review with both concepts in place.

PROPOSAL

This proposal is based off of modifications made in X3T10.1/95a128r0.  This proposal added a return code field to the SCSI STATUS SMS.

The proposal affects three areas, the front restrictions area, the SCSI STATUS SMS, and the Auto Contingent Allegiance section.

This proposal makes the following changes to SSA-S2P (X3T10.1/1121 rev 1).

- Replace the second bullet under Restrictions in the Introduction section with Contingent Allegiance and add the following paragraph under the bullet:

The SCSI-2 concept of Contingent Allegiance may have unexpected consequences for SSA since the next command resets the Contingent Allegiance condition, and the next command may be in the SSA pipeline prior to the status being received by the initiator.  Therefore, Contingent Allegiance should only be used when the initiator does not care that the Contingent allegiance condition may be reset inadvertently, and does not need more sense data than the sense key/code/qualifier fields of SCSI sense data which will be returned in the SCSI STATUS SMS.

- Following the restrictins section in the introductiion section, add a paragraph outside of the restictions bullet list consisting of the one word “Additions” and add a single bullet entitled “The SSA SCSI-2 Protocol supports Auto Contingent Allegiance.”.  Add the following paragraph under the bullet.

 The SSA SCSI-2 Protocol will adopt the SCSI-3 concept of Auto Contingent Allegiance where the Auto Contingent Allegiance condition must be explicitly reset by a CLEAR ACA CONDITION SMS (SSA Message Structure), thereby allowing for recovery procedures to be performed while still in the Auto Contingent Allegiance state.  The selection of Auto Contingent Allegiance is controlled by a bit in the CDB control byte as defined in SAM.  This bit does not exist in parallel SCSI-2.

- Add three more entries (bytes 7, 8, and 9) to the SCSI STATUS SMS table under 5.2 with the field names of sense key, sense code, and sense qualifier.

- Add the following paragraph to the end of section 5.2 SCSI STATUS SMS.

The sense key, sense code, and sense qualifier fields are the same as the fields of the same name in the SCSI sense data.  If status field has a CHECK CONDITION STATUS value, then the sense key, sense code, and sense qualifier fields shall be included in the SCSI STATUS SMS, otherwise, the fields are ignored and the SCSI STATUS SMS may be truncated after the return code field.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com

Restrictions:

- Untagged queuing can be simply emulated with Tagged Queuing

In SSA, the target effectively disconnects after each frame because of frame multiplexing.  Hence each command must have a tag for identification.  Untagged command queuing can be simply emulated by having the initiator have only one outstanding command, re-use the same tag, and use the simple queue type.

- Contingent Allegiance

The SCSI-2 concept of Contingent Allegiance may have unexpected consequences for SSA since the next command resets the Contingent Allegiance condition, and the next command may be in the SSA pipeline prior to the status being received by the initiator.  Therefore, Contingent Allegiance should only be used when the initiator does not care that the Contingent allegiance condition may be reset inadvertently, and does not need more sense data than the sense key/code/qualifier fields of SCSI sense data which will be returned in the SCSI STATUS SMS.

- Additions

- The SSA SCSI-2 Protocol supports Auto Contingent Allegiance.

 The SSA SCSI-2 Protocol will adopt the SCSI-3 concept of Auto Contingent Allegiance where the Auto Contingent Allegiance condition must be explicitly reset by a CLEAR ACA CONDITION SMS (SSA Message Structure), thereby allowing for recovery procedures to be performed while still in the Auto Contingent Allegiance state.  The selection of Auto Contingent Allegiance is controlled by a bit in the CDB control byte as defined in SAM.  This bit does not exist in parallel SCSI-2.

- 5.2  SCSI STATUS

The SCSI STATUS SMS is sent from a target to an initiator to indicate that a command and any associated data transfer have been terminated or completed.  The SCSI STATUS SMS is returned using the path and port associated with the initiator id field specified in the SCSI COMMAND SMS.

The SCSI STATUS SMS is returned for each SCSI COMMAND SMS unless the command is rejected (with an ASYNC ALERT SMS) or the command is cleared by any of the following:

- Abort tag SMS

- Abort SMS

- Clear queue SMS

- Device reset SMS

The contents of the data field in the SCSI STATUS SMS are shown in Table 1.

Table 1 - SCSI STATUS SMS

| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | ulp code = 80h |  |  |  |  |  |  |  |
| 1 | sms code = 11h |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | status |  |  |  |  |  |  |  |
| 5 | reserved=0000 00b |  |  |  |  |  | flag | link |
| 6 | return code |  |  |  |  |  |  |  |
| 7 | sense key |  |  |  |  |  |  |  |
| 8 | sense code |  |  |  |  |  |  |  |
| 9 | sense qualifier |  |  |  |  |  |  |  |

The tag  field is a copy of the tag field in the corresponding SCSI COMMAND SMS.  It allows the initiator to associate the status with the correct I/O process.

The status field contains status as defined by SCSI-2 with the addition of Auto Contingent Allegiance Active status (30h) (See Annex A).

The SSA SCSI-2 Protocol uses one SMS code with imbedded flag and link bits, rather than create three different completion messages as parallel SCSI-2 did.  The flag and link bits indicate different completion messages as shown in Table 2.

Table 2 - Meaning of Flag and Link bits

| flag | link | Corresponding parallel SCSI-2 completion message |
| --- | --- | --- |
| 0 | 0 | Command Complete |
| 0 | 1 | Linked Command Complete |
| 1 | 0 | Reserved |
| 1 | 1 | Linked Command Complete with Flag |

The return code field indicates the result of the SCSI COMMAND SMS, and the values are shown in Table 6.

Table 3 - RETURN CODE values for the SCSI STATUS SMS

| Value | Description |
| --- | --- |
| 00h | The SCSI command was parsed successfully. |
| 01h-FEh | reserved |
| FFh | Invalid field |

The sense key, sense code, and sense qualifier fields are the same as the fields of the same name in the SCSI sense data.  If status field has a CHECK CONDITION STATUS value, then the sense key, sense code, and sense qualifier fields shall be included in the SCSI STATUS SMS, otherwise, the fields are ignored and the SCSI STATUS SMS may be truncated after the return code field.

- 6.1.2  Contingent Allegiance Condition

The SCSI-2 concept of Contingent Allegiance may have unexpected consequences for SSA since the next command resets the Contingent Allegiance condition, and the next command may be in the SSA pipeline prior to the status being received by the initiator.  Therefore, Contingent Allegiance should only be used when the initiator does not care that the Contingent allegiance condition may be reset inadvertently, and does not need more sense data than the sense key/code/qualifier fields of SCSI sense data which will be returned in the SCSI STATUS SMS.

- 6.1.3  Auto Contingent Allegiance Condition

The SSA SCSI-2 Protocol supports the SCSI-3 concept of Auto Contingent Allegiance Condition.  Auto Contingent Allegiance allows the initiator to perform error recovery while in the Auto Contingent Allegiance condition and requires an CLEAR ACA CONDITION SMS.  The choice of Contingent Allegiance or Auto Contingent Allegiance processing is controlled by an aca flag in it 2 of the control field of the CDB.  This field is new to SCSI-2 CDBs but is part of SCSI-3 CDBs.

When a target sends Check Condition status to an initiator it creates an Auto Contingent Allegiance condition for that initiator and Logical Unit or Target Routine.  The Auto Contingent Allegiance condition causes the target to either suspend or purge queued I/O processes for that nexus, depending on the value of the QErr bit in Mode Select parameter page 0Ah.

The Auto Contingent Allegiance condition also causes subsequent I/O processes received for the same nexus to be rejected with Auto Contingent Allegiance Active status (30h), unless the queue cntl field specifies Auto Contingent Allegiance.  Such an I/O process will be referred to as an ACA I/O process.  ACA I/O processes are executed normally during an Auto Contingent Allegiance condition.

Only one ACA I/O process can be active for a given nexus at a time.  If a target receives an ACA I/O process while it has another active ACA I/O process for the same nexus then it rejects the second I/O process with Auto Contingent Allegiance Active status.

If a target receives an ACA I/O process while it does not have an Auto Contingent Allegiance condition for the same nexus, then it returns Check Condition status.  The sense data contains a sense key of Illegal Request and an additional sense code of Invalid Message Error.

The Auto Contingent Allegiance condition is cleared by any of the following conditions.

- clear aca condition SMS

- device reset SMS

- Total/Absolute Reset frame (See SSA-PH)

- Local Reset (See SSA-PH)

After the Auto Contingent Allegiance condition is cleared by a clear aca condition SMS, the target resumes processing any remaining suspended I/O processes for that nexus.

The Auto Contingent Allegiance condition is associated with a particular initiator and Logical Unit or Target Routine.  It does not affect other I/O processes.  Multiple Auto Contingent Allegiance conditions can exist for different initiators, Logical Units and Target Routines at the same time.