Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a129R0
Date:	May 23, 1995
Project:	1121D
Ref Doc.:	X3T10.1/1121D rev 02
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Protocol Services proposal for SSA-S2P

BACKGROUND

To be SAM compliant, any SCSI protocol needs to show how it maps the SAM services onto the SCSI protocol layer.  Although informative, the editor feels the section is necessary to pass public (and X3T10) review.

PROPOSAL

Due to time pressures, of the mailing, the proposal is not as clean as I would like.  I still need to work on style and on how to handle ports.

This proposal is informative only, and does not place any requirements on the implementation, other than those requirements already specified in the specification.

This proposal is based off rev 16 of SAM (the public review version).

This proposal consists of several changes.

- First, add the following definition to section 3.2. Symbols and Abbreviations...
SAM		SCSI-3 Architectural Model

- Change the title of the CLEAR ACA CONDITION SMS to CLEAR ACA to match SAM rev 16.  This affects sections 5.7 and 5.9.

- Replace the placeholder section 5 (SAM SERVICES) with the following pages (excluding the editors notes).

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com

- Contents

5. Protocol Services Model

5.1 Conventions

5.1.1 Definitions changes from SCSI-2 to SCSI-3

5.1.2 Object definitions

5.1.3 Service naming conventions.

5.1.4 Notations

5.1.5 Modifications to the SAM Services model

5.2 SCSI Command Model

5.2.1 SAM / Protocol / Transport activity for command execution

5.2.2 Send SCSI Command protocol service request

5.2.3 SCSI Command received protocol service indication

5.2.4 Send Command Complete protocol service response

5.2.5 Command Complete Received protocol service confirmation

5.3 Data Transfer Protocol Services

5.3.1 Send Data-In protocol service request

5.3.2 Data Delivered protocol service confirmation

5.3.3 Receive Data-Out protocol service request

5.3.4 Data-Out Received protocol service confirmation

5.4 Task Management Functions

5.4.1 Abort Task

5.4.2 Abort Task Set

5.4.3 Clear ACA

5.4.4 Clear Task Set

5.4.5 Target Reset

5.4.6 Terminate Task

5.5 Task Management Protocol Services

5.5.1 Send Task Management Request (Object Address, Function Identifier ||)

5.5.2 Task Management Request Received (Object Address, Function Identifier ||)

5.5.3 Task Management Function Executed (Object Address, Service Response ||)

5.5.4 Received Function-Executed (Object Address, Service Response ||)

# Protocol Services Model

This clause demonstrates how the SAM Services model can map onto the SSA-S2P protocol.  This clause in informational only, and does not place any requirements on the implementation, other than those requirements already specified in this specification.  Since the SAM Service model is a logical model, certain liberties were taken in order to map it onto a more detailed implementation (See 5.1.5).

## Conventions

This section describes how to map the various SAM objects into SSA-S2P fields, and describes some of the notations used in this clause.

### Definitions changes from SCSI-2 to SCSI-3

This specification refers to both the SCSI-2 specification, and to the SCSI-3 Architectural model (SAM).  This section gives an approximate correlation between SCSI-2 terms and SAM terms.  Refer to the appropriate specifications for more detailed definitions.

- SCSI-2 I/O process relates to SCSI-3 Task.

- SCSI-2 Target relates to SCSI-3 Device Server.

- SCSI-2 Initiator relates to SCSI-3 Application Client

### Object definitions

The Port Identifier is a single byte value the device assigns to a physical port, which is always the same for a given port.

The Target Identifier is the device’s 64 bit Unique ID.  The Unique ID is globally unique, and is the value is the same to each device in the SSA web.

The Initiator Identifier is the device’s 64 bit Unique ID.  The Unique ID is globally unique, and is the value is the same to each device in the SSA web.

The Logical Unit Number is a 7 bit value that resides in the luntrn field (byte 8) of those SMSs that contain a Logical Unit Number (SCSI COMMAND, ABORT, CLEAR QUEUE, and CLEAR ACA).  The SCSI-2 concept of  Target Routines is also included in byte 8.

The Tag is a two byte value that resides in the tag field (bytes 2 and 3) of all SSA-S2P SMSs.

The Task Attribute is a two bit value that resides in the queue cntl field of the SCSI COMMAND SMS.

### Service naming conventions.

The term Protocol Services refers to those services between the SSA-S2P layer and the SAM layer, and in following SAM the name contains first letter capitalized words.  The term Transport Services refers to those services between the SSA-TL1 transport layer and the SSA-S2P protocol layer, and in following SSA-TL1 and SSA-S2P conventions the name is all letters capitalized.

### Notations

The services are indicated with indented text as illustrated below.

Service-name ( Input parm1, [Optional input parm2] || Output parm1, [Optional Output parm2] )

The service name precedes the parenthesis.  The parenthesis enclose the parameter lists.  Any parameter enclosed in angle brackets ([...]) is considered optional.  The double bar (||) separates the input parameters from the output parameters.

### Modifications to the SAM Services model

The SAM Services model is a logical model.  In an attempt to show a more detailed implementation of the SAM Services model on SSA-S2P, certain liberties were taken as shown in the following.

- The Send SCSI Command Protocol Service is missing an optional input parameter entitled Command Byte Count.  This is a recognized error in SAM rev 16, and will be corrected in SAM rev 17.

- Any implementation must have a know parameter list.  Therefore, I use a test for null to determine if a parameter is included.

- The SAM Services model (a logical model) assumes data is queued and sent with the service request or confirmation.  In an actual implementation, pointers to host or device memory will be sent and data will be written to or read from memory below the SAM layer.  There the data structures are replaced with memory address pointers, and the names changed to add Address to the name.

### The SAM/Protocol/Transport layers

Here is a partial list of the major responsibilities of the SAM, Protocol, and Transport layers.

The SAM layer is responsible for SCSI Command generation, queuing, and execution.  SAM also handles sense data building, Contingent Allegiance or Auto Contingent Allegiance conditions, and command termination.

The Protocol layer provides the function to map the SAM Protocol services onto the SSA-TL1 transport layer.  In addition, the Protocol layer maintains a data structure of all outstanding commands to allow for deadlock avoidance and error handling in case of illegal operations resulting in a deadlock situation.

The transport layer provides the function to map the Transport Services onto the SSA-PH1 physical environment.  It also provides procedures for configuration, error reporting, error handling, and configuration management.  The Transport layer also converts the Protocol Unique ID into an Initiator ID used by some SMSs by the use of the Initiator table.

### Implementation Specific Options

The SSA protocol and transport services have several implementation specific features as described below.

- The transport layer is responsible for controlling hardware and allocating data transfer resources, including Ichannel and Tchannel values.  If sufficient resources exist to allocate them at Command time (Send Command protocol service) rather than Data Transfer time(Initiator Read or Initiator Write transfer services), then overhead can be reduced during command overhead (Refer to the ddrm and ichannel fields of the SCSI Command SMS).

- The willingness to accept data transfers which are out of order (non-contiguous) for a command can improve performance significantly (Refer to the oot field of the SCSI COMMAND SMS).

- The Initiator protocol layer is responsible for maintaining a data structure for all outstanding commands (See 5.2.2 for more details).

- SAM does not control the use of alternate paths, but does require that command be received in order.  If the implementation can control this ordering and still use multiple paths to a device, then the transport layer is responsible for this function.

## SCSI Command Model

### SAM / Protocol / Transport activity for command execution

Table 1 shows the typical services activity to execute a command that does not involve data transfer, where ==> and <== indicate direction.

Table 1 - Command execution services activity

| Initiator |  |  |  | Cable | Target |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAM | Protocol |  | Transport | Cable | Transport | Protocol |  | SAM |
| Protocol Services |  | Transport Services |  | Cable | Transport Services |  | Protocol Services |  |
| ==> Send SCSI Command |  | ==> SEND SMS |  | ==> SCSI Command SMS | ==> SMS RECEIVED |  | ==> SCSI Command Received |  |
| <== Command Complete Received |  | <== SMS RECEIVED |  | <== SCSI Status SMS | <== SEND SMS |  | <== Send Command Complete |  |

### Send SCSI Command protocol service request

Send SCSI Command (Task Address, CDB, [Task Attribute], [Data-Out Buffer Address], [Data-In Buffer Address], [Command Byte Count], [Autosense Request] ||)

The protocol layer shall maintain a data structure entitled outstanding commands of all outstanding commands as shown below.  A command is outstanding after the Send Command protocol service request is received until the Send Command Complete service response is sent.  In addition a single bit Buffer Full Condition Flag for the entire data structure will be maintained for deadlock avoidance.

Editor’s note: Is the Buffer Full indication by Initiator or by the entire command queue.  Some devices allocate a few queue slots to each Initiator and the remainder of the queue slots are in a general pool.

- The Task Identifier parameter.

- The Data-Out Buffer Address parameter if present.

- The Data-In Buffer Address parameter if present.

- The Command Byte Count parameter if present.

Upon receipt of the Send SCSI Command protocol Service request, the protocol layer will construct an SCSI COMMAND SMS and will instruct the transport layer to transmit it.

The SSA-S2P protocol will construct a SCSI COMMAND SMS into a data structure entitled sms structure as follows:

- The ulp code and sms code fields are set to 82h and 10h respectively.

- The Tag and Logical Unit Identifier of the Task Address parameter go in the tag and luntrn fields respectively.

- The initiator id field is the Initiator ID parameter from the invocation of the GET INITIATOR ID (Initiator Identifier, Port || Initiator ID, Return Code) Transport service.  The Initiator Identifier parameter is part of the Task Address parameter.

- The CDB parameter goes in the cdb field.

- The Task Attribute parameter goes in the queue cntl field (the SIMPLE value is used if the Task Attribute parameter is null).

- The use of the ddrm, ichannel, and oot fields is implementation dependent.

- The use of the resume field depends on the status of the buffer full condition (See 5.2.6).

The Autosense parameter is ignored in the SSA-S2P protocol

The SSA-S2P protocol will then invoke the transport layer service of SEND SMS as follows.

SEND SMS (Control, Initiator Identifier, Data Field Buffer Address, Data Field Length || Return Code)

- The control field is always 00h.

- The Initiator Identifier part of the Task Address parameter is passed as the Initiator Identifier parameter.

- The Data Field Buffer Address parameter is the location of the sms structure data structure.

- The Data Field Length parameter is the length of the sms structure data structure.

The transport layer will perform the following actions as a result:

- Construct a frame with a data field of sms structure data structure.

- Construct the address field for the frame using the return path found in the Initiator Table pointed to by Initiator Unique ID parameter, and the Channel parameter.

- Construct the control field of the frame from the Control parameter.

- Send the frame and return the status (FUNCTION SUCCESSFUL or FUNCTION FAILED) in the Return Code parameter.

If the Return Code parameter indicates FUNCTION FAILED or INVALID PARAMETER, then the protocol layer will generate a Command Complete Received protocol service confirmation with a Service Response parameter value of SERVICE DELIVERY OR TARGET FAILURE.

### SCSI Command received protocol service indication

SCSI Command Received (Task Address, Task Attribute, CDB ||)

When the transport layer generates an SMS RECEIVED transport service request, and the SMS is validated as a SCSI Command SMS, then a SCSI Command Received Protocol service indication is generated as follows.

- The tag and luntrn fields of the SCSI COMMAND SMS are combined and returned as the Task Address parameter.

- The Initiator Identifier part of the Task Address is the Unique ID parameter from the invocation of the GET UNIQUE ID (Initiator ID, Port || Unique ID, Return Code) Transport service.  The Initiator ID parameter is taken from the initiator id field of the SCSI COMMAND SMS

- The queue cntl field of the SCSI COMMAND SMS is returned as the Task Attribute parameter.

- The cdb field of the SCSI COMMAND SMS is returned as the CDB parameter.

### Send Command Complete protocol service response

Send Command Complete (Task Identifier, Status, Service Response ||)

Upon receipt of the Send Command Compete protocol service, the protocol will perform the following actions.

The SSA-S2P protocol will construct a SCSI STATUS SMS into a data structure entitled sms structure as follows:

- The ulp code and sms code fields are set to 82h and 10h respectively.

- The Tag portion of the Task Identifier parameter goes in the tag field.

- The Status parameter goes into the status field.

- The Service Response parameter will cause the link and flag fields to be set as defined in Table 2.

Table 2 - Service Response parameter and its impact on flag and link fields

| Service Response parameter | flag field | link field |
| --- | --- | --- |
| Task Complete | 0b | 0b |
| Linked Command Complete | 0b | 1b |
| Linked Command Complete (with flag) | 1b | 1b |
| Service Delivery of Target Failure | 0b | 0b |

The SSA-S2P protocol will then invoke the transport layer service of:

SEND SMS (Control, Destination ID, Data Field Buffer Address, Data Field Length || Return Code)

- The control field is always 00h.

- The Initiator Identifier part of the Task Address parameter is passed as the Destination ID parameter.

- The Data Field Buffer Address parameter is the location of the sms structure data structure.

- The Data Field Length parameter is the length of the sms structure data structure.

The transport layer will perform the following actions as a result:

- Construct a frame with a data field of SMS Structure parameter.

- Construct the address field for the frame using the return path found in the Initiator Table pointed to by Destination ID parameter, and a Channel of 00h.

- Construct the control field of the frame from the Control parameter.

- Send the frame and return the status (FUNCTION SUCCESSFUL or FUNCTION FAILED) in the Return Code parameter.

If the Return Code parameter indicates FUNCTION FAILED or INVALID PARAMETER, then the protocol layer will reissue the SEND SMS to the transport layer at a later time (hopefully after error recovery has corrected the problem).

### Command Complete Received protocol service confirmation

Command Complete Received (Task Address, [Data-In Buffer Address], Status, Service Response ||)

When the transport layer generates an SMS RECEIVED transport service request, and the SMS is validated as a SCSI STATUS SMS, then a Command Complete Received protocol service indication is generated as follows.

- The tag and luntrn fields of the SCSI COMMAND SMS are combined and returned as the Task Address parameter.

- The Data-In Buffer Address from the outstanding commands data structure is returned as an optional Data-In Buffer Address parameter.

- The status field is returned as the Status parameter.

- The Service Response parameter shall be returned with TASK COMPLETE.

If any of the following transport services returns a FUNCTION FAILED Return Code parameter, then the protocol will generate a Command Complete Received protocol service indication.

- SEND SMS associated with the Send SCSI Command protocol service request.

- INITIATOR READ parameters are invalid.

- Initiator Write parameters are invalid.

In any of the previous cases, then a Command Complete Received protocol service indication is generated as follows.

- The tag and luntrn fields of the SCSI COMMAND SMS are combined and returned as the Task Address parameter.

- The Data-In Buffer parameter is not used.

- The Status parameter is not used.

- The Service Response parameter shall be returned with DELIVERY OR TARGET FAILURE.

### Deadlock avoidance

A potential deadlock situation could occur in SSA since resources are limited in nodes, and SMSs are unsolicited.  Deadlock is not possible with data transfers, since they are solicited with the use of data transfer SMSs.  Deadlock avoidance is a process whereby deadlock situations on the SSA web are avoided.

Deadlock avoidance consists of two parts as described in the following paragraphs.

First, the number of non-SCSI COMMAND SMSs are limited and sufficient resources exist for them.  Both the Transport and Protocol layers are responsible for this.  The Protocol layer simply holds any service that would send a non-SCSI COMMAND SMS while any non-SCSI COMMANDS SMSs are outstanding.

Editor’s note: Should we have a time-out?

Secondly, SCSI COMMAND SMS resource limitations are controlled by the use of Queue Full and the Protocols outstanding commands data structure.  When the Protocol layer receives a SCSI STATUS SMS with status field of QUEUE FULL, it generates a Command Complete Received protocol service response for all commands in its outstanding commands data structure.  The outstanding commands data structure is naturally cleared by this operation, and the Protocol layer sets the Buffer Full Condition Flag to 1b (The flag is initialized to 0b upon power up or a reset condition).  When the Buffer Full Condition flag is set to 1b, and a Send SCSI Command protocol service request is received, the SCSI COMMAND SMS generated will have the resume field set to 1b, and the Buffer Full Condition Flag will be set to 0b.

## Data Transfer Protocol Services

Table 3 shows the typical services activity to transmit Data-In, where ==> or <== indicates direction.

Table 3 - Data-In services activity

| Initiator |  |  |  | Cable | Target |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAM | Protocol |  | Transport | Cable | Transport | Protocol |  | SAM |
| Protocol Services |  | Transport Services |  | Cable | Transport Services |  | Protocol Services |  |
|  |  | Initiator Read <== |  | DATA READY <== ==> DATA REPLY data<== | Target Read <== |  | Send Data-In <== |  |
|  |  |  |  |  | Read Data Completed ==> |  | Data Received ==> |  |

Table 4 shows the typical services activity to transmit Data-Out, where ==> or <== indicates direction.

Table 4 - Data-Out services activity

| Initiator |  |  |  | Cable | Target |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAM | Protocol |  | Transport | Cable | Transport | Protocol |  | SAM |
| Protocol Services |  | Transport Services |  | Cable | Transport Services |  | Protocol Services |  |
|  |  | Initiator Write <== |  | DATA REQUEST<== ==>data | Target Write <== |  | Receive Data Out <== |  |
|  |  |  |  |  | Write Data Completed ==> |  | Data Out Received ==> |  |

### Send Data-In protocol service request

Send Data-In (Task Identifier, Device Server Buffer, Application Client Buffer Offset, Request Byte Count ||)

Upon receipt of the Send Data-In protocol service request, the protocol will invoke the following transport service.

Target Read (Destination ID, Port, Tag, Buffer Address, Byte Offset, Byte Count, Start Count, Threshold Count || Return Code)

- The Tag portion of the Task Identifier becomes the Tag parameter

- The Initiator ID portion of the Task Identifier becomes the Destination ID parameter

- The Device Server Buffer becomes the Begin Buffer Address parameter

- The Application Client Buffer Offset becomes the Byte Offset parameter

- The Request Byte Count becomes the Byte Count parameter

- The use of the Start Length and Threshold Length parameters are implementation dependent.  These parameters can be used when the target does not have Byte Count worth of data in its buffer at the time of invocation (but it does have Start Count), and will use the transport layer to throttle the data transfer.  Refer to the Target Read service in the SSA-TL1 specification for more information on this function.

If the Return Code parameter indicates FUNCTION FAILED, then the protocol layer will generate a Command Complete Received protocol service confirmation with a Service Response parameter value of SERVICE DELIVERY OR TARGET FAILURE.  If the Return Code parameter indicates FUNCTION SUCCESSFUL, then the

The invocation of the Target Read transport service will cause the Initiator protocol to receive an Initiator Read transport service request.  This will cause the Initiator protocol to perform the following functions.

Initiator Read (Tag, Byte Offset, Byte Count || Initiator Read Buffer Address, Start Count, Threshold Count, Return Code)

- If the Command Byte Count or Buffer Address entries referenced by the Tag parameter in the outstanding commands data structure are null, then return FUNCTION FAILED in the Return Code parameter.

- If the Byte Offset plus the Byte Count exceeds the Command Byte Count entry referenced by the Tag parameter in the outstanding commands data structure, then return FUNCTION FAILED in the Return Code parameter.

- If neither of the conditions in a) nor b) occurred, then set up the data transfer to begin at the location pointed to by the Buffer Address entry in the outstanding commands data structure plus the Byte Offset parameter.  The length of the transfer shall be the Byte Count parameter.

- The Protocol layer returns the Initiator Read Buffer Address parameter from the Data-In Buffer Address parameter of the appropriate entry in the outstanding commands data structure.

- The use of the Start Length and Threshold Length parameters are implementation dependent.  These parameters can be used when the initiator does not have Byte Count worth of buffer space at the time of receipt of this service (but it does have Start Count), and will use the transport layer to throttle the data transfer.  Refer to the Initiator Read service in the SSA-TL1 specification for more information on this function.

### Data Delivered protocol service confirmation

Data Delivered (Task Identifier ||)

When the data transfer initiated by the Send Data-In protocol service request completes, the transport layer will inform the protocol layer with a Read Data Complete transport service and return a Tag value, which will cause the protocol to invoke a Data Delivered protocol service with a Task Identifier parameter.

### Receive Data-Out protocol service request

Receive Data-Out (Task Identifier, Application Client Buffer Offset, Request Byte Count, Device Server Buffer ||)

Upon receipt of the Receive Data-Out protocol service request, the protocol will invoke the following transport service.

Target Write (Destination ID, Port, Tag, Buffer Address, Byte Offset, Byte Count || Initiator Write Buffer Address, Return Code)

- The Initiator Identifier portion of Task Identifier becomes the Destination ID parameter

- The Tag portion of the Task Identifier becomes the Tag parameter

- The Device Server Buffer becomes the Start Buffer Address parameter

- The Application Client Buffer Offset becomes the Byte Offset parameter

- The Request Byte Count becomes the Byte Count parameter

If the Return Code parameter indicates FUNCTION FAILED, then the protocol layer will generate a Command Complete Received protocol service confirmation with a Service Response parameter value of SERVICE DELIVERY OR TARGET FAILURE.  If the Return Code parameter indicates FUNCTION SUCCESSFUL, then the

The invocation of the Target Write transport service will cause the Initiator protocol to receive an Initiator Write transport service request.  This will cause the Initiator protocol to perform the following functions.

Initiator Write (Tag, Byte Offset, Byte Count ||Initiator Write Buffer Address, Return Code)

- If the Command Byte Count or Buffer Address entries referenced by the Tag parameter in the outstanding commands data structure are null, then return FUNCTION FAILED in the Return Code parameter.

- If the Byte Offset plus the Byte Count exceeds the Command Byte Count entry referenced by the Tag parameter in the outstanding commands data structure, then return FUNCTION FAILED in the Return Code parameter.

- The Protocol layer returns the Initiator Write Buffer Address parameter from the Data-Out Buffer Address parameter of the appropriate entry in the outstanding commands data structure.

- If neither of the conditions in a) nor b) occurred, then set up the data transfer to begin at the location pointed to by the Initiator Write Buffer Address plus the Byte Offset parameter.  The length of the transfer shall be the Byte Count parameter.

### Data-Out Received protocol service confirmation

Data-Out Received (Task Identifier ||)

When the data transfer initiated by the Receive Data-Out protocol service request completes, the transport layer will inform the protocol layer with a Write Data Complete transport service and return a Tag value, which will cause the protocol to invoke a Data-Out Delivered protocol service with a Task Identifier parameter.

## Task Management Functions

This section describes the common actions of all Task Management functions with the exception of the Terminate Task function, which always returns a Service Response of FUNCTION REJECTED.

Upon receipt of the Task Management Function protocol Service request, the protocol layer will construct a partial SMS and will instruct the transport layer to complete it and transmit it.  The type of SMS depends on the Task Management function, and will be detailed in each Task Management section.

In all cases, the SSA-S2P protocol will then invoke the transport layer service of SEND SMS as follows.

SEND SMS (Control, Initiator Unique ID, Channel, Data Field Buffer Address, Data Field Length || Return Code)

- The control field is always 00h.

- The Initiator Identifier part of the Task Address parameter is passed as the Initiator Unique ID parameter.

- The Channel parameter is always 00h.

- The Data Field Buffer Address parameter is the location of the sms structure data structure.

- The Data Field Length parameter is the length of the sms structure data structure.

The transport layer will perform the following actions as a result:

- Construct a frame with a data field of sms structure data structure.

- Replace bytes 4 through 7 of the data field with the initiator id found in the Initiator Table pointed to by the Initiator Unique ID parameter.

- Construct the address field for the frame using the return path found in the Initiator Table pointed to by Initiator Unique ID parameter, and the Channel parameter.

- Construct the control field of the frame from the Control parameter.

- Send the frame and return the status (FUNCTION SUCCESSFUL or FUNCTION FAILED) in the Return Code parameter.

If the Return Code parameter indicates FUNCTION FAILED, then the protocol layer Service Response value of SERVICE DELIVERY OR TARGET FAILURE. If the Return Code parameter indicates FUNCTION SUCCESSFUL, then the protocol layer Service Response value of FUNCTION COMPLETE.

It is the responsibility of the protocol to make sure that only one Task Management function is outstanding at a time from any given Initiator.  Additional Task Management functions received while another Task Management function is active from that initiator will be held until the outstanding Task Management function completes.

### Abort Task

Service Response = ABORT TASK (Task Address ||)

The SSA-S2P protocol will construct an ABORT TAG SMS into a data structure entitled sms structure as follows:

- The ulp code and sms code fields are set to 82h and 30h respectively.

- The tag field is set to 00h.

- The Tag of the Task Address parameter is passed in the tag2 field.

### Abort Task Set

Service Response = ABORT TASK SET (Logical Unit Identifier ||)

The SSA-S2P protocol will construct an ABORT SMS into a data structure entitled sms structure as follows:

- The ulp code and sms code fields are set to 82h and 31h respectively.

- The tag field is set to 00h.

- The Logical Unit Identifier parameter is passed in the luntrn field.

### Clear ACA

Service Response = CLEAR ACA (Logical Unit Identifier ||)

The SSA-S2P protocol will construct a CLEAR ACA SMS into a data structure entitled sms structure as follows:

- The ulp code and sms code fields are set to 82h and 34h respectively.

- The tag field is set to 00h.

- The Logical Unit Identifier parameter is passed in the luntrn field.

### Clear Task Set

Service Response = CLEAR TASK SET (Logical Unit Identifier ||)

The SSA-S2P protocol will construct a CLEAR QUEUE SMS into a data structure entitled sms structure as follows:

- The ulp code and sms code fields are set to 82h and 32h respectively.

- The tag field is set to 00h.

- The Logical Unit Identifier parameter is passed in the luntrn field.

### Target Reset

Service Response = TARGET RESET (Target Identifier ||)

The SSA-S2P protocol will construct a TARGET RESET SMS into a data structure entitled sms structure as follows:

- The ulp code and sms code fields are set to 82h and 33h respectively.

- The tag field is set to 00h.

### Terminate Task

Service Response = TERMINATE TASK (Task Address ||)

The terminate Task protocol service is optional in SAM and is not supported in SSA-S2P.  The Service Response is always unconditionally returned as FUNCTION REJECTED.

## Task Management Protocol Services

### Send Task Management Request (Object Address, Function Identifier ||)

Upon receipt of the Send Task Management Request protocol service, the protocol layer will perform the actions defined in the appropriate Task Management Function section.

### Task Management Request Received (Object Address, Function Identifier ||)

Upon receipt of an SMS RECEIVED transport service, and the SMS is validated as a Task Management SMS, the protocol will generate a Task Management Request received protocol service with the appropriate parameters.

### Task Management Function Executed (Object Address, Service Response ||)

Upon receipt of a Task Management Function Executed protocol service, the protocol layer will construct a partial SCSI RESPONSE SMS and will instruct the transport layer to complete it and transmit it.

The SSA-S2P protocol will construct a partial SCSI Response SMS into a data structure entitled sms structure as follows.

- The ulp code and sms code fields are set to 82h and 03h respectively.

- The tag field is set to 00h.

- The return code field shall be set to value based on the value of Service Response as shown in Table 5.

Table 5 - return code field value based on Service Response

| Service Response parameter value | return code field value |
| --- | --- |
| FUNCTION REJECTED | ??? |
| FUNCTION COMPLETE | REQUESTED FUNCTION WAS COMPLETED SUCCESSFULLY |
| SERVICE DELIVERY OR TARGET FAILURE | ??? |

The SSA-S2P protocol will then invoke the transport layer service of SEND SMS as follows.

SEND SMS (Control, Initiator Unique ID, Channel, Data Field Buffer Address, Data Field Length || Return Code)

- The control field is always 00h.

- The Initiator Identifier part of the Task Address parameter is passed as the Initiator Unique ID parameter.

- The Channel parameter is always 00h.

- The Data Field Buffer Address parameter is the location of the sms structure data structure.

- The Data Field Length parameter is the length of the sms structure data structure.

The transport layer will perform the following actions as a result:

- Construct a frame with a data field of sms structure data structure.

- Replace bytes 4 through 7 of the data field with the initiator id found in the Initiator Table pointed to by the Initiator Unique ID parameter.

- Construct the address field for the frame using the return path found in the Initiator Table pointed to by Initiator Unique ID parameter, and the Channel parameter.

- Construct the control field of the frame from the Control parameter.

- Send the frame and return the status (FUNCTION SUCCESSFUL or FUNCTION FAILED) in the Return Code parameter.

### Received Function-Executed (Object Address, Service Response ||)

Upon receipt of an SMS RECEIVED transport service, and the SMS is validated as a SCSI RESPONSE SMS, the protocol will generate a Received Function Executed protocol service with the appropriate parameters.  The return code field maps into the Service Response parameter as defined in Table 5.