Accredited Standards Committee

X3, Information Processing Systems

Doc: X3T10.1/95a142R0

Date: April 17, 1995

Project:  UIG-95 Documents

Ref Doc:  UIG95PH-9509

UIG95SP-9508

Reply to: Ed Carmona

To:	X3T10.1 Membership

From:	Ed Carmona

Subject:	Comments on UIG-95 Documents

UIG-95PH, Revision 0 Comments

GENERAL ISSUES:

1.No section numbers appear in the document, yet the table of contents shows section numbers. Also, references in the document to sections have all defaulted to section 0.  Section numbers should be added in the document.

2.Document references “primary initiators” instead of “initiators”. This should be changed. This was approved in the SSA Plenary meeting in Houston, 1994.

3.Document references “networks” instead of “webs”. Should this be changed? Do we want to be somewhat consistent in     terminology with the standard TL document?

4.Document references “messages” instead of “SMSs”. This needs to be changed to be consistent with the UIG-95SP document.

SPECIFIC ISSUES:

1.Abstract section, 1st page: "Serial SCSI-3 Architecture" should be replaced with "Serial Storage Architecture".

2.Introduction section, 2nd paragraph: There are several issues concerning this paragraph including incorrect references to document, the ANSI committee, and a reference to a SCSI-3 document which should not be referenced by this document. Therefore the paragraph should be re-written as follows:

"SSA defines a serial interface for use within future storage sub-systems. It is divided into a common transport layer and several alternative upper-level protocols. This standard concerns itself only with the following documents.

UIG-95PH	The physical transport layer (this document)

UIG-95SP  	UIG-95SP is a mapping of the existing SCSI-2 protocol, as defined by the ANSI X3T10 committee for a parallel bus, with extensions to map SCSI-2 to the SSA serial interface."

3.Networks section, 3rd bullet item: "Switch (4-126   ports)" should be changed to "Switch (4-96 ports)".

4.Networks section, Switches sub-section, 2nd paragraph: The number of ports that can be addressed by a switch needs to change from "126" to "96". In the example given, the number of ports addressed needs to change from "118" to "88".

5.Protocol section, Address field sub-section, Switch node, 1st paragraph: The number of ports needs to change from "126" to "96".

6.Link management section, Port states sub-section, Enabled, 2nd paragraph: Reference to section number is not valid.

7.Link management section, Port modes sub-section, Normal mode, 2nd paragraph: Reference to section number is not valid.

8.Link management section, Port operational sub-section, 1st bullet item: Reference to section number is not valid.

9.Link management section, Beginning communication sub-section, Items 7 & 10: Reference to section number is not valid.

10.Link management section, Resets sub-section, Local reset, 1st paragraph: Reference to section number is not valid.

11.Link management section, Resets sub-section, Total/Absolute reset, Item 6: Reference to section number is not valid. Also, in last paragraph, 1st sentence, the word "receive" needs to be replaced with "receives".

12.Link management section, Resets sub-section, Network reset, 1st set of bullet items, Item 2: Reference to section number is not valid.

13.Link management section, Resets sub-section, Link reset, 1st paragraph: Reference to section number is not valid.

14.Link management section, Configuration sub-section, List of SSA PH messages: References to section numbers are not valid.

15.Link management section, Configuration sub-section, Master node, 1st paragraph: Reference to section number is not valid.

16.Link management section, Configuration sub-section, Master node, 1st paragraph: Reference to section number is not valid.

17.Link management section, Configuration sub-section, Configuration table, 5th paragraph: Reference to section number is not valid.

18.Error handling section, Link ERP sub-section, Asynchronous alerts, List of SSA PH messages: References to section numbers are not valid.

19.Error handling section, Link ERP sub-section, Asynchronous alerts, Invalid messages: This section needs clarification on how to handle the specific errors it is trying to deal with. The section references 'Message Reject' which is no longer a valid Async_alert code. The section also does no clarify what to do with messages with an unknown Initiator_IDs/Return_paths, and unknown ULP code. To clarify all this, the paragraph should be re-written as follows:

"If a destination node receives an invalid message, the node returns either a Response message to the Initiator or Async_alert message to the Master depending on the type of invalid message received. The list below identifies the invalid messages detected by the destination node and the appropriate action taken:

Invalid Return_path/Initiator_ID - Node returns an Async_alert specifying 'Unknown Return_path or Initiator_ID'. A node should not check this field when receiving a Query_node/Query_node_reply message.

Invalid Message code - Node returns an Async_alert specifying 'Unknown messagecode'.

Invalid ULP code - Node returns an Async_alert specifying 'Unknown message code'.

Invalid message length(too short) - Node returns an Async_alertspecifying 'Message too short'.

Invalid message parameter - Node returns a Response specifying 'Invalid Parameter'.

If node needs to generate an Async_alert in response to an invalid message, then the node proceeds as follows:

1. It generates an Async_alert specifying the appropriate alert code. The port remains in Normal mode. The Master returns an Async_reply"

The remain items listed in this section remain the same.

20.Table 7., Type/Subtype/Type_info fields  table: Need to align items correctly.

21.Privileged messages section, Async_alert sub-section, Return_code: Spelling correction, "unkonwn" should be "unknown".

22.Privileged messages section, Configure_port sub-section, List of Return_codes: 03h, Unknown Return_path is now an Async_alert code. Remove this item as a valid Return_code.

23.Privileged messages section, Query_node_reply sub-section, 1st paragraph: Reference to section number is not valid.

24.Privileged messages section, Response sub-section, List of Return_codes: 03h, Unknown Return_path is now an Async_alert code. Remove this item as a valid Return_code and add it to the list of Reserved Return_codes.

25.Privileged messages section, Data_reply sub-section, 2nd & last paragraph: The handling of an invalid Data_replymessage is inconsistent with respect to the Return_codes and Async_alert codes defined. The 'Unknown Return_path' code is now an Async_alert and the 'Protocol Error' does not exist.

The last two paragraphs should be re-written as follows:

"In the case of a node receiving an invalid or unexpected Data_reply message, the node will generate an Async_alert to the Master. The list below identifies the invalid/unexpected conditions detected by the  node and the appropriate action taken:

Unexpected Data_reply received - Node received a Data_reply with no Data_ready message sent. The node returns an Async_alert specifying 'Unexpected Data_reply message'.

Unexpected data frame received - Node received a data frame when no more data or no data was expected. The node returns an Async_alert specifying 'Unexpected data frame'.

Invalid number of Data_replys received - Node received an invalid number of Data_reply messages. The node returns an Async_alert specifying'Too many outstanding Data_replys'.

Invalid Initiator_ID - Node received a Data_reply with an invalidInitiator_ID. The node returns an Async_alert specifying 'Unknown Return_path or Initiator_ID'.

Invalid Data_reply parameter - Node received a Data_reply with an invalid parameter (Tag, Byte_count, IChannel, Transfer_ID). The node returns an  Async_alert specifying 'Invalid field'.

The associated I/O process identified by the Tag value (if any) is not terminated, instead the Data_reply is simply rejected with a Async_alert message."

UIG-95SP, Revision 0 Comments

GENERAL ISSUES:

1.Document references SSA-PH throughout. We need to be careful not to confuse the reader when referencing the UIG-95PH document. References to the "SSA-PH" need to be replaced with "UIG-95PH".

SPECIFIC ISSUES:

1.Section 3.2: Align LUNTAR & LUNTRN descriptions properly.

2.Section 4: There are several incorrect references to documents and a references to documents outside the scope of the UIG-95 documents should not be used. Therefore the paragraph should be re-written as follows:

"SSA defines a serial interface for use within future storage sub-systems. It is divided into a common transport layer and several alternative upper-level protocols. This standard concerns itself only with the following documents.

UIG-95PH  The physical transport layer.

UIG-95SP  UIG-95SP is a mapping of the existing SCSI-2 protocol, as defined by the ANSI X3T10 committee for a parallel bus, with extensions  to map SCSI-2 to the SSA  serial interface(this document)."

3. Section 5.10.3, Item1: The alert code description 'UNKNOWN INITIATOR ID' should be replaced with 'UNKNOWN RETURN PATH OR INITIATOR ID' as specified in UIG-95PH.

Ed Carmona

Conner Peripherals, Inc.

Voice: (408) 456-4082

Email: ed.carmona@conner.com