Accredited Standards Committee

X3, Information Processing Systems

Doc: 	X3T10.1/95a141R1

Date:	July 28, 2026

Project: 	N/A

Ref Doc:	X3T10.1/989D Rev 4

X3T10.1/1121DT Rev 1

Reply to:	Ed Carmona

To: 	X3T10.1 Membership

From: 	Ed Carmona

Subject: Proposed Solution to SSA Deadlock

BACKGROUND

This problem was first identified by Ed Gardner.  A specific example is shown below:

IA = Initiator A, IB = Initiator B

TA = Target A, TB = Target B

IA ====> TB <====> TA <====IB

IA sends SCSI commands to TA	IB sends SCSI commands to TB

TA sends SCSI status to IA		TB sends SCSI status to IB

Initiator A rapidly sends a large number of SCSI commands to Target A and exhausts the Target's queue space.  Target A must then temporarily withhold RR to throttle the inbound messages.  Target A will also generate a SCSI status message indicating 'Queue full' for each subsequent SCSI command message.  The same scenario can simultaneously occur for Initiator B and Target B.  This may result in deadlock with each Target waiting for the other to send RR. However neither Target can send an RR until it has been able to send a SCSI status message and release a space in its queue.

This problem arises due to a combination of 2 features in SSA:

- The SCSI command message is unsolicited, i.e. an Initiator is free to send it at will.  (The problem cannot occur with data because all SSA data transfers are solicited.)

- The SSA-PH communication protocol is local to each link.  For example, SSA-PH does not provide any end-to-end pacing to allow the Target to throttle the generation of messages.  (This feature is central to the concept of spatial reuse; shared media technologies such as parallel busses, P1394 and FC-AL have end-to-end pacing but they do not allow spatial reuse.)

PROPOSED SOLUTION

This proposal is based on a solution proposed by Ian Judd. A summary of the solution is given below.

When a Target has no room to store a SCSI command SMS in the command

queue, it enters a mode where it discards SCSI command SMSs from that Initiator.  The Target also returns a SCSI status SMS indicating QUEUE FULL which contains the first Tag discarded.  Since the Target only needs to remember the first Tag discarded, it continues to receive inbound messages and it does not withhold RR.  The Initiator must keep a record of the order in which it issued the Tags so that it can identify the discarded commands and reissue them later (with an appropriate throttling mechanism to reduce the probability of filling the queue again.)

To prevent a Target from being overrun with SSA-PH SMSs, an Initiator can have only one outstanding SSA-PH SMS at a time on a given Target. These SMSs include the following: Configure port, Master alert, Query port,  Query protocol, and Quiesce.

The QUERY NODE SMS is an exception. A Target is required to provide storage for a minimum number of QUERY NODE SMS; however, if more are received than there is room, the Target will discard the extra QUERY NODE SMSs without generating a response. An Initiator will be required  to provide a time-out and retry the Query node if no Query node reply is received.

To prevent a Target from being overrun with the remaining SSA-S2P SMSs, an Initiator can have only one outstanding SSA-S2P SMS at a time on a given Target  (excluding the SCSI command SMS). These SMSs include the following: ABORT, ABORT TAG, CLEAR QUEUE, DEVICE RESET, and CLEAR ACA CONDITION SMS.

PROPOSAL

Changes to SSA-TL1, X3T10.1/989D Rev 4

1) Change clause 11.1.1, SMS byte 0 validation to the following

"5)	If byte 0 of the SMS is supported in Table 13, byte 0 corresponds to the Configure port, Master alert, Query port,  Query protocol, or Quiesce SMS, and one of these SMSs is already outstanding in the node from this Initiator, then generate a RESPONSE SMS with an OVERLAPPED SMSs ATTEMPTED return code.

6) If  the SMS is a QUERY NODE, and the node has no room to store the SMS, the  node shall  discard the QUERY NODE SMS. The node shall not generate any type of response. An Initiator shall employ a time-out and may retry the QUERY NODE if no QUERY NODE REPLY is received.

7) 	If Table 13 matches the byte 0 of the SMS, the FRAME_TYPE of the SMS, and the node type, then continue processing the SMS as defined in the appropriate sections of clauses 11 or 12."

2) Add the following text to clause 10.6, Communication Blocked, after the last paragraph.

"Initiators and Targets shall avoid deadlock conditions by conforming to the requirements below.

- An Initiator shall only have one Configure port, Master alert, Query port,  Query protocol or Quiesce SMS outstanding to a given destination node at a time. In addition, an Initiator shall only issue such a message if it has previously successfully registered with the destination node (i.e. the destination has accepted a QUERY NODE SMS with DR = 0b).

- Each node which is required to support an Initiator table shall provide space to queue one Configure port, Master alert, Query port,  Query protocol or Quiesce SMS for each entry in its Initiator table. It is recommended that each node maintain a minimum of 64 Initiator table entries.

- The Master shall provide space to queue one ASYNC ALERT for each port in the network.  It is recommended that the Master maintain a minimum of 256 ASYNC ALERT entries.

- Each node shall provide space to queue at a minimum 4 QUERY NODE SMSs. If a node receives a QUERY NODE SMS when no room to store the SMS is available, the node shall simply discard the QUERY NODE SMS without generating a response. An Initiator shall employ a time-out and may retry the QUERY NODE if no QUERY NODE REPLY SMS is received."

3) Add the following return code to Table 30, Return Code Values, clause 11.2.12,

| Type | Description |
| --- | --- |
| 04h | Overlapped SMSs Attempted |

Changes to SSA-S2P, X3T10.1/1121DT Rev 1

1) Add the following return code to Table 12, Return Code Values for the SCSI RESPONSE SMS, clause 5.8,

| Value | Description |
| --- | --- |
| 04h | Overlapped SMSs Attempted |

2) Change clause 5.10.1, SMS code validation to the following

)RAME TYPE field does not have a value of APPLICATgenerate a

2

) If  the SMS is a SCSI COMMAND SMS, then node shall perform the following:

a) if the node has no room to store the SMS and the node has not discarded the previous SCSI COMMAND SMS from this Initiator, the node shall discard the SMS and generate a SCSI STATUS SMS indicating QUEUE FULL status.

b) if the node has discarded the previous SCSI COMMAND SMS from this Initiator and the SMS contains Resume = 0b, the node shall discard the SCSI COMMAND SMS even if room to store the SMS has become available. The node shall not generate a response to the discarded SMS.

c) if the node has no room to store the SMS, the node has discarded the previous SCSI COMMAND SMS from this Initiator, and the SMS contains Resume = 1b, the node shall discard the SMS and generate a SCSI STATUS SMS indicating QUEUE FULL status.

d) 	if the node has not discarded the previous SCSI COMMAND SMS from this Initiator and the SMS contains Resume = 1b, the node shall discard the SMS and generate an SCSI RESPONSE SMS with an INVALID FIELD return code.

e) if the node has room to store the SMS, the node has discarded the previous SCSI COMMAND SMS from this Initiator, and the SMS contains Resume = 1b, the node shall continue SMS validation processing as defined below.”

3) Add the following flag to the SCSI COMMAND SMS in Table 2, clause 5.1,

| 10 | DDRM | OOT | RESUME | reserved = 000b | QUEUE_CNTL |
| --- | --- | --- | --- | --- | --- |

4) Add the following text to clause 5.1 SCSI COMMAND after paragraph 8,

"The Initiator normally sets the  Resume = 0b when processing SCSI COMMAND SMSs. After receiving a SCSI STATUS SMS from a node indicating QUEUE FULL status, the Initator shall set the Resume = 1b for the next SCSI COMMAND SMS issued to the node in an attempt to resume SCSI COMMAND SMS processingnode. Rving a SCSI COMMAND SMS with Resume = 1b informs the node to stop discarding SCSI COMMAND SMSs due to the previously detected SMS buffer full condition (see clause 5.11). "

5) Add the following clause, 5.11 SMS SMS Buffer Full Condition

“A node shall generate an SMS buffer full condition when a SCSI COMMAND SMS is received by the node and no room is available to store the SMS. Upon entering an SMS buffer full condition, the node shall discard the SCSI COMMAND SMS and generate a SCSI STATUS SMS indicating a QUEUE FULL status. The node shall then discard subsequent received SCSI COMMAND SMS with Resume = 0b from the same Initiator even if room is available to store the SMS.

The node shall exit the SMS buffer full condition after receiving a SCSI COMMAND SMS with Resume = 1b from the Initiator with the SMS buffer full condition. If the node still has no room to store the SMS, the node shall once again enter the SMS buffer full condition.

When an Initiator receives a SCSI STATUS SMS indicating QUEUE FULL status, the Initiator shall mark the command identified by the SMS and any subsequent commands issued to the node as having been discarded. TSMS buffer full conditionhe next SCSI COMMAND SMS with Resume = 1b.

An Initiator shall only have one ABORT, ABORT TAG, CLEAR QUEUE, DEVICE RESET, or CLEAR ACA CONDITION SMS outstanding to a given Target at a time. A Target shall provide storage to process one of these SMSs for each Initiator

and therefore, these SMSs will never be discarded.”

Ed Carmona

Conner Peripherals, Inc.

Voice: (408) 456-4082

Email: ed.carmona@conner.com