Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a164R0
Date:	June 27, 1995
Project:	
Ref Doc.:	
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: ASYNC REPLY enhancement proposal

BACKGROUND

Since the ASYNC REPLY does not specify which ASYNC ALERT it is associated with, we can get out of sync when an ASYNC ALERT is retransmitted and the original and retransmission is responded to.  For example, see the following example.

- ASYNC ALERT #1 is sent.

- ASYNC ALERT #1R (repeated) is sent due to a time-out

- ASYNC REPLY  for #1 is sent

- ASYNC ALERT #1 and #1R is considered no longer outstanding

- ASYNC ALERT #2 is sent

- ASYNC REPLY for #1R is sent

- ASYNC ALERT #2 is considered no longer outstanding

PROPOSAL

Make the following changes

- Section 11.2.2 (ASYNC REPLY) Table 21 - Add the following...
Byte 4 - type
Byte 5 - subtype
Byte 6 - type information

- Section 11.2.2 (ASYNC REPLY) add the following paragraphs to the end of 11.2.2...

The type field is a copy of the type field from the associated ASYNC ALERT SMS. 

The subtype field is a copy of the subtype field from the associated ASYNC ALERT SMS. 
The type information field is a copy of the type information field from the associated ASYNC ALERT SMS. 

The target shall ignore any ASYNC REPLY SMS with type, subtype and type information fields that do not match those of the outstanding ASYNC ALERT SMS.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com