Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a166R0
Date:	June 27, 1995
Project:	
Ref Doc.:	
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Port table proposal

BACKGROUND

The device is responsible for maintaining information from the CONFIGURE PORT SMS to generate ASYNC ALERT SMS for each port .  This should be documented as a non-binding example similar to the Configuration Table and the Initiator Table.

PROPOSAL

Add the following to the end of section 9.7.

- 9.7.5   Port table

The device is responsible for maintaining the information necessary to generate ASYNC ALERT SMSs.  The information is supplied by the CONFIGURE PORT SMS, and is on a per port basis.

The format of the initiator table is implementation-dependent.  However each entry contains the fields defined in Table 1.

Table 1 - Port table entry

| Field | Description |
| --- | --- |
| aa valid | A flag indicating if the port table entry is valid |
| aa port | The port to be used for the ASYNC ALERT SMS |
| aa return path | The path to be used for the ASYNC ALERT SMS frame |
| aa tag | The tag field value to be used for the ASYNC ALERT SMS |

When the node powers up, the Port table is set up with an entry for each port and with the aa valid flag set to false.  When a CONFIGURE PORT SMS is received, the Port table entry associated with the port that the CONFIGURE PORT SMS was received on is set as follows.

- The aa valid flag is set to true.

- The port field from the CONFIGURE PORT SMS are loaded into aa port field.

- The return path field from the CONFIGURE PORT SMS are loaded into aa return path field.

- The tag field from the CONFIGURE PORT SMS are loaded into aa tag field.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com