Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a180R0
Date:	July 30, 2026
Project:	0989-D
Ref Doc.:	X3T10.1/0989 rev 06
Reply to:	Lawrence J. Lamers

To: 	   	X3T10.1 Membership
From:	Lawrence J. Lamers

Subject: Proposal for Mode Bits

BACKGROUND

REVISIONS

PROPOSAL

The mode field specifes the mode a port is set to or it specifies that a port enters wrap.  Table 25 defines the values of the mode field.

Table 1 - Mode field values

| Value | Description |
| --- | --- |
| 00b | no change to the current mode. |
| 01b | EnterWrap |
| 10b | set Normal mode. |
| 11b | set Privileged mode. |

If a node receives a CONFIGURE PORT SMS with mode field value of NO CHANGE TO THE CURRENT MODE and the specified port has an Asynchronous Alert pending then it shall sends an ASYNC ALERT to the Master before sending the RESPONSE SMS for the CONFIGURE PORT.

A mode field value of ENTER WRAP  is only valid if the node received the CONFIGURE PORT SMS via a different port than that being configured.  Otherwise the node returns a RESPONSE SMS with return code value  of INVALID FIELD and the port mode is unchanged.  When Wrap is set the port also becomes not Operational (see section Error: Reference source not found).