Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95-188R0
Date:	7/30/2026
Project:	
Ref Doc.:	
Reply to:	Lamers/Hallam

To: 	X3T10.1 Membership
From:	Lamers/Hallam

Subject: Minutes of X3T10.1 Working Group : August 27-30,1995

Agenda

- 1. Opening Remarks

- 2. Attendance and Membership, Introductions

- 3. Approval of Agenda

- 4. Approval of  Minutes

- 5. Document Distribution

- 6. Review of Old Action Items

- 7. Protocol Topics

7.1 Review of SSA-S2P working draft [Scheible]

- 8. Transport Layer Topics

8.1 Review of SSA-TL1 working draft [Scheible]

- 9. Physical Topics

9.1 Review on PH Ad Hoc Meeting [Ham]

9.2 LRC [Bouvier]

9.3 Review of PH1 Revision 5a [Ham]

9.4 Clarification of Faults [Kapraun] (95a155r0)

9.5 40 MB/sec SSA-PH

- 10. Call for Patents

- 11. Action Items

- 12. Meeting Schedule

- 13. Adjournment

# Opening Remarks

Ken Hallam convened the meeting at 9:00 am.  He thanked Chris Maartin of Pathlight Technology for hosting the meeting.

As is customary, the people attending introduced themselves.  A copy of the attendance list was circulated for attendance and corrections.

It was stated that the meeting had been authorized by X3T10.1 and would be conducted under the X3 rules.  Ad hoc meetings take no final actions, but prepare recommendations for approval by the X3T10.1 task group.  The voting rules for the meeting are those of the parent committee, X3T10.  For the ad hoc, other than straw votes, the voting rules  are:  one vote per participating company.

The minutes of this meeting will be posted to the X3T10 BBS and the SSA Reflector and will be included in the next X3T10.1 committee mailing.

Ken stated that the X3T10.1 mailings are now part of the X3T10 mailings.  Persons that want to receive documents should subscribe to the X3T10 mailings by sending their request to the secretariat.

# Attendance and Membership, Introductions

Attendance at working group meetings does not count toward minimum attendance requirements for X3T10.1 membership.  Working group meetings are open to any person or company to attend and to express their opinion on the subjects being discussed.

The following people attended the meeting.

| Name | Company | Telephone | Email |
| --- | --- | --- | --- |
| James M. Avery | Adaptec | 303-516-4743 | jima@btc.adaptec.com |
| Jim Comstock | Adaptec | 408-957-7946 | jcomstoc@corp.adaptec.com |
| Lawrence Lamers | Adaptec | 408-957-7817 | ljlamers@aol.com |
| Dr. Robert Selinger | Adaptec | 408 957-7175 | selinger@adaptec.com |
| Bob Atkinson | Amp, Inc | 717-780-4274 | rdatkins@amp.com |
| Charles Brill | Amp, Inc | 717-592-6198 | cebrill@amp.com |
| Michael Wingard | Amphenol Interconnect Products | 607-786-4241 |  |
| Richard Wagner | Cable Design Technologies | 508-791-3161 | rwagner@mcdt3.iii.net |
| Ian Morrell | Circuit Assembly Corp | 714-855-7887 | crctassmbl@aol.com |
| Dhiru N. Desai | Cirrus Logic | 510-226-2144 | ddesai@cirrus.com |
| Ed Carmona | Conner Peripherals, Inc | 408-456-4082 | ed.carmona@conner.com |
| Paul Petersen | Conner Peripherals, Inc | 408-456-4311 | paul.petersen@conner.com |
| Bill Gintz | Conner Peripherals, Inc. | 408-456-3648 | bill.gintz@conner.com |
| Bill Anderson | DDK Electronics | 408-980-8344 | 102501.451@compuserve.com |
| Dan Bouvier | Dell Computer Corp. | 512-728-5316 | bouvierd@ccmail.us.dell.com |
| Tony Tayyar | Dell Computer Corp. | 512-728-2059 | tony_tayyar@ccmail.del.com |
| Bill Ham | Digital Equipment Corp. | 508-841-2629 | ham@subsys.enet.dec.com |
| Charles Monia | Digital Equipment Corp. | 508-841-6757 | monia@shr.dec.com |
| Ken Hallam | ENDL Associates | 714-364-9626 | 3450626@mcimail.com |
| Kevin Pokorney | Fujitsu Computer Products of America | 303-682-6449 | pokorney@intellistor.com |
| Adge Hawes | IBM Corporation | 011-44-1705-486363 |  |
| Ray Muggli | IBM Corporation | 408-256-0988 | rmuggli@vnet.ibm.com |
| Chris Parker | IBM Corporation | 912-892-2719 |  |
| Richard Rolls | IBM Corporation | 408-256-3531 | rrolls@vnet.ibm.com |
| John Scheible | IBM Corporation | 512-823-8208 | scheible@vnet.ibm.com |
| Robert Hasenfratz | ITT Cannon | 49-7151699 285 |  |
| Monica Roy | ITT Cannon | 714-757-8472 | monicar@mailgw.cannon.itt.com |
| Scott Annis | Litton/Winchester Electronics | 203-945-5084 | 73227.1351@compuserve.com |
| Tim Katynski | LSI Logic | 408-433-4175 | Katynski@lsil.com |
| Andrew Lever | LSI-Logic Europe PLC | 01366 413248 | andrewl@sli-logic.co.uk |
| Brian Harvie | Madison Cable | 508-752-2884 |  |
| Phil Bryan | Micropolis | +44 161 973 8359 | Phil_Bryan@microp.com |
| Gary Manchester | Molex | 708-527-4043 |  |
| Scott Murruy | Net Sales Co. | 607-756-6592 |  |
| Mark DeWilde | Pathlight Technology | 607-266-4000 | mark@ironics.com |
| Said Rahmani | Pathlight Technology | 607-226-4000 | saidr@pathlight.com |
| Herb Silverman | Peer Protocols, Inc | 714-476-1016 | herb@cerf.net |
| Ken Erickson | Samsung | 408-434-5589 | kerickso@samsung.com |
| David Deming | Solution Technology | 408-338-4285 | ddeming@scruznet.com |
| Thai Nguyen | Storage Technology Corp. | 303-673-2237 | thai_nguyen@stortek.com |
| Michael Anderson | Symbios Logic Inc. | 719-573-3594 | Michael.Anderson@symbios.com |
| Greg Kapraun | Symbios Logic Inc. | 970-225-4843 | greg.kapraun@symbios.com |
| Pete Tobias | Tandem Computers | 408-285-9913 | tobias_pete@tandem.com |
| Brad Kitson | VLSI Technology, Inc. | 408-434-7553 | Bradk@sanjose.vlsi.com |
| Sam Sanyal | VLSI Technology, Inc. | 408-922-5371 | sanyal_s@sanjose.vlsi.com |
| Paul F. O'Toole | Volex Inc | 508-995-9500 | paulo@volex.com |
| Neil Edmunds | Xyratex | 011-44-1705-486363 | nedmunds@vnet.ibm.com |

# Approval of Agenda

The agenda was approved unanimously as shown on page 1.

# Approval of  Minutes

The minutes of the last Working Group meeting (see 95a162r0) were approved with corrections to the meeting schedule.

# Document Distribution

Larry Lamers stated  that the next mailing deadline is Wednesday September 27, 1995.

# Review of Old Action Items

None.

# Protocol Topics

## Review of SSA-S2P working draft [Scheible]

John Scheible reveiwed the rev 3 document.  In preparing rev 3 several proposals were developed.

Document 95a172r0 as modified to allow use of buffer full ratios was accepted.  Many editorial comments described in 95a173r2 were accepted with modifications.   Most of clause 8 will move to an informative annex.

The possiblity of allowing a command to be delivered on a port other than the one the data transfer is directed over was discussed.  A proposal will be developed for consideration.

# Transport Layer Topics

## Review of SSA-TL1 working draft [Scheible]

John Scheible reviewed the changes incorported into revisions 6a and 6b.  Revision 6a was editorial only.  Revision 6b was the result of the Austin Editor’s Session.  The Austin changes included:

Undefined character codes were defined;

A definition of an inactive state that actually existed but had not been defined was added;

Wrap Mode was changed to the Wrap Flag;

A reflect flag was defined to clarify port operation.

Greg Kapraun noted that proposal 95a165r1 was not incorporated into revision 6b, but should be per agreement at the last meeting.

Considerable discussion was held on Wrap mode versus flag nomenclature.  Ed Carmona requested that the process for exiting Wrap be defined.  Paul Petersen objected to the use of ‘flag’ since there are also flag characters.

Consensus was to move Wrap back to being a mode. Ed Carmona presented a proposal (95a186) to allow a master to ‘unwrap’ a port wich was accepted.  If in Wrap Mode and a Configure Port SMS is received indicating Nomal Mode it will be rejected with a response of INVALID FIELD. A Configure Port SMS indicating Privileged Mode for a port in Wrap Mode will be accepted and the affected port will enter the Begining Communication process, thus moving to Priviledged Mode.

Phil Bryan requested that a change be made to table 21 MEANING CODES. He asked that the Type Information field of Meaning Code 06h 02h 00h be changed to XX to denote vendor specific.  The consensus was to have this field reference SPC for its definition which will allow some vendor specific codes.  The Meaning Code name for 06h 02h will also change to match that in SPC and avoid the trademark on PFA, (Predictive Failure Analysis).

John Scheible presented a propsal (95a185) to require zero fill for pad on any SMS.  Accepted without objection.

John presented the revised SAT algorithm description (95a168r4).  Accepted with reservations by Symbios pending further review by their engineers and by IBM-UK regarding use of the the Reflect bit.

John presented his comments on SSA-TL1 (95a171r2).  Accepted with editorial modifications.

Discussion was held regarding a present requirement that all Targets verify that commands with an Initiator ID come in only from the port that the Initiator ID was registered on. Although there were some who wanted to eliminate this restriction, no proposal was made and no action taken.

A proposal from Larry Lamers on a QUERY SWITCH SMS was considered.  This would allow the architected maximum number of ports to be reported in the mask.  It would also result in truncation of the QUERY NODE REPLY SMS to 20 bytes.  This proposal will be revised taking into account the comments received.

A proposal to codify the "operation complete" function was discussed.  This proposal does not change existing implementations but does allow software drivers a simplification if future protocols follow this method of indicating normal completion of an I/O process.  This item was defered for considered in TL2.

A proposal to remove the eight-byte boundaries because of impacts on device drivers and the fact that there is no architectural reason for it.  The proposal was voted down  1:9.

Greg Kapraun proposed, (95a174r0) that a non-control frame with less than 7 data bytes be considered a Protocol Error. Mark DeWilde felt that there were undoubtedly many frame combinations that could be generated that were not according to the specification and it would tax the committee to identify and list them all. It was agreed to add a note to the definition for Protocol Error that advised of the situation described by Greg.

A question was raised regarding TL1 section 10.6 as to what the value of the ‘anti-hang timer’ should be.  Because this was not a TL  function as defined , it belongs in the Upper Layer Protocol.  Subclause 10.6 in TL1 will be moved to S2P. In addition, section 10.7 will be renamed from Communication Blocked to Healthy Web Function.

Other questions emerged regarding timer values not specified in the TL1 draft. In TL1 section 10.7, item 4 requires a time-out for Query Node Reply, but gives no guidance for its value. However, section 9.3.2 requires every Initiator to poll the current Master Node with a Query Node SMS every 100 ms. The paragraph in section 9.3.2 with this time-out value will be relocated to section 10.7, however, this related time-out value was calculated to be too low for a worst case condition. .  The recommended timer value is 10 seconds for an initator to detect a QUERY NODE from the master and 5 seconds for a master to detect a response from the intiator, (this implies but does not mandate a maximum Node delay of 5 ms for processing a frame in a Node for a maximum number of  hops which is 505). In addition, it was decided to add a 'Master is Alive' bit to the Query Node SMS that will reset the timer in any Initiator when used in the Healthy Web process.

Dave Deming rasied several concerns regarding how the transport layer will deal with a device being hot plugged when an LRC is being used.  Re-establishing the link without dealing with frames ‘in the pipeline’ to the device that was removed may cause problems.  The existing configuration processes need to be examined to understand if there is a real problem.  Charles Monia will lead a reflector discussion on hot-plugging.

# Physical Topics

## Review on PH Ad Hoc Meeting [Ham]

Bill Ham reported on the separate PH ad hoc held earlier in the day. It was agreed to remove Fig 21 in rev 5a because it is not relevant to the connector specification. Table 11 will have the line for Input Switching Threshold eliminated. It was redundant and confusing.

AMP presented an alternative internal connector along with high frequency performance data.  This is an active proposal to provide a sheilded internal solution.  They have agreed to provide information on how this can be intergrated into the existing internal cable assembly.

Molex has reaffirmed their commitment to produce the two-bay connector for internal adapter use.

## LRC [Bouvier]

Dan Bouvier discussed the need for an LRC circuit.  Empirical test data should be available by next meeting.  Implementation guidelines are definitely needed and will be considered within the SSA IA-TECH group.

## Review of PH1 Revision 5a [Ham]

The PH1 description now makes it clear that measurement of electrical parameters is to be done at the connectors. Connectors are to be as close as possible to the transceiver chip. The node boundry ends at the device connector and anything outside that boundry falls under the cable impeadance budget. This leaves undefined any impeadance requirements for the type of circuit known as LRC when it is part of the path of a link.

Bill Gintz stated that the termination circuit affects the cable assembly (i.e., reflections) and more attention should be paid to reactive components.  A more precise definition of the termination impeadance is needed, with a tolerance on all reactive components.

Although PH1 states that equalization, (if used) must be external to the device, there is no guidance on implementation. It is possible to have an equalization scheme that negatively impacts signal quality.  This is probably not critical for transfer rates of 20 MB but will be important for 40 MB operation.

Node capacitance and inductance from trace lengths can be critical issues as transfer rates increase.  These parameters can affect interoperability between devices. A proposed requirement for a maximum of 10 pF node capacitance was discussed.  Bill Gintz stated that the requirement should be for 5 pF or less, measured at the chip pin. Bill Ham felt that there was some risk that maximum allowable cable lengths may be reduced to insure interoperability at 40 MB transfer rates. Mark DeWild felt that ESD protection would add capacitance to a node and thus must also be considered. Due to the lack of data, no recommendation was adopted. The PH1 ad hoc group meeting in September will attempt to provide more data on this subject.

When dealing with higher transmission frequencies, besides termination at the receiver, absorbing reflected energy at the driver also needs consideration.  A combination of these two specifications for the 40 MB spec may provide the best solution for interoperability between 20 and 40 MB devices. This would leave the 20 MB spec as is.

A PH1 ad hoc meeting on termination and reflection issues was proposed for 9/25/95 in the Denver area starting at 9:00 a.m.

Torque specs for the external connector are 5-inch pounds for the jack screw.  A value for the detent is still needed.  The thread is M1.6; material is hardened stainless steel.

The present external connector does not fit 4 up on the back of a PC, so dual port on a single PC board is not feasible.  Also on the 9-pin external connector has only 6 pins used.  AMP made a proposal for an alternative 6-pin external connector with a ‘squeeze to release’ detent that could do 4 up on a PC board.  DDK also made a proposal of one of their off-the-shelf connectors that is physically viable but has not been tested in high-speed serial applications.

Alternative connector proposals will be considered for the PH2 draft.

## Clarification of Faults [Kapraun] (95a155r0)

Greg Kapraun asked that the group revisit his proposal from the last meeting regarding detection of line faults. He felt a requirement for transmitters to detect breaks in the web was needed.  Presently there is no requirement on either the driver or receiver to detect line faults, it is an option for implementors to decide. The proposal would require line fault detection by the driver.  Bill Ham argued that it would require detection of a third state, (other than negated or asserted) and would reduce noise margins. The proposal was not accepted by a 3:4 vote.

## 40 MB/sec SSA-PH

A proposal to deal with 40 MB/sec transfers in the physical layer was put forward by Larry Lamers (see 95-178r0).  Ken Hallam stated that this material should be considered for SSA-PH2.

The working group recommended by a vote of 12:1 to include the QUERY PORT REPLY SMS modifications from 95a178r0 into TL1 so that speed parameters may be communicated. Not included was the section for a speed negotiation process.

Bill Ham proposed and the group agreed unanimously that a goal of any 40 MB physical draft will be to use the same interconnect as the 20 MB draft, exclusive of any equalizers or LRC components.

Greg Kapraun noted that without some modifications to SSA-PH1, a PH2 device would not meet the PH1 standard.  Several members felt that the slew rate and spectral content need to be adjusted in PH1 to allow for backward compatibility with 40 MB devices. Other members felt that with proper specification changes to the receiver and transmitter parameters for the 40 MB draft, there would be no downward compatibility issues with 20 MB devices.

All agreed that more data is needed on 40 MB/sec transceivers to justify parameters changes.

It was also agreed that the receiver eye diagram will have to change for 40 MB if the 20 MB interconnect is to be used.

An LRC or equalizer interfers with the electrical path, (link) between 2 ports.  The reflection properties will be dramatically different from a link without either of these.  The present physical layer specifications are defined at the connector but the LRC does not have an SSA connector nor does an equalizer.  Equalizers and possibly LRC components will behave differently as the transmission frequency changes.

# Call for Patents

Ken Hallam requested that anyone aware of any patents required for the proposals be disclosed early in accordance with the ANSI patent policy.  Refer to the minutes of prior meetings for items already identified.

# Action Items

10)

# Meeting Schedule

The next working group meeting of X3T10.1 will be Monday, October 30, 1995, in Hampshire, England at the Botley Park Hotel and Country Club, Winchester Road, Boorley Green, Hampshire  SO32 2UA, TEL:  1489-780888, 	FAX:  1489-789242, hosted by Xyratex.  The meeting will begin at 9:00 AM.

The X3T10.1 planned future meetings are:

Week of October 30, 1995 in Hampshire, UK hosted by Xyratex

Week of December 11, 1995 in San Jose, CA hosted by VLSI

Week of February 26, 1996 in San Jose, CA hosted by Samsung

Week of April 29, 1996 in Burlington, VT hosted by IBM

Week of June 24, 1996 in St. Petersburg Beach, FL hosted by AMP

Week of August 26, 1996 in Ft. Collins, CO hosted by Symbios Logic, Inc.

Please note that changes to this schedule may occur.  Changes to meeting dates, location, and agendas are posted to the SSA reflector.

# Adjournment

The meeting adjourned at 7:00 p.m. on Wednesday.