Accredited Standards Committee

X3, Information Processing Systems

Doc:X3T10.1/95a176r0

Date: August 8, 1995

Project:  N/A

Ref Doc:  X3T10.1/989D Rev 6

Reply to: Ed Carmona

To:   X3T10.1 Membership

From: Ed Carmona

Subject: SSA Port Speed Reporting

BACKGROUND

In preparation for supporting multiple speeds on SSA, the following change should be implemented in order to communicate each port's operating speed on a node.

PROPOSAL

The following proposal identifies how a node can use the QUERY PORT REPLY to report a port's operating speed. Change bars denote the changes required.

11.2.8 QUERY PORT REPLY SMS

The QUERY PORT REPLY SMS defined in Table 32 is sent from a target to an initiator in response to a QUERY PORT SMS.

Table 32 - QUERY PORT REPLY SMS

Byte Bit 7   6     5     4     3     2     1   Bit 0

0                   SMS CODE (0BH)

1                      RESERVED

2                        TAG

3                        TAG

4                 LINK ERP ERROR COUNT

5                LINK ERP ERROR COUNT

6                  ALARM THRESHOLD

7                  ALARM THRESHOLD

| 8                 CURRENT SPEED MASK

| 9                 CURRENT SPEED MASK

| 10               SUPPORTED SPEEDS MASK

| 11               SUPPORTED SPEEDS MASK

The TAG field is copied from the QUERY PORT SMS.  It identifies the QUERY PORT SMS for which this response is being generated.

The LINK ERP ERROR COUNT field is the count of the number of times the link ERP has been invoked for the selected port since the last power on, total/absolute reset or the last QUERY PORT SMS with the CLE bit set.  A value of FFFFh is returned if the count is greater than or equal to 65535.

The ALARM THRESHOLD field is the current setting of the alarm threshold for the selected port.  See 11.2.3.

The CURRENT SPEED MASK contains a bit vector of  the current operating speed of the port. Table 33 defines the format of the current speed mask. A node shall set only a single bit in the current speed mask corresponding to the current operating speed of the port.

|                Table 33 - Current Speed mask

|		Bit         	 Port Operating Speed

|		Byte 8,Bit 0     20 MB/s

|		Byte 8,Bit 1     40 MB/s

|		Byte 8,Bits 2-7  To be defined in future standard

|		Byte 9,Bits 0-7  To be defined in future standard

The SUPPORTED SPEEDS MASK contains a bit vector of each speed supported by the port. Table 34 defines the format of the supported speeds mask. A node shall set a bit for every speed supported by the port.

|               Table 34 - Supported Speeds mask

|               Bit              Port Operating Speed

|               Byte 10,Bit 0    20 MB/s

|               Byte 10,Bit 1    40 MB/s

|               Byte 11,Bits 2-7 To be defined in future standard

|               Byte 11,Bits 0-7 To be defined in future standard

Ed Carmona

Conner Peripherals, Inc.

Voice: (408) 456-4082

Email: ed.carmona@conner.com