Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a173r3
Date:	August 30, 1995
Project:	X3T10.1 / 1121D
Ref Doc.:	SSA-S2P rev 3
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Comments on SSA-S2P rev 3 by John Scheible

BACKGROUND

As part of my editing job on SSA-S2P, I routinely review the document for changes.  As part of this review, I have come up with the following comments.  They are grouped into three categories: Major, Minor and Editorial.  The Major items need committee discussion.  The Minor items are fixes to what I believe is the intent of the committee, but involve some technical change and therefore need to be addressed.  The Editorial issues are simply editorial and will be folded into the next revision unless someone objects.

PROPOSAL

Make the following major technical changes to SSA-S2P

- Add a new clause after clause 6.  The subclauses will include 7.5.1 and 7.5.2 thereby eliminating the need for 7.5.  The new clause is shown at the end of this proposal.

- UNRESOLVED: Due to lack of time, I was unable to verify if the SCA write-up matched ACA definition exactly (now that it has a different name, it must specify ALL cases).

Make the following minor technical changes to SSA-S2P

- Almost global - Change SSA-TL1 references to SSA-TL, and define SSA-TL as SSA-TL1 or SSA-TL2.  We do not want to change the spec when we define SSA-TL2 which SSA-S2P should run on.

- 6 - Change heading from SMSs to “SSA-S2P message structure”.

- 6, after third paragraph - Add a two new table entitled SSA-S2P message structure (one for initiator generated SMSs and one for target generated SMSs).  The tables will have byte 0 as the sms code = 82h, byte 1 as the s2P code,  and up to byte 31 as “”.  This matches SSA-TL1.  We need to differentiate between SSA-TL1 and SSA-S2P SMSs since the sms code field is in a different byte.  Add a table reference at the end of the third paragraph.

- 6, last sentence of  fourth paragraph - Change from:
“Since the Channel component may be only a single byte it is left-aligned.  Bit 7 is set to 0b in the last valid byte and to 1b in the preceding byte, if any”
to
“If the Channel component is a single byte, the ichannel field contains the Channel component left aligned, with the second byte padded with any value by the source node and ignored by the destination node.”

- 6, last sentence of the eighth paragraph - change (for consistence with SSA-TL1)
“Padding a SMS with additional bytes of data is not an error as long as the data field does not exceed 32 bytes.”
to
“An SMS has a maximum length of 32 bytes.  Only the bytes defined in the appropriate SMS are required, however the source node may pad the SMS with any value to a total length of 32 bytes.  The destination node shall ignore the pad bytes.

- 6.2.1, bullet b) - Change
“...response to the discarded SMS.”
 to 
“...SCSI STATUS SMS or SCSI RESPONSE SMS to the discarded SCSI COMMAND SMS.”

- 6.4. third paragraph, second sentence - Change:
“The value used in the tag field becomes available for re-assignment when the initiator receives a SCSI STATUS SMS for the command.”
to
“The value used in the tag field becomes available for re-assignment when
 the I/O process is terminated.

- 6.4. third paragraph, third and fourth sentences - Change:
“If the initiator is directly mapping the functions of parallel SCSI-2 then the Tag field of the SCSI-2 QUEUE TAG message can be included as one byte of this field.  The other byte can contain the SCSI-2 target address, LUNTAR and LUNTRN.”
to a new paragraph following the existing paragraph without sentences 3 and 4:
“Implementer’s note: Parallel SCSI-2 requires all tag field values to be unique for a given I_T_L nexus, but SSA requires that the tag field values be unique for an I nexus.  SSA uses the tag field value to identify which Target/LUN and which SCSI COMMAND SMS the SCSI STATUS SMS is associated with.  Therefore, an easy conversion from a parallel SCSI-2 device driver to SSA-S2P is to include the parallel SCSI concepts of tag field (1 byte), target address (4 bits), and LUNTAR/LUN (1+3 bits) into the 2 byte SSA-S2P tag field.”

- Add a new section after 6.3 entitled “6.4  Tag values” with the following paragraph:
“The tag field values of all outstanding SSA-S2P SMSs from a given initiator must be unique.  The initiator uses the tag field value of the SCSI STATUS SMS or SCSI RESPONSE SMS to determine both the source (target, LUN, target routine) and which SMS is associated with the SCSI STATUS SMS or SCSI RESPONSE SMS.  tag field values outstanding at a target need not be unique between initiators, since the target identifies processes with both a tag and initiator id field.

- 6.3, second to last paragraph, last sentence - change:
“...or when the ddrm field is set to 1b..”
to
“...or when the ddrm field is set to 0b.”

- 7.1.2, list in the sixth paragraph - Change bullets c) and d) to:
”c)   Hard reset condition (including SSA-TL1 concepts of Local, Total, and Absolute resets).”

- 7.2.3, first line of first paragraph - Change “...fixed frame overhead...” to “...low frame overhead...”.

- 7.4.3, second paragraph (after h)) - Change:
“Until this issue is resolved, the following SCSI commands are not supported.”
to
“The following SCSI command CDBs are not supported.”.

- 7.4.3, after the second list a)b)c)d) - Add a heading since the following three paragraphs are not related to 7.4.3:
“7.4.4   Unit Attentions”

- 7.4.3, fourth paragraph - Change:
“When a target generates a Unit Attention it sets a field in each unique initiator in the initiator table.  If the Unit Attention was generated by a SMS from an initiator then the target does not set the field(s) for that particular initiator.”
to
“When a target generates a Unit Attention it sets an SSA-S2P specific unit attention flag in all entries of the Initiator table (only one flag for all paths to a given initiator).  If the Unit Attention was generated by an SMS, then the target does not set the unit attention flag for the associated initiator.”.

- 7.4.3, fifth paragraph - Change:
“When a target receives a SCSI COMMAND SMS it locates the entry for that initiator by accessing the table with the Initiator ID.  If the field is set then the target presents Check Condition status and generates sense.  It then clears all of the fields for the corresponding Unique ID.  Thus the Unit Attention is eventually presented only once to each initiator.”
to
“When a target receives a SCSI COMMAND SMS it locates the unit attention flag for the initiator by accessing the Initiator table entry containing the initiator id field value.  If the unit attention flag is set, then the target clears the flag and terminates the SCSI COMMAND SMS by generating a SCSI STATUS SMS with a status field value of CHECK CONDITION STATUS, and generates sense data.  Thus the Unit Attention is eventually presented only once to each initiator.”

- 7.6.1, entire paragraph - Change:
“SCSI-2 third party commands will not work in the SSA SCSI-2 Protocol, and SCSI-3 has not yet approved a new third party command definition.  To avoid defining a conflicting method, the SSA SCSI-2 Protocol does not support third party commands at this time.  See 7.4.3 for more information.”
to
“SSA-S2P does not support third party commands (see 7.4.3).”.

- 8.3.2, Editor’s note after the second list - Delete it.
“Editor’s note: Is the Queue Full indication by Initiator or by the entire command queue.  Some devices allocate a few queue slots to each Initiator and the remainder of the queue slots are in a general pool.  The write-up assumes a Queue Full indication for the entire command queue.”

- Add the following changes to the new sections (already included in the attached section):
Add to 7.1 paragraph 2
“However, to avoid indefinite circulation, one node in each Cyclic Path should be programmed to block propagation of User Defined characters.”

Make the following editorial changes to SSA-S2P

- Global - Change “User-defined” to “User Defined” for consistency with SSA-TL1. (no -, capital D)

- Global - Change “initiator table” to “Initiator table” (capitalize initiator).

- Global - Change “SSA SCSI-2 Protocol” to “SSA-S2P”.

- Global - Change the heading 1’s to NOT begin on a new page for consistency with SSA-TL1.  ANSI does start on a new page, ISO does not, SSA-TL1 does not.

- Global - Change all table headers to Bit 7 and Bit 0 to match SSA-TL1.

- Global - Change “Path address” to “Path component” if applicable.

- Global - Change “Channel address” to “Channel component” if applicable.

- Global - Change “Channels” to “channel” if appropriate (general reference).

- Global - Change “Tag” to “tag value” if appropriate.

- Global - Change “message” to “SMS” if appropriate.

- Global - Change ASYNC ALERT “error code” to “meaning code”.

- Global - Change “SMS Buffer full condition” to “SMS Buffer Full condition”.

- Global - Change all SMS tables (preceding paragraph) from 
“...SMS are shown in Table xx.” 
to 
“...SMS are defined in Table xx.” for consistency with SSA-TL1.

- Global - Make sure all field names are small caps without large capitals (i.e. 6.4, three lines from end, link field not Link field).

- Back of cover page - Add a underscore to Ken Hallam’s internet address.

- Back of cover page - change FTP site to /pubs/io/standard/ssa.

- Back of cover page - remove abstract (it is on the cover page).

- Introduction - Add the following paragraph in the beginning (for consistency)
“This standard is divided into the following clauses and annexes.”

- 2, SCSI-2 reference - Add ANSI X3.131-1994

- 3.1 - Make spacing between word-definition consistent (3 spaces).

- 3.1 - Remove 3.1.11, 3.1.20, 3.1.26 if not used in spec (need to check).

- 3.2 - Remove CMOS, FSN, LSI if not used (need to check).

- 3.2 - Change SAM definition to SCSI-3 architectural model.

- 3.2 - Add “LUN		Logical Unit”

- 4.2, last paragraph - Change “QUERY NODE” reference to “SCSI STATUS” to use SSA-S2P SMS.

- 4.2 - Add to the end:
“Variable names are in italics.”

- 4.2 - Add to the end:
“The byte ordering convention is Big Endian (i.e., the most significant byte of a number is sent first).”

- Table 2 - Change the header from “Send” to “Sent from” and “Receive” to “Received by”.

- 6.2.1, bullet d) - change 
“...and the SMS has the resume...”
to
“...and the SCSI COMMAND SMS has the resume...”

- 6.2.3, bullet 3) - Invalid field should be all caps.

- 6.4, first paragraph, third sentence - Change “Refer to section for invalid...” to “Refer to 6.2 for invalid...”

- 6.4 through 6.11 - change:
“The contents of the data field in a ... are shown in Table ...”
to
“The contents of the data field in a ... are defined in Table ...”

- 6.5, third paragraph after Table 5 - Change:
“The SSA-SCSI-2 Protocol uses one SMS code with imbedded flag and link fields, rather than...”
to
“SSA-S2P uses a single sms code value with imbedded flag and link bits, rather than...”

- 6.4, seventh paragraph, second sentence - Remove:
“Note that the SSA SCSI-2 Protocol provides 7 bits for luntrn, although SCSI-2 has only 3 bits.”
This is mentioned in 7.4.2.  Also delete the corresponding sentences in 6.7, 6.8, and 6.10.

- 7.1.2, end of second paragraph - change:
“...QErr bit in MODE SELECT field page 0Ah.”
to
“...Qerr bit in mode page 0Ah of the MODE SELECT command.”

- 7.1.2, third paragraph, first sentence - Change:
“...rejected with Serial Contingent Allegiance status (30h), unless...”
to
“...rejected with SCSI STATUS SMS status field value of SERIAL CONTINGENT ALLEGIANCE ACTIVE (see Annex A), unless...”.

- 7.1.2, end of the fourth paragraph - Change
“... with SERIAL CONTINGENT ALLEGIANCE ACTIVE status.”
to
“with a SCSI STATUS SMS status field value of SERIAL CONTINGENT ALLEGIANCE ACTIVE.”

- 7.1.2, fifth paragraph - Change:
“... then it returns CHECK CONDITION status.”
to
“... then the target returns a SCSI STATUS SMS with a status field value of CHECK CONDITION STATUS.”

- 7.1.3, second paragraph - Change “Error! Reference source not found.” to “6.2”.

- 7.2.1, end of first paragraph - Change “(400 ns to 600 ns).” to “(400 ns to 600 ns when running on SSA-PH1).”

- 7.2.3, end of the only paragraph - Change “...arrives, reducing latency.” to “...arrives, thereby reducing latency.”.

- 7.3.1, fourth paragraph, end of third sentence - Remove double period.

- 7.3.1, after first sentence of last paragraph - Change single space sentence delimiter to double space.

- 7.3.2.1, first a)b) list - Clean up the formatting (spacing).

- 7.3.2.1, last sentence of next to the last paragraph - Change 
“...should ”throw away” any...” 
to 
“...should ignore any...”.

- 7.5.1, second paragraph, sentence in () - Remove (), i.e. make it a real sentence.

- 7.5.1, last paragraph: Change:
“The origination and receipt of SYNC characters is controlled by the rpl field in page 4h of the SCSI Mode Select fields.”
to
“The origination and receipt of SYNC characters is controlled by the rpl field in mode page 4h of the Mode Select command.”.

- 8, end of paragraph - Change reference “(See 8.1.1)” to “(See 8.1.2 and 8.1.3)”.

- 8.3.2, Editor’s note after the second list - Delete it.

- Table 20 under 9.1.1 - Order the table.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com

# Transport related items

## SYNC character

SSA-S2P defines the 8B/10B character K28.0 as a SYNC character for rotating media as spindle sync, which can be interleaved within frames on the link subject to the rules for User Defined characters in SSA-TL.

One node in the web should be nominated to originate the SYNC characters at the nominal rotation rate of the disk drives.  For example, this node could be an array controller or a disk drive.  The other disk drives decode the SYNC characters and use the decode to synchronize their spindle servos.  The decode replaces the Master sync pulse that was provided on a separate cable for the parallel SCSI bus.  In the event that the originating node fails a backup node can be nominated to replace it.

The propagation of User Defined characters is controlled by the EUDC bit of the Configure port SMS (See the SSA-TL specification for details.).  Dual-port and switch nodes should normally be programmed to propagate User Defined characters received on one port to the other port(s).  However, to avoid indefinite circulation, one node in each Cyclic Path should be programmed to block propagation of User Defined characters.

The origination and receipt of SYNC characters is controlled by the rpl field in mode page 04h of the SCSI Mode Select command.

## Option Block usage of the SYNC pin

SSA-PH defines an options bay that includes a Sync pin.  This Sync pin provides the same function as the SYNC character over the SSA links, but allows synchronization across SSA webs.

Implementer’s note: The device should provide a way to block generation and receipt of any signal on the SYNC pin for those customers who wish to use the SYNC character on the SSA links but still use a backplane or cable structure that connects the Sync pin.

## Unit attention flag in the Initiator table

SSA-S2P adds an additional flag, unit attention flag, to each Initiator table entry (See Error: Reference source not found).

## Tag interaction between SSA-TL and SSA-S2P

Since there is no overlap between the responses to SSA-S2P SMS, and SSA-TL SMSs (including ASYNC ALERT SMSs), there is no requirement that the tag field values be unique across outstanding SSA-S2P and SSA-TL SMSs.

## Effects of reset conditions

SSA-TL defines five types of reset:  Link Reset, Local Reset, Total Reset, Absolute Reset, and Power On Reset.

Link Reset has no affect on the SCSI constructs.

Local Reset, Total Reset, Absolute Reset, and Power On Reset will reset SCSI constructs (equivalent to a SCSI-2 Hard reset).  In addition to the normal clearing of all I/O processes, reservations, and operating conditions to their last saved positions; the Initiator table (target) and Configuration table (initiator) will be cleared.

Neither SSA-TL nor SSA-S2P provide a global reset equivalent to the RST signal in the parallel SCSI bus.