Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a172r1
Date:	August 28, 1995
Project:	X3T10.1 / 1121D
Ref Doc.:	SSA-S2P rev 3 
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Mode page changes proposal for SSA-S2P

BACKGROUND

Several mode page parameters either do not apply to SSA-S2P, or have different meanings.  This proposal addresses these differences.

REVISIONS

The August SSA Working group changed the proposal to not ignore the read buffer full ratio and write buffer full ratio fields.

OPTIONS

The following options were chosen to minimize device driver changes when converting from parallel SCSI-2 to SSA-S2P.

- In page 02h (Disconnect/Reconnect parameters) the maximum burst size field could be used to limit the byte count field in any associated DATA READY or DATA REQUEST SMS.  However, I chose to make most parameters ignored due to the ease in changing the byte count field in the DATA REPLY SMS (read) and by pausing the data transfer for a write (by the initiator).  If there are strong performance advantages to hardware controlled data transfer, then we could reconsider it.

- I chose to have the ignored parameters still be saved and echoed back with the Mode Sense command.

- I chose not to specify the initial or changeable parameters.

PROPOSAL

Add a new section 7.7 under clause 7 (Some differences between SSA SCSI-2 Protocol and parallel SCSI-2) as shown on the following page.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	scheible@vnet.ibm.com

- 7.7   Mode page parameter differences

Several of the parallel SCSI-2 mode page parameters need to be redefined for SSA-S2P.

- 7.7.1   Page 02h - Disconnect/Reconnect parameters

Several parameters within the SCSI-2 Disconnect/Reconnect parameters page are not required by SSA due to the frame multiplexing nature of the link.  The the following parameters are ignored.

- bus inactivity limit

- disconnect time limit

- connect time limit

- maximum burst size

- dtdc

The target shall return the Page 02h parameters in the Mode Sense command that were issued with the Mode Select command, according to the rules regarding current and saved values.  SSA-S2P does not specify what values the parameters shall be initialized to, nor does SSA-S2P specify which parameters are changeable.

- 7.7.2   Page 0Ah - Control Mode parameters

Since the optional concept of Extended Contingent Allegiance (ECA) is not supported by SSA-S2P, the following parameters are fixed in Page 0Ah (Control Mode parameters).  These parameters are initialized to the specified values and are not changeable.

- eeca bit = 0b

- raenp bit = 0b

- uaaenp bit = 0b

- eaenp bit = 0b

- ready aen holdoff period field = 0000h