Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a168R5
Date:	September 24, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 6 
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: SAT algorithm clarification proposal

BACKGROUND

The SAT algorithm section (8.5) was sometimes misunderstood for the following reasons.

- The purpose of a quota and b quota were not clearly defined.

- The format of the variables was confusing (a quota is different than A quota) are prone to error, and do not fit with the ANSI style.

- The language was not definitive and the terms not defined clearly resulting in ambiguity.

- The use of node was incorrect (most rules apply to port, not node).

- The type of the term was not specified (what is a field, a variable, a counter).

- The use of Token was confusing, it is really a character (do not use two names for one thing).

- The reflect bit was added at a later date and was not incorporated into the text fully.

- Formulas such as A or B and C were used which should be interpreted as A or (B and C) which can lead to confusion.

PLEASE REVIEW

I completely rewrote section 8.5 and its subsections as shown in the following pages to clarify the SAT algorithm.  Since the text was ambiguous in some places, I clarified it as shown below.  Please carefully review the following assumptions for accuracy.

- Does a port having no frames to originate before the SAT character arrives cause it to be Satisfied?  In other words, is a port Satisfied if a port has nothing to originate, then later has something to originate and starts sending frames before the SAT character arrives?  I assumed this is not the case.  The original text is shown below:
“A port is Satisfied if it has originated at least a pre-defined frame quota, a quota, since it previously forwarded a SAT token, or it has no frames to originate.”

- If a port is generating 3 concurrent data transfers, it can send 3*a quota frames, even if most frames are associated with a single data transfer, while the other data transfers get no (or less) frames.

- The generation of timed out SAT characters is described as causing it to be forwarded.  No mention is made about allowing an port that is not Satisfied to originate frames.  I assume it  forwards the SAT character and resets the frame counter, but does not “hold” the SAT character and allow up to A Quota of frames to be originated while holding cut through frames.  The original text is as follows:
“All ports incorporate a timer to detect loss of the SAT token due to a link error.  If a port does not receive the next SAT token within 100 ms of forwarding the previous token then it generates a new token and renews its quota.  This may lead to multiple SAT tokens circulating in the same direction if several nodes generate new tokens simultaneously.  However the fairness algorithm will still operate correctly and the tokens will eventually merge into a token as described below.”

- I assume frame quota’s are only applicable to Application frames.  The following text implies that any frame counters toward the frame quota, although it is later it stated that privileged and control frames are not controlled by the SAT algorithm.
“A port is Satisfied if it has originated at least a predefined frame quota, q quota...”

- The SAT’ at the end of the following original text is confusing.  The term forward includes the definition of reflecting a SAT’ as a SAT.  The original text is as follows.
“For a string or loop with only a few nodes the SAT token can rotate very rapidly if all nodes are Satisfied.  This could impact frames being transmitted in the other direction.  Therefore a node must not forward SAT until at least 100 character periods have elapsed since it received the previous SAT or SAT’.

- The reflect bit in the CONFIGURE PORT SMS states “When the reflection flag is set to 0b in both ports the node shall forward...”.  This implies that the node must remember the reflect on a per port basis, even though it is a node function.  It would be better to simply change the reflect/forward status when a CONFIGURE PORT for either port occurred (i.e. do not need an act on one then the other port).

PROPOSAL

Make the following changes

- Replace section 8.5 and its subsections with the text on the following pages.

- Change the fourth sentence of paragraph 6 under section 7 (Application frames) from:
“They can only be send when the transmitting port is in the ready state and Normal mode.” to
“Application frames can only be originated, subject to the SAT algorithm (see 8.5), or forwarded when the transmitting port is in the Ready state and Normal mode.”

- Add the following definition after 3.1.3.
Cut-through frames: Frames which are not addressed to the receiving node and are passed from the input lines of one port to the output lines of another port with minimal latency according to the routing rules defined in 8.2.

- Add the following definition after 3.1.14
Originating frames: Frames that are generated from the node itself and are sent out of the output lines on one of the ports of the node.  Any retransmitted Application frames that were originated by the port increment the frame counter for SAT algorithm purposes.

- Move 3.1.13 (SMS) to after 3.1.18.

- Add the following sentence to the end of the third paragraph after table 1 (3.3), “Ignored bits...”:
“Variable names are shown in italics, such as Idle quota.”

- Change the reflect bit definition in 11.2.3 (first paragraph after a)b) list to:
“The reflect bit is one of the controls affecting the reflection flag node function (See n.n.n).  If the node is a dual port node and the reflect bit is set, the reflection flag is set for the node.  If the node is a dual port node with the operational flag  set for both ports and the reflect bit is cleared, the reflection flag is cleared for the node.”  The reflect bit is ignored for single ported and switch nodes.

- Add a new section entitled “reflection flag “under Node Functions (to be included in the next rev of SSA-TL1) with the following text:
“The reflection flag  of a node, causes all ports on that node to either forward or reflect SAT and SAT’ characters as defined in 8.5.2.”

The reflection flag is set in any of the following conditions:
   a)  The node is a single port node or switch node.
   b)  One of the ports of a dual ported node becomes not Operational.
   c)  A dual ported node processes a CONFIGURE PORT SMS with the reflect bit set.

The reflection flag is cleared when a dual ported node with the operational flag set for both ports processes a CONFIGURE PORT SMS with the reflect bit cleared.”

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com

- 8.5   SAT

The origination of Application frames for strings and loops is controlled by the SAT algorithm where SAT refers to Satisfied.  The SAT algorithm helps balance between any node from being starved of bandwidth and permits spatial reuse of the individual links.

- 8.5.1   Terminology

Some SAT terms are defined in the following list.

- a–quota field in the CONFIGURE PORT SMS is used to determine how many frames a port can originate while holding cut through frames.

- b quota field in the CONFIGURE PORT SMS is used to determine the number of frames a port may originate if no cut through traffic exists (the link is idle with two consecutive FLAG characters).

- reflect bit in the CONFIGURE PORT SMS is one of the factors to instruct the port to reflect SAT and SAT’ characters, which effectively breaks the loop or string into two SAT regions (see 8.5.6).

- reflection flag is a node function that determines whether the all ports of a node reflect or forward the SAT and SAT’ characters (See 8.5.6).

- Hold quota is a variable containing the a quota field value times the number of current outstanding data transfers from the port.  This dynamic scaling of the a quota ensures fairness when a port is originating several outbound data transfers concurrently.  The number of frames sent does not have to be balanced between each of the concurrent data transfers.

- Idle quota is a variable containing the b quota field value times the number of current outstanding data transfers from the port.  This dynamic scaling of the b quota ensures fairness when a port is originating several outbound data transfers concurrently.  The number of frames sent does not have to be balanced between each of the concurrent data transfers.

- frame counter is a counter which counts the number of Application frames originated since the last time the SAT character was forwarded or reflected by the port.  Forwarding or reflecting a SAT character sets the frame counter to zero.

- Satisfied refers to the condition of a port whereby either of the two following conditions is met.

  - The port has originated at least Hold quota number of frames since it previously forwarded or reflected a SAT character.

  - The port has no frames to originate when holding the SAT character.

  - 8.5.2   Reflection versus forwarding

If the reflection flag is cleared, all port(s) of a node shall forward SAT and SAT’ characters.  If the reflection flag is set, all port(s) of a node shall reflect SAT and SAT’ characters.

A port shall forward a SAT character after receiving a SAT character by holding it according to the rules of the SAT algorithm and then generating a SAT character on the outbound lines of the port which did not receive the SAT character.  A dual port node shall unconditionally forward a SAT’ character after receiving a SAT’ character by generating a SAT’ character on the outbound lines of the port which did not receive the SAT’ character.

A port shall reflect a SAT character after receiving a SAT character by holding it according to the rules of the SAT algorithm and then generating a SAT’ character on the outbound lines of the same port that received the SAT character.  A SAT’ character is reflected unconditionally by generating a SAT character on the outbound lines of the same port that received the SAT’ character.

- 8.5.3   The SAT algorithm

The node at the end of a string can be a single-port node, a dual-port node with one port not operational or a switch (a dual port node with both ports operational is not at the end of a string by definition).  Thus all ports shall implement the SAT algorithm.

The SAT algorithm will be described initially for a loop of dual-port nodes, with enhancements for a string shown in 8.5.4.  Application frames entering a loop are controlled by a SAT character which circulates in the opposite direction to the frames that it controls, as shown in Figure 1.  Circulating the SAT character in the opposite direction maximizes spatial reuse.

The SAT algorithm is applied independently to clockwise and counter-clockwise traffic.  Therefore there are two SAT characters circulating on the loop, one in each direction.

Figure 1 - SAT operation for a loop

When a port receives a SAT character it forwards or reflects the character via the other port to the next node if it is Satisfied.  Otherwise the port holds the character until it is Satisfied, even if it is waiting for RR.  Then it forwards or reflects the SAT character and sets the frame counter to zero.

Privileged and control frames are not controlled by the SAT algorithm and, subject to the link protocol, they may be sent at any time.  A port is allowed to originate an Application frame, subject to the link protocol, only under either of the following conditions.

- The port is holding the SAT character and the port is not Satisfied.

- There is no cut through frame waiting to be forwarded from another port, and Idle quota is greater than frame counter.

The ability of a port to originate frames can be broken into two cases (a and b), with one case (c) not able to originate frames.

- frame counter < Hold quota.
Before the SAT character arrives, the port shall only originate frames when there are no cut through frames.  After the SAT character arrives, the port shall hold cut through frames until Satisfied, after which the SAT character is forwarded or reflected and the frame counter shall be set to zero.

- frame counter ³ Hold quota and frame counter < Idle quota.
Before the SAT character arrives, the port shall only originate frames when there are no cut through frames.  When the SAT character is received, it shall be forwarded or reflected and the frame counter shall be set to zero.

- frame counter ³ Idle quota.
After frame counter ³ Idle quota, the port shall not originate any frames.  When the SAT character is received, it shall immediately be forwarded or reflected, the frame counter shall be set to zero and Application frames may be originated according to the SAT algorithm.

Correct operation of the SAT algorithm requires b quota ³ a quota.  Otherwise a node will never be Satisfied and it will not forward or reflect the SAT character.

A port shall be able to receive SAT and SAT' characters while it is in any state, provided that character synchronization is maintained.  However the port shall only transmit SAT or SAT’ characters when in the Ready state.

- 8.5.4   Extensions of the SAT algorithm for strings

The extensions to the algorithm for a string are as follows.

- If a port at the end of a string receives a SAT character then it reflects the SAT character as a SAT' character traveling in the opposite direction after it is Satisfied (see Figure 2).

- A dual-port node with both ports operational forwards SAT' received from either port unconditionally.

- If a port at the end of a string receives a SAT' character then it reflects the SAT' character as a SAT character traveling in the opposite direction immediately without the frame counter.

Figure 2 - SAT operation for a string

The reflection of SAT as SAT' and SAT' as SAT is dynamic.  If a string of dual-port nodes is connected into a closed loop then SAT' characters are no longer generated.  To avoid a previous SAT' circulating indefinitely a dual-port node with both ports operational shall discard SAT' as follows:

- A SAT' received on port 1 is discarded if no SAT has been received on port 2 since the previous SAT' was received on port 1.

- A SAT' received on port 2 is discarded if no SAT has been received on port 1 since the previous SAT' was received on port 2.

- 8.5.5   SAT regions

In some applications where each adapter talks exclusively with a subset of the devices, the traffic on a string or loop is naturally divided into separate regions, as illustrated in Figure 3.  In this example, significant traffic is only carried between the following nodes: Adapter1-Drive1, Adapter1-Drive2, Adapter2-Drive3 and Adapter2-Drive4.

Figure 3 - Application with 2 SAT regions

Editors note: In Figure 3 change the lines above the right side to arrows.  Add the following text above each set of lines: Application frames.

In this application, any trade-off between fairness and spatial reuse can be avoided by logically dividing the web into 2 regions, each with its own SAT algorithm.  This can be achieved by setting the reflect bit to 1b in the CONFIGURE PORT SMSs for either port on Device 2 (or Device 3).  The ports on Device 2 will then reflect SAT as SAT' and SAT' as SAT.  Device 2 still behaves as a dual-port node for frame routing, e.g. to allow each system to check the operation of the other.

This facility allows 100% spatial reuse between regions and perfect fairness in each region if a quota=b quota.

If one adapter fails the remaining system can take over all of the devices after possibly altering the reflect, a quota, and b quota fields of the CONFIGURE PORT SMS.

- 8.5.6   Timing requirements of the SAT algorithm

All ports incorporate a sat time-out timer to detect loss of the SAT character due to a link error.  The sat time-out timer is set to zero when the port forwards or reflects a SAT character.  If the sat time-out timer counts 100 ms, then a SAT character is created and treated as if it has been received.  This may lead to multiple SAT characters circulating in the same direction if several ports generate new SAT characters simultaneously.  However the fairness algorithm will still operate correctly and the characters will eventually merge into a single character as described below.

If a port receives a SAT character while it is holding the previous SAT then it discards the second SAT character.

Each of the following functions shall not introduce a delay exceeding 10 character periods per node:

- Forwarding the SAT character when the port is Satisfied, subject to the delay described in the paragraph below.

- Reflecting SAT as SAT' when the port is Satisfied.

- Forwarding SAT'.

- Reflecting SAT' as SAT.

For a loop or string with only a few nodes the SAT character can rotate very rapidly if all nodes are Satisfied.  This could impact frames being transmitted in the other direction.  Therefore ports shall perform the following two delays.

- A port shall not forward a SAT character until at least 100 character periods have elapsed since it forwarded the previous SAT character.

- A port shall not reflect a SAT character as a SAT’ character until at least 100 character periods have elapsed since it refelcted the previous SAT character.

- 8.5.7   Configuring the quotas

The a quota and b quota values for each port are allocated by the master node in a CONFIGURE PORT SMS.  The optimum values depend on the total number of nodes in each string or loop and the traffic pattern.  This section shows how they can be determined.

Let sat rotation be the time taken by the SAT character to circulate once round a string or loop, as measured in character periods.  sat rotation is a minimum when all ports are Satisfied and they forward the SAT characters immediately (SAT’ characters are always forwarded immediately).  sat rotation will increase when the web becomes heavily loaded and individual ports deliberately hold the SAT character.

Let sat delay be the minimum time to propagate or reflect SAT or SAT' characters through a single node whose inbound port is Satisfied, as measured in character periods.

For a string with N nodes the minimum sat rotation is 2*N*sat delay characters.  The corresponding value for a loop is only N*sat delay since the propagation delay for SAT' is eliminated.  sat rotation is always greater than 100 character periods.  However after a link failure the loop becomes a string and the minimum sat rotation could double.  Therefore it is advisable to always allow for a minimum sat rotation of 2*N*sat delay.

The performance advantages of SSA are partly due to its ability to frame-multiplex SMSs and data with low latency.  For optimum frame multiplexing and spatial reuse the sat rotation time should be minimized.  This applies both under light load (when most nodes are Satisfied) and also under heavy load (when nodes may hold the SAT character).

For long strings sat delay is a major contributor to sat rotation.  Consequently sat delay has been specified as 10 characters maximum.  The SAT and SAT' characters also have a high priority for transmission, exceeded only by the second character of a protocol pair (e.g. ACK-ACK).

Let frame length be the total length of a frame.  This is typically 136 characters for a data frame (FLAG, control field, address(Path and Channel), 128 bytes of data and 4 bytes of CRC).  To avoid throttling when a single node wants to originate data frames, b quota ³ (sat rotation/frame length) ³ (2*N*sat delay/frame length)  For the extreme case of a string with 129 nodes this requires a minimum b quota of 19 frames.

In general there is a compromise between fairness and spatial reuse.  Setting b quota=a quota guarantees perfect fairness but it limits spatial reuse.  Choosing b quota»a quota permits maximum spatial reuse and therefore maximum total through-put.  However in some scenarios this would highly favor the active node which is furthest upstream since cut through traffic normally has priority.  Therefore the upstream node might be able to originate b quota frames for each rotation of the SAT character, whereas each down-stream node could only originate a quota frames.  A reasonable compromise for most applications is b quota»4*a quota

For Informative reference only (not part of this standard) I. Cidon and Y. Ofek, “MetaRing: A Full-Duplex Ring with Fairness and Spatial Reuse”, IEEE INFOCOM ‘90, pp. 969-981.