Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a187R0
Date:	September 25, 1995
Project:	X3T10.1 / 1051
Ref Doc.:	SSA-S2P rev 4
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Alternate pathing proposal

BACKGROUND

At the last plenary (August 1995 in Ithica NY), the possibility of an initiator issuing commands to a given target down a single path while using multiple paths for the data transfer to allowing load balancing was discussed with interest.  At the time, I (foolishly) offered to generate a proposal on this topic.  Here is the proposal.

PROPOSAL

The current write up (8.5 Alternative paths) is confusing and thereby misleading.  I propose the following section replace the current section 8.5.

- 8.5   Use of multiple paths

An I/O process for a read command consists of the SCSI COMMAND SMS, optional DATA READY SMS(s), optional DATA REPLY SMS(s), read data and a SCSI STATUS SMS.  The optional DATA READY SMS, read data and SCSI STATUS SMS will use the same path and port (as specified by the initiator id field of the SCSI COMMAND SMS).  The SCSI COMMAND SMS and optional DATA REPLY SMS could arrive on any port from any path.

An I/O process for a write command consists of the SCSI COMMAND SMS, DATA REQUEST SMS(s), write data and a SCSI STATUS SMS.  The DATA REQUEST SMS and SCSI STATUS SMS will use the same path and port (as specified by the initiator id field of the SCSI COMMAND SMS).  The SCSI COMMAND SMS and write data could arrive on any port from any path.

The use of multiple paths between a given initiator and target can be broken into four methods as follows.  Which category used will depend on whether the I/O processes must arrive in order at the target or the data and status must arrive in order at the initiator.

- 8.5.1   One path for all I/O processes

If only one path is used for all I/O processes, then the ordering of commands received by the target can be guaranteed.  The initiator will receive SCSI STATUS SMSs in the order they were executed within the target.

This method is the most restrictive since the alternative paths are not used.  Instead, when a path fails, the alternative path is used and any response from the failed path is ignored.

Aborting or clearing commands can be done by sending an ABORT SMS, CLEAR QUEUE SMS or DEVICE RESET down the path used by the commands.  A SCSI RESPONSE SMS will assure that no aborted or cleared command’s SCSI STATUS SMS(s) will arrive after the SCSI RESPONSE SMS for the aborting of command(s) or clearing of the queue.

- 8.5.2   All Commands use one path, remainder of I/O process can use multiple paths

If all I/O processes use the same path to issues commands, then the ordering of commands received by the target can be guaranteed.  If the remainder of the I/O processes use several paths, the initiator may receive the SCSI STATUS SMSs in a different order than the I/O processes were executed in the target.

This method allows the initiator to maintain command ordering while allowing the remainder of the I/O process to use multiple paths.  For example, the initiator can balance the link usage by directing the data on the path that may not be as busy.  A long transfer may be using one set of links, while another transfer uses another set of links.  For targets that can handle multiple simultaneous transfers, two reads could occur at the same time using different ports.

Aborting or clearing commands can be done by sending an ABORT SMS, CLEAR QUEUE SMS or DEVICE RESET down all the paths used by the I/O processes.  When all the SCSI RESPONSE SMSs have been received the initiator is assured that no aborted or cleared commands SCSI STATUS SMS(s) will arrive after the last SCSI RESPONSE SMS for the abort or clearing of commands.

- 8.5.3   Different I/O processes can use different paths

If multiple paths are used to send I/O processes (but all of an I/O process uses a single path), then the ordering of commands received by the target cannot be guaranteed.  The initiator may receive the SCSI STATUS SMSs in a different order than the I/O processes were executed in the target.

Aborting or clearing commands can be done by sending an ABORT SMS, CLEAR QUEUE SMS or DEVICE RESET down all the paths used by the I/O processes.  When all the SCSI RESPONSE SMSs have been received the initiator is assured that no aborted or cleared commands SCSI STATUS SMS(s) will arrive after the last SCSI RESPONSE SMS for the abort or clearing of commands.

- 8.5.4   Each I/O process can use multiple paths

If multiple paths are used to send I/O processes (and an I/O process can use multiple paths), then the ordering of commands received by the target cannot be guaranteed.  The initiator may receive the SCSI STATUS SMSs in a different order than the I/O processes were executed in the target.

Aborting or clearing commands can be done by sending an ABORT SMS, CLEAR QUEUE SMS or DEVICE RESET down all the paths used by the I/O processes.  When all the SCSI RESPONSE SMSs have been received the initiator is assured that no aborted or cleared commands SCSI STATUS SMS(s) will arrive after the last SCSI RESPONSE SMS for the abort or clearing of commands.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com