Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a195R0
Date:	September 24, 1995
Project:	x3t10.1/0989-D
Ref Doc.:	SSA-TL1 rev 6c 
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Link Reset Handling process

BACKGROUND

The Link ERP process defines what happens when a link detects an error and sends a Link Reset frame to the remote port.  Nowhere is it described what happens when a Link Reset frame is received.

PROPOSAL

Add the following section under the Error handling section.

- 10.x   Link Reset Handling

When a port receives a Link reset frame, the following procedure is executed.  To facilitate cross referencing, when the Link Reset handler exits, the identified alert code value exactly matches the alert code value of the Asynchronous Alert  which is generated.

- The port waits until the transmitter has finished sending the current frame, if any.  Optionally the transmitter may choose to abort the current frame.  The port is then placed in the Check state.

- The node then builds the Link Status Byte by reference to the port’s hardware.  The port shall transmit a Link Reset frame.  If no acknowledgment is received the port retransmits the Link Reset frame once.  If no acknowledgment is received to the retransmitted frame, then the node waits 25 ms to ensure that the remote port recognizes an unrecoverable error, sets the port in the Disabled state, and exits with an alert code value of  Link reset failed.

- The node calculates the number of outbound frames for which an acknowledgment is outstanding for the local port.  When using strict cyclic buffers this can be obtained as follows:

- Q=(transmit pointer - retry pointer) mod N

- where N is the number of transmit buffers provided.  Q should be 0 or 1 frames.

- The node also calculates the number of outbound frames for which the local port is expecting an acknowledgment but which have not been received by the remote port, where local refers to the ports transmit sequence number and remote refers to the rsn field received in the Link Status Byte.

- P=(local transmit sequence number  - remote rsn)mod 4

- where P should be less than or equal to Q.

- If either Q > 1 or P > Q then the ERP waits 25 ms to ensure that the remote port recognizes an unrecoverable error, places the port in the Disabled state and it exits with an alert code value of  Invalid retry status.

- The node arranges to resend the number of frames indicated by subtracting P from its transmit pointer, modulo N.  Those outbound buffers that do not need to be retransmitted shall now be discarded using the following algorithm.

- Do while retry pointer ¹ transmit pointer;
		Discard buffer at retry pointer;
		Increment retry pointer modulo N;
	End;

- At this point the frame buffers and transmit pointer is set to retransmit the proper number of frames after the node re-establishes communication following the port being placed in the Disabled state.

- The port waits until the receiver detects DIS characters.  This is required to synchronize the local and remote ports and prevent the transmitter sending RR while the remote port is still in the Check state.

- The ERP places the port in the Enabled state.

- If the receiver detects a FLAG character within 5 ms of placing the port in the Enabled state, the ERP places the port in the Ready state, otherwise the ERP exits with an meaning code of Time-out waiting for Ready state.  This may indicate that the remote node has powered-off or suffered a catastrophic error.

- The Link Reset process terminates successfully.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com