Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a185R1
Date:	August 30, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 6a
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Source zero fill, destination ignore proposal

BACKGROUND

In a previous meeting, the pad fill of SMSs was changed to don’t care.  This is a compatibility problem.  It should be expected that people will begin shipping product with the TL1 version specified for this standard prior to this standard being finalized (it’s wrong but a fact of life).  If we wish to add an additional byte to an SMS we have a problem.  Since someone claiming the official SSA-TL1 can pad with non-zero, X3T10.1 cannot know how devices will respond (was the sender official and want the new function, or was he “bad” and pad filled with non-zero?).  The result is that X3T10.1 will face resistance to adding new bytes.

I propose forcing zero pad for the transmitter, but the receiver can ignore pad.  If a new byte is defined, he must check it (but the transmitter sent zero so it’s OK).

REVISIONS

Revision 1 was approved at the X3T10.1 August 95 plenary meeting and involves a very minor wording change.

PROPOSAL

Change the second paragraph of 11 from 
“An SMS has a maximum length of 32 bytes.  Only the bytes defined in the appropriate SMS structure are required, however the SMS may be padded to a total length of 32 bytes.  The pad bytes shall be ignored.
to
“An SMS has a maximum length of 32 bytes.  Only the bytes defined in the appropriate SMS structure are required, however the SMS may be padded up to a total length of 32 bytes.  The source node shall pad (if any) with zeros, and the destination node shall ignore any pad bytes.”

The SSA-S2P spec needs to make the same change.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com