Accredited Standards Committee

X3, Information Processing Systems

Doc:		X3T10.1/95a174r1

Date:		July 27, 1995

Project:

Ref Doc.:	SSA-TL1 rev 5

Reply to:	Kurt Kastein

To: X3T10.1 Membership

From:	Kurt Kastein

Subject: Clarification of “Frame Less Than 6” Protocol Error Proposal:

Modify clause 10.1.6 of SSA-TL1 revision 5

Essence of Change:

A possibility for an error on the link that is not flagged by the link error protocol exists.  The TL covers the case of a frame being received that is less than 6 data characters.  This, however, does not cover all cases of non-control frames that are too short.  A non-control frame that has 6 data characters and is received without other errors (including good CRC) is NOT considered a “bad” frame by the TL, even though it is clearly invalid.

Specifics of Change:

A sample bad frame with only 6 data characters follows:

Flag--Control (00h)--Path (00h)--CRC1--CRC2--CRC3--CRC4--Flag

Note the absence of a Channel byte in the above frame.  If the CRC calculated across the Control and Path bytes is correct, and CRC1 happens to have a value equal to a channel which is allocated by the destination node, the frame will be received by the destination node without error.

A protocol error should be identified when a non-control frame is received (cut-through or to destination node) that has less than 7 total data characters.

Actual Section Modification:

Under section 10.1.6, add implementors note stating:

In reference to item a) above:  A short non-control frame with less than 7 data characters between 2 FLAG characters should also be identified as a protocol error.

Note:  This should be recommended in SSA-TL1 as current known silicon including the IBM SIC chip and the Symbios SSAsquatch devices will not report this as an error.  It is likely also that any other designs in progress using the released intellectual property from IBM will contain this protocol error issue.  Future TL documents should require this for compliance.

Sincerely,

Kurt Kastein

Symbios Logic Inc.

(970) 225-4827	PH.

(970) 226-9686	FAX