Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a171R3
Date:	September 24, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 6 
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Comments on SSA-TL1 rev 6 by John Scheible

BACKGROUND

I placed all my editorial comments/changes in rev 6a of SSA-TL1 (and should not have changed anything technical).  This proposal documents my technical changes.  In this first revision, some issues will just be raised without proposed solutions.  Then I will document the agreed to solutions in a later revision.

REVISIONS:

Revision 1 added technical changes 5 and 6 along with editorial changes 4,8,9,10, 14 and 15.  Revision 2 deleted items 5, 7, and 16, modified items 4 and 6, and resolved items 2,8,9,13,14,15,and 20.  Revision 3 includes those changes approved at the August 1995 plenary session, where 3,10, and 13 were modified and 6 was deleted.

PROPOSAL

This proposal consists of two sections, technical changes to rev 6 of SSA-TL1 and editorial changes to rev 6a.

Make the following changes to rev 6 of SSA-TL1:

- Title page - The abstract states:
“SSA defines a hierarchy of serial interfaces for use within future storage sub-systems and other purposes within its distance and performance characteristics.”
change this to:
“SSA defines a serial interface hierarchy to be used for purposes within its distance and performance characteristics, including but not limited to storage subsystems.”

- 1, last sentence of the first paragraph.
SSA-TL1 does not need to be SAM compliant.  Remove the sentence.

- 5.3 - “Cyclic path is mentioned by not defined.
  Delete all references.

- 7.3, last sentence of third paragraph states (referring to channels), delete the sentence:
”In practice, a device may support only 1 data channel, but an adapter or controller will typically provide several.”

- 7.4 Data field, fourth paragraph - Change first sentence from
“Except for SMS frames and the frame containing the last byte of a data transfer, the length of the data field shall be a multiple of 8 bytes.”
to
“Except for the Data frame containing the last byte of a data transfer, the length of the data field of any Data frame shall be a integer multiple of 8 bytes.”

- 9.6.2, last paragraph - It states:
“This resets the items listed in 9.2.1.”
The items referenced are in 9.6.6 “Disabled state”.  9.6.6 needs to be folded into 9.2.1.

- 9.6.4 - bullet 8) change:
“If the Current node is a switch, the Master marks the port indicated by the Current port field in QUERY NODE REPLY SMS  as Selected and continues with step 9.”
“If the Current node is a switch, the Master marks the port indicated by the port field in QUERY NODE REPLY SMS  as Selected and continues with step 9.”

- 9.7.1 - The editors note states:
“EDITOR’S NOTE: add IEEE footnote.”
UNRESOLVED: Do we know who to point to?  Dal says there will not be a document reference.

- 10.1.9, bullet a) - Change
“The frame contains more than 139 data characters.  Note that the receiver must continue to accumulate the CRC until the trailing FLAG in order to verify that there hasn't been a transmission error, e.g.  a corrupted FLAG character.”
to
“The frame contains more than 139 data characters.  The receiver must continue to accumulate the CRC until the trailing FLAG since a CRC ERROR ERP exit takes precidence over a FRAME REJECT EPR exit.

- 10.2.3, bullet 13 - The third line of indented text states:
“Increment Retry pointer modulo;”
Change this to “increment Retry pointer modulo N”

- 10.4.2, bullet 4) - change:
“Each initiator unconfigures the affected link(s) and node(s).”
to
“Each initiator invalidates the entries in the Configuration table for the affected link(s) and port(s).”
Similar changes are needed in 10.4.9 (3 places) and 11.2.3.

- 11.2.3, the last sentence of the paragraph starting “The reflect bit...” states:
“The reflect bit shall be cleared for single port and switch nodes.”
If this means the reflect bit shall be set to 1b for switches or single port nodes, then we need to say what happens if it is set to 1b (INVALID FIELD return code value in RESPONSE SMS?).  I propose changing it to:
“The reflect bit shall be ignored when the node is a switch, a single port node, or a dual port with one port’s operational flag cleared.”

- 11.2.3 , last (2) sentences in the third paragraph after Table 25 - They state:
“Subsequent increments to the counter above the threshold shall not generate an asynchronous alert;;  an initiator shall set the Link ERP counter to restart the alarm threshold function.  See 11.2.8.”
Change this to:
“Subsequent increments to the Link ERP counter above the alarm threshold value shall not generate an Asynchronous Alert.  If a valid QUERY PORT SMS is received with the cle flag set, the specified port shall set the Link ERP counter to zero and thereby restart the alarm threshold function (See 11.2.7).”

- 12.3 - states:
“Editor’s note: Where is the configuration process?”
UNRESOLVED: The Configuration process seems like a key process.  We have Web Reset, could we have a Configuration process, even if only for a string and loop?  If we have one, we could clarify undefined terms such as “walking the web”.

- 13.4.11, under bullet b) - It states:
Editors note: Do any implementations require the data to arrive at the target on the same port the DATA REQUEST was sent from?  In other words is the DMA hardware set to accept data from a particular port?
UNRESOLVED: I believe the answer is yes, in which case I need to add an Outstanding Command table to allow the proper path/port to be picked that was used for the command.

Make the following editorial changes to rev 6a of SSA-TL1

- Global - Remove all single quote from words (such as ‘special’ in 6.1.2).  The original intention was to have these words have a special meaning.  This could be changed to first letter capitalized words which are officially defined.

- Global - Make sure ill defined terms such as “walking the web” are properly defined.

- Global - Verify all table text is 9 point text (esp. all caps).  Some macros may have changed it to 10 pitch.

- Global - Change network to web.  Should web be Web (capitalized)?

- Back of the title page - Email address needs a period (i.e. ken_hallam@mcimail.com)

- Back of the title page - Greg is not acting, his area code is 970, and his FAX number is 970-226-9686.

- Page ii - The ii, iv, vi, viii, and x page numbers are missing.

- Page vi - Remove the first line from the Table List (Contents).

- Page x, first paragraph - The standard is divided into 13 major clauses (not 12).

- 3.3 - Move to 4.2 to match ANSI style

- 4 - Add  clause “4.1 Interface layers” after 4.

- Figure 7 - Change the last column header from 0 to Bit 0 for consistency.

- 10.2 - The “Reference source not found” needs to be “10.4”.

- 10.2.3.2 - The “Reference source not found” references need to be “10.4”.

- 11 - Rename clause 11 to SSA-TL1 message structures.

- Table 19 header - sms code and SMS frame type should be small caps.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com