Accredited Standards Committee
X3, Information Processing Systems

Doc: 		X3T10.1/95a191r0
Date:		September 30, 1995
Project:	
Ref Doc.:	X3T10.1/xxxxD, PH1
Reply to:	Mark DeWilde

To: 	   	X3T10.1 Membership
From:	Mark DeWilde

Subject: Minimum ESD Tolerance Specification for SSA Physical Layer Devices.

BACKGROUND

The ESD tolerance specification has been dropped from PH1.  Additionally, current studies of termination will most probably lead to a severe limitation of the capacitance and inductance that is permissible as measured at the SSA connector.  This limitation will make the use of ESD protection external to the SSA chip difficult, and will discourage their use.  As SSA silicon speeds increase, ESD tolerance becomes more and more difficult to build into the silicon, and will be left to the designer of the external board to accommodate.  Since the external connectors specified have their contacts easily accessible at the surface of the connector, ESD to them is highly likely.  The lack of a consistent and reasonable value for ESD resistance will permit vendors of extremely cost reduced products to produce ESD sensitive devices.  These devices could result in poor reliability in some SSA systems, and would harm the reputation of SSA in general.  While not a pure technical requirement,  including some guidance on product ESD resistance into the spec could  help ensure SSA product quality and ruggedness.

PROPOSAL

Add a requirement or recommendation to the SSA-PH1 specification that all SSA Inputs and Outputs connected to accessible contacts tolerate a minimum differential or single ended voltage pulse relative to chassis ground that approximates the electrostatic discharge of a human being.  A specification like IEC 801-2 or the EMC’s generic immunity standard EN50082-1 is excellent guidance on the test pulse. IEC 801-2 calls out a model of the human ESD characteristics consisting of a main (body)capacitance, a series resistor, and a secondary (hand) capacitance then connected to the test “finger” that is discharged to the equipment under test.  Total capacitance is 150 pF, with the a 330 ohm series resistor.  Either direct contact or air discharge may be used, with the initial voltage on the capacitors set according to the table below.  The current waveform has two peaks, the first, emulating the discharge of the hand capacitance, has a rise time of 1 nS and a duration of ~3 nS, superimposed on a slower waveform emulating the body capacitance discharge peaking at 30 ns. The table indicates the voltages depending on the level of the test. Level 1 is the least stringent.  The standard describes tests applying 300 such pulses and keeping statistics on transient & soft recoverable, soft non-recoverable and hard errors. No operator intervention is required to recover from transient and soft-recoverable errors. The LERP of the link would handle all such errors.  Soft Non-recoverable will require a node reset and re-start, and hard errors require equipment repair.

| Level | Direct Injection voltage | Air Discharge Voltage | First Current Peak (+/- 10%) | Current peak at 30 ns (+/- 30%) | current at 60 ns(+/- 30%) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 KV | 2 KV | 7.5 A | 4 A | 2 A |
| 2 | 4 KV | 4 KV | 15 A | 8 A | 4 A |
| 3 | 6 KV | 8 KV | 22.5 A | 12 A | 6 A |
| 4 | 8 KV | 15 KV | 30 A | 16 A | 8 A |

The definition of a  “tolerable” failure rate is subjective, but typical values might be  no more than 0.5% hard errors, 5% soft non-correctable, 94.5% soft correctable and transient errors. The Generic ESD immunity standard EN50082-1 calls out the 8 KV air discharge test for most equipment. I propose Level 3 be recommended for external SSA connectors, and Level 1 for Internal SSA connectors, since reasonable ESD protection mechanisms such as grounding straps can be expected to be used by internal component installers.

Sincerely,

Mark A. DeWilde

Principal System Architect

Pathlight Technologies

Voice:	(607)266-4000 X-403
FAX: 	(607)266-0352

Email:	mark@pathlight.com