Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a194R0
Date:	September 25, 1995
Project:	X3T10.1 / 0989
Ref Doc.:	SSA-TL1 rev 7
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Assumptions made in SSA-TL1 rev 7x

BACKGROUND

During the editing of SSA-TL1, I came across several areas that needed clarification or needed to be consistent.  This proposal documents the decisions/actions, and raises some points for discussion.

The proposal is broken into two sections.  The first is clarifications that need discussion.  The second is clarification which just make sections consistent and do not need discussion (in my view).

PROPOSAL (Needs discussion)

- Does QUERY NODE SMS with the dr bit set remove the entries from the Initiator table?
I propose to change the third sentence under the dr bit definition paragraph from:
“appropriate If the dr bit is set, the Initiator table shall not be updated”. 
to
“If the dr bit is set, the appropriate elements are deleted from the port array, return path array, and initiator id array fields of the Initiator table entry associated with the unique id field value.”

- What happens when a Master receives an ASYNC ALERT SMS with a MASTER GENERATED ASYNC ALERT type field value?  I said (already in rev 7)...
“The MASTER GENERATED ASYNC ALERT type field value shall not be used by Targets.  The Master will generate an ASYNC REPLY SMS, log the error, but will not send a MASTER ALERT SMS to the other initiators.”

- What so we do in an initiator does not respond to the Master Alert during Asynchronous Alert handling?
I believe if there is no RESPONSE SMS to MASTER ALERT SMS then simply performs a Web Reset process.  This may seem drastic but in this case the problem is usually not in the initiator that is addressed by the MASTER ALERT SMS; it is more likely to be a communication block in the Web (e.g. another node may be hung and not processing its inbound buffers).  This is precisely the situation for which the Web Reset process was intended;  A total reset frame is unlikely to be effective because there is no way to determine which node is at fault.
I propose changing section 12.5 bullet 6 from...
“6)   Wait for RESPONSE SMS from each of the initiators for the MASTER ALERT SMS.”
to...
“6)   Wait for RESPONSE SMS from each of the initiators for the MASTER ALERT SMS.  If any initiator fails to return a RESPONSE SMS within 200 ms, then the Web Reset process shall be performed (See 9.5.6).”

- In the QUIESCE SMS with the 3 party bit cleared, do we match the return path field as being registered by the unique id field?  If 3) above is accepted, then this does not matter (answer no).  If 3) is rejected, then I propose no change to the wording (no checking).  In either case, no change to the wording.

- The return path field should not be validated (as registered) for the CONFIGURE PORT SMS.  The reason is that for loops, the alert path must be set for each port.  If the registration of the return path is checked, the initiator must register both paths.
The return path field should not be validated (as registered) for the QUERY PROTOCOL SMS.  The reason is that the initiator may wish to know if it can talk the same language as the node before registering.
The return path field should not be validated (as registered) for a QUIESCE SMS.  The initiator was using one path that has broken, so it issues a QUIESCE SMS down the alternate path that it may not be registered on.  By checking the registration, we are forcing a QUERY NODE SMS/QUERY NODE REPLY SMS pair just to quiesce.  It is cleaner to remove the restriction.
I propose that we do not check the return path field as being registered for any of the transport layer privileged messages.  The following transport layer messages use return path...
CONFIGURE PORT - Should not check for the reasons stated above
MASTER ALERT - Since we have a problem, perhaps we should not check.  An SMS from someone “strange” could cause extra errors logged or an extra configuration process.  Ho hum.
QUERY NODE - We do not check now since this is the SMS that registers.
QUERY PORT - The link erp counter could cleared by an unauthorized person.  Ho hum.
QUERY PROTOCOL - We should not check for the reasons above.
QUIESCE - We should not check for the reasons above.
I propose we change 11.1.3 first paragraph and number 1) from...
“The fields of an SMS shall be validated with the exception of  those in the QUERY NODE SMS, QUERY NODE REPLY SMS, CONFIGURE PORT SMS, and the response SMS associated with the CONFIGURE PORT SMS.  If an Asynchronous Alert is generated then the message is then terminated, without any other response
1)   If the return path or initiator id field is unknown, then generate an ASYNC ALERT SMS with an Unknown Return path or INITIATOR id meaning code, with two exceptions.  The QUERY NODE accepts any return path, since it builds the Initiator table.  The CONFIGURE PORT SMS responds with a RESPONSE SMS with a return code of UNKNOWN RETURN PATH for an unknown return path, since an alert path may not be accepted yet for the ASYNC ALERT SMS.”
to...
“The fields of an SMS shall be validated with the exception of the return path field or the pad bytes in the QUERY NODE SMS and QUERY NODE REPLY SMSs.  If an Asynchronous Alert is generated then the message is then terminated, without any other response.
1)   If the initiator id field is not registered in the Initiator table, then generate an ASYNC ALERT SMS with an Unknown return path or INITIATOR id meaning code.”

PROPOSAL (Clarifications, discussion not mandatory)

- The Disabled state states that the rr pending flag and waiting for rr flag are set and the ack pending flag and waiting for ack flag are cleared in the Disabled state.  The Acknowledgments and Pacing sections state they are initialized in the Ready state.
I made them all initialized in the Disabled state.

- In the Total Reset or Absolute Reset sections, the text said...
        a)	A transmitting port shall send Total or Absolute resets while in the Enabled or Ready states.  It 	cannot send frames when Enabled.  It should be Ready state.
        b)	A receiving port shall recognize Total or Absolute Reset frames in any state.  Clearly 	impossible and should be the Ready or Check states.
        c)	If the output port is in the Disabled or Check states, a Total or Absolute Reset frame shall be 	held until it enters the Enables or Ready states.  This should be if the output port is in the 	Check state, it will hold it until the output port enters the Ready state.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com