Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a186r0
Date:	7/30/2026
Project:	
Ref Doc.:	X3T10.1/0989D rev 6b
Reply to:	Ed Carmona

To: 	X3T10.1 Membership
From:	Ed Carmona

Subject: Exiting Wrap Mode Considerations

BACKGROUND

Currently, the Configure Port SMS does not define a way for the Master to place a port out of Wrap mode once placed in Wrap mode. The following proposal defines a method for exiting Wrap mode using the Configure Port SMS.

PROPOSAL

The following proposal identifies the changes needed to the Configure Port SMS description in order to appropriately support the exiting of Wrap mode. Change bars indicate the changes required to the Configure Port SMS description.

Clause 11.2.3 CONFIGURE PORT SMS, paragraph 13

A mode field value of SET WRAP MODE is only valid if the node received the CONFIGURE PORT SMS via a different port than that being configured.  Otherwise the node returns a RESPONSE SMS with return code value  of INVALID FIELD and the port mode is unchanged.  When Wrap mode is set the port also becomes not Operational (see section 9.2.3).

Once a port has been placed in Wrap mode, the Master can cause the node to exit the Wrap mode on the port by sending a CONFIGURE PORT SMS via a different port with a mode field value of SET PRIVILEGED MODE. The port shall exit the Wrap mode and invoke the Beginning Communication process (see section 9.4).

If a port currently configured in Wrap mode recieves a CONFIGURE PORT SMS via a different port with a mode field value of SET NORMAL MODE, the node returns with a RESPONSE SMS with return code value  of INVALID FIELD and the port mode is unchanged.

Ed Carmona

Conner Peripherals, Inc.

Voice: (408) 456-4082

Email: ed.carmona@conner.com