Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a182R1
Date:	September 29, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 6a
Reply to:	Lawrence J. Lamers

To: 	   	X3T10.1 Membership
From:	Lawrence J. Lamers

Subject: Proposal for QUERY SWITCH SMS

BACKGROUND

This proposal corrects the existing limitations in the port mask allowing the architectural limit of 128 ports to be reported.

PROPOSAL

### QUERY NODE REPLY SMS

The QUERY NODE REPLY SMS as defined in Table 4 is returned when a QUERY NODE SMS is received.  The QUERY NODE REPLY SMS is returned on the same port that received the corresponding QUERY NODE SMS.

Table 1 - QUERY NODE REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (01h) |  |  |  |  |  |  |  |
| 1 | port |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | upper level protocol |  |  |  |  |  |  |  |
| 5 | itf | master priority |  |  | Reserved |  |  |  |
| 6 | total ports |  |  |  |  |  |  |  |
| 7 | ssa-tl1 version (02h) |  |  |  |  |  |  |  |
| 8 | unique id |  |  |  |  |  |  |  |
| 9 | unique id |  |  |  |  |  |  |  |
| 10 | unique id |  |  |  |  |  |  |  |
| 11 | unique id |  |  |  |  |  |  |  |
| 12 | unique id |  |  |  |  |  |  |  |
| 13 | unique id |  |  |  |  |  |  |  |
| 14 | unique id |  |  |  |  |  |  |  |
| 15 | unique id |  |  |  |  |  |  |  |
| 16 | initiator id |  |  |  |  |  |  |  |
| 17 | initiator id |  |  |  |  |  |  |  |
| 18 | initiator id |  |  |  |  |  |  |  |
| 19 | initiator id |  |  |  |  |  |  |  |
| 20 | p1o | p2o | reserved | reserved | reserved | reserved | reserved | reserved |

The port field indicates the number of the port currently being used

The total ports field contains a value that is one less than the number of ports implemented. .  If this value exceeds two the QUERY SWITCH SMS should beused toretrieve a port mask for the switch.

### QUERY SWITCH SMS

The QUERY SWITCH SMS is sent from a initiator to each switch operational node during the configuration process to determine the number of ports on the node.

The QUERY SWITCH SMS is defined in Table 2.

Table 2 - QUERY SWITCH SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (nnh) |  |  |  |  |  |  |  |
| 1 | reserved |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | return path |  |  |  |  |  |  |  |
| 5 | return path |  |  |  |  |  |  |  |
| 6 | return path |  |  |  |  |  |  |  |
| 7 | return path |  |  |  |  |  |  |  |
| 8 | unique id |  |  |  |  |  |  |  |
| 9 | unique id |  |  |  |  |  |  |  |
| 10 | unique id |  |  |  |  |  |  |  |
| 11 | unique id |  |  |  |  |  |  |  |
| 12 | unique id |  |  |  |  |  |  |  |
| 13 | unique id |  |  |  |  |  |  |  |
| 14 | unique id |  |  |  |  |  |  |  |
| 15 | unique id |  |  |  |  |  |  |  |
| 16 |  | Reserved |  |  |  |  |  |  |

The ssa-tl1 version field defined in Table 3 identifies the version of SSA-TL1 being used by the sender.

Table 3 - QUERY NODE SMS

| Version | Description |
| --- | --- |
| 00h-01h | SSA-TL1 implementations before standardization |
| 02h | This standard |
| 03h-FFh | Reserved |

The tag field is returned in the QUERY NODE REPLY SMS.  The tag is assigned by the initiator and it shall be unique among the tag values that are currently active from that initiator.

The return path field specifies the path component that shall be placed in the Initiator table entry created in response to this QUERY NODE SMS, if an entry is created.  This value is used for the address field of the resulting QUERY NODE REPLY SMS, and will be used for the address field of any future application SMS that utilizes this Initiator table entry.

The unique id field contains the Unique ID of the initiator that issued QUERY NODE SMS.

The disable registration (dr) flag controls the updating of the Initiator table in the target.  If the dr flag is set to 0b and the destination node is a target, then the target shall enter the specified return path and unique id into its Initiator table.  If the dr flag is set to 1b, the Initiator table shall not be updated. If an initiator intends to use several alternative paths to the same target node then it must issue QUERY NODE SMS with the dr flag is set to 0b once over each path.

The destination node normally returns a QUERY NODE REPLY SMS.  If a field is invalid then the destination node returns a RESPONSE SMS instead of QUERY NODE REPLY SMS.

### QUERY SWITCH REPLY SMS

The QUERY SWIRCH REPLY SMS as defined in Table 4 is returned when a QUERY NODE SMS is received.  The QUERY NODE REPLY SMS is returned on the same port that received the corresponding QUERY NODE SMS.

Table 4 - QUERY SWITCH REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (xxh) |  |  |  |  |  |  |  |
| 1 | port |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | port mask |  |  |  |  |  |  |  |
|  | port mask |  |  |  |  |  |  |  |
| 23 | port mask |  |  |  |  |  |  |  |

The port field indicates the number of the port currently being used.

The port mask field is a bit vector, where a bit that is set indicates that the port is present.