Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a184R0
Date:	August 27, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 6a
Reply to:	Lawrence J. Lamers

To: 	   	X3T10.1 Membership
From:	Lawrence J. Lamers

Subject: Proposal for an OPERATION COMPLETE

BACKGROUND

Software drivers would benefit from the codification of an operation complete under normal circumstances.  This construct is in effect today for SSA but there is no guarantee that it will be preserved in future protocol definitions if it is not codified.

PROPOSAL

Operation Complete SMS:  When the 1st byte of an SMS is 80h or above and the 2nd byte is 11h, the I/O process is complete.  If there is no logical completion to the I/O process, this does not imply that a completion SMS must be used (however, it is recommended since it allows the Initiator to release the resources for the I/O process without knowing the underlying protocol).  Bytes 4 and up are application specific.

In table 19, add the following line:

| SMS | SMS code | APP code | Frame type | Sent from | Received by |
| --- | --- | --- | --- | --- | --- |
| OPERATION COMPLETE | 80h-8Fh | 11h | Protocol | Target | Initiator |

3.    Table 18 of the Transport Layer should specify the tag position to always be in bytes 2 and 3.  This allows an SMS to be matched to internal structures without regard to the Upper Level Protocol.