Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a193r0
Date:	September 14, 1995	
Project:	SSA-40 (and up)
Ref Doc.:	X3T10.1/
Reply to:	Mark DeWilde

To: 	X3T10.1 Membership
From:	Mark DeWilde

Subject: SSA Port Speed Negotiation Proposal:

SSA-PH1 in section 6, last paragraph, states:

“The receiver shall acquire character synchronization from either of the comma characters K28.1 or K28.5. “  [... ]  “However, the receiver must always achieve character synchronization within a total of 200 character periods”.

SSA-TL1 in section 9.2.1 and 9.2.2 states:

“The minimum duration of the disabled state is 200 character periods. This allows sufficient time to acquire character synchronization when beginning communication since the remote port cannot enter the ready state until the local port changes to the enabled state and sends FLAG characters.”

“When a port is ready to begin communications, the node processor will change the port to the Enabled state (see 9.5). In the Enabled state, the transmitter sends FLAG characters and the receiver looks for FLAG characters.  When a receiver decodes a FLAG character the port automatically enters the Ready state.”

Problem 1: Editorial

The character set is not defined in PH1, but rather in TL1.  If the definition remains in TL1, then the PH1 clause should be moved to TL1, where the word “character” is defined.  Alternately, the definitions of the character set could be moved to PH1, in which case the discussion of character synchronization in the receiver would use terms defined at that level of the specification.  I would recommend the latter.

Problem 2:  Technical

The quoted sections could be interpreted in three ways.  One interpretation  based on the word “character” meaning “received character” would indicate that prior to entering the enabled port state, synchronization of the port  receiver must be achieved.  In TL1, it is made clear that the minimum duration of the disabled state is 200 character periods, and there is no maximum stated.   If the transmitter and receiver are operating at the same speed, and the receiver is synchronized, and a minimum of 200 DIS characters have been  received, then and only then would the enabled state be entered  (“When a port is ready to begin communications”) and FLAGs  sent.  A port could not be “ready to begin communication” if it’s receiver was not synchronized. The receipt of a flag character indicates the remote port’s receiver has also reached enabled state and has achieved synchronization.

The second interpretation is that the word “character” means “transmitted character”.  In this interpretation, the transmitter will transmit a fixed number  (200 or more) of  DIS characters, and then the node processor will cause it to transition to the enabled state, with no other dependencies.  At this point the transmitter transmits flags, and the receiver looks for FLAG characters, whether it has achieved synchronization or not  The implications are either that  the receiver will have synchronized within the 200 character periods if it ever will do so, or that the receiver is also capable of synchronizing on FLAG characters and achieving synchronization while in the Enabled state.   The latter is probably true in most cases, but then the enabled state is really not a transient state as described. The port could remain in the enabled case forever if synchronization did not occur using this interpretation. If this interpretation is used, then the receiver must be able to synchronize on DIS and FLAG characters, and PH1 should say so.

A third interpretation would be that the transmitter must send 200 DIS Characters, the receiver needs to be synchronized, but does not need to have received a full 200 synchronized characters prior to entering the enabled state. In this  interpretation, if the transmitter and receiver are operating at the same speed, and the receiver is synchronized for at least one character, and a minimum of 200 DIS characters have been transmitted, then and only then would the enabled state be entered and FLAGs  sent.  If the receiver was not synchronized after the transmitter had sent  200 DIS characters, then the transmitter would continue to transmit DIS characters until the receiver was synchronized.

It would seem to make little sense for a port to go to the enabled state (“when a port is ready to begin communication”), if character synchronization in both directions has not occurred.  The PH1 requirement that the receiver must (shall?) achieve synchronization within a total of 200 character periods when speed negotiation is going on is too restrictive on the negotiation algorithms using the second interpretation. The third interpretation could allow the receiver to report synchronization within a few character periods, and would allow transition to more operational states without a reasonable period to allow excessive noise on a link to be detected and force the link to a lower speed.  I favor the first interpretation, since it is compatible with all of the current proposals for speed negotiation, and requires a number of characters exceeding the frame size to be received prior to reporting a good link.

In order to allow for multiple link speeds, and clarify these section,  I recommend the following:

1.	Move sections 6.0, 6.1, 6.20, 6.21, and 6.22 to PH1, so that characters and synchronization may be discussed in PH1.  If, for clarity’s sake it is desirable to leave copies of the special character descriptions in TL1, then do so.

2.	Make clear in PH1 that synchronization and speed negotiation are performed in the disabled state, and that the port remains in the disabled state until receiver synchronization is maintained for a minimum of 200 character periods. Doing this also helps avoid the Live-lock synario described in 95a192r0, since 200 characters exceeds frame size.

3.	Describe the transition from disabled to enabled state and the changing from DIS characters to FLAG characters in PH1, so that  the “final” act of synchronization within PH1 is to enter the enabled state.  TL1 can take over from this point.

4.	Select one of the speed negotiation  algorithms described in the next section.

Speed Negotiation Background

There are two different speed negotiation algorithms currently being discussed for multi-speed SSA systems.  The first algorithm (described in X3T10.1/95a179r0) starts attempting to synchronize at the highest speed and steps down to the next lower speed if it fails to do so after 1 mS.   Note that this time interval requires more than the transmitted 200 character minimum, so interpretation #2 is not acceptable.   Since we are discussing more than two possible speeds for SSA, it can be shown that for a pair of ports with different numbers of supported speeds, the two ports will always negotiate to the lowest speed and stay there if the method as described is used..  If “1” represents the lowest speed supported, and “5” the highest, then:

attempt #	Port A Speed	Port B speed

1			5		4

2			4		3

3			3		2

4			2		1

5			1		1   ports synchronized.

This table assumes that the ports cannot determine if synchronization failed because the received bit rate was too high.   If  port B could determine that the received rate was too high, then it could choose not to step the speed down and synchronization at the highest speed would be achieved.  In this case, synchronization is achieved at the highest speed.  Detecting this cause of failure is logically easy if there are no bit errors in transmission. If there are, then a single failure rate measurement may give an incorrect reason for failure and both the nodes will step down, resulting in failure to synchronize at the highest possible rate.  This implies that the node must not only detect multiple comma characters to make a decision on synchronization, but must also detect the failure cause multiple times to reach a reliable decision.

The second algorithm has each transmitter transmitting at it’s highest speed, and the receiver of the other node cycling through it’s speeds until it locks. If it reaches lock at a speed, then it changes it’s transmitter to transmit at that speed. If it does not reach lock, the receiver continues cycling through all of it’s speeds. This method will lock at the highest speed, but requires the transmitter and receiver of each port to be capable of operating at different bit rates, which can be difficult and potentially expensive to implement.

Proposed new method  #1

A simple and fast method of synchronization trial can be achieved if we vary the time that a node will attempt synchronization based on it’s current speed. Standardize on  a maximum time for 20 MB/sec SSA, and then reduce it by half  for each higher speed step. Since the lower speed port will remain at that speed longer than the total sum of the dwell times at the higher speed, the two will synchronize at the highest common speed.  For example, choose 1 millisecond for 20 MB/sec (speed 1) Then times for speed 2, 3, 4, and 5 would be 500, 250, 125, and 62 microseconds respectively.  This assumes that higher numbers have higher bit rates and so the number of bits received during the trial time remains relatively constant.  Examples:

Time	    Port A speed	     Port B Speed

(us)

--------------------------------------------------------------------

0-62		5		4

63		4		4  Synchronized

--------------------------------------------------------------------

0-62		5		3

63-125		4		3

126-187		4		3

188-250		3		3 Synchronized

---------------------------------------------------------------------

0-62		5		2

63-125		4		2

126-187		4		2

188-250		3		2

251-312		3		2

313-375		3		2

376-437		3		2

438-500		2		2 Synchronized

----------------------------------------------------------------------

Input from the silicon designers should be used to establish the minimum synchronization trial interval at the highest clock speeds anticipated for SSA, and then the actual times derived from those numbers. This method does not require detecting that the received bit rate is higher than the transmitted one, and would minimize the number of attempts to synchronize the two ports.  Both the transmitter and receiver on a port operate at the same rate.  Allowing the ports to jump to the highest speed and retry again would probably be unnecessary, but can be considered.

Proposed new method  #2

A fourth possible method of synchronizing the speeds would be to require all nodes begin communication at 20 MB/sec.  When receiver synchronization has occurred, a node capable of a higher speed sends a string of one of the reserved UDC’s.  If the node receives a string of UDC’s back while sending of (arbitrarily) 100 UDC’s, it steps it’s transmitter and receiver up to the next highest speed and then begins synchronization.  If synchronization is achieved, then the node sends a string of the second reserved UDC to indicate success.  If the other node does not send a string of the second character back, it indicates that it could not achieve synchronization, even though capable of operating at that speed.  If no return is detected for 1000 transmit character periods, then the nodes drop back to the lower speed and remain there.  If one node synchronized, but the other could not, then the one synchronized but not receiving acknowledgment would drop to the lower speed.  The node that never synchronized would drop to the lower speed after the attempted synchronization interval.  If both nodes do synchronize at the higher speed, then both would acknowledge and would receive acknowledgment.  In this case, the process would repeat until no higher speeds were available.

Obviously, ports would need to be configured to never pass UDC’s.  Since there are numerous problems with the current definitions of UDC’s, configuration, usage, EUDC setting, switch handling of them, etc. This may not be a problem.  Nodes using SIC are not broken by this approach, since sic would not acknowledge the speed change request. The primary disadvantage is that 20 MB/sec speeds must be supported in all nodes into the future that support speed negotiation.  My suggestion is that we clarify the sections in part 1 of this proposal, and then finalize the algorithm for speed negotiation at the Havant meeting. In this way we can assure consistency between the requirements of the synchronization sections and the negotiation sections.

Sincerely,

Mark A. DeWilde, Principal System Architect

Pathlight Technologies

Voice:	(607)266-4000 X-403
FAX: 	(607)266-0352

Email:	mark@pathlight.com