Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a179r0
Date:	September 29, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 6 
Reply to:	Lawrence J. Lamers

To:  	X3T10.1 Membership
From:	Lawrence J. Lamers

Subject: Proposed Changes to SSA-TLx for 40 MB/sec

The following is 11.2.8 of SSA-TL1 modified to support 40 MB/sec.

### QUERY PORT REPLY SMS

The QUERY PORT REPLY SMS defined in Table 32 is sent from a target to an initiator in response to a QUERY PORT SMS.

Table 1 - QUERY PORT REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (0Bh) |  |  |  |  |  |  |  |
| 1 | reserved |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | link erp error count |  |  |  |  |  |  |  |
| 5 | link erp error count |  |  |  |  |  |  |  |
| 6 | alarm threshold |  |  |  |  |  |  |  |
| 7 | alarm threshold |  |  |  |  |  |  |  |
| 8 | supported speed |  |  |  |  |  |  |  |
| 9 | supported speed |  |  |  |  |  |  |  |
| 10 | current speed |  |  |  |  |  |  |  |
| 11 | current speed |  |  |  |  |  |  |  |
| 12 | reserved |  |  |  |  |  |  |  |
| 13 | reserved |  |  |  |  |  |  |  |
| 14 | reserved |  |  |  |  |  |  |  |
| 15 | reserved |  |  |  |  |  |  |  |

The tag field is copied from the QUERY PORT SMS.  It identifies the QUERY PORT SMS for which this response is being generated.

The link erp error count field is the count of the number of times the link ERP has been invoked for the selected port since the last power on, total/absolute reset or the last QUERY PORT SMS with the CLE bit set.  A value of FFFFh is returned if the count is greater than or equal to 65535.

The alarm threshold field is the current setting of the alarm threshold for the selected port.  See 11.2.3.

Table x defines the supported speed field.

Table x - supported speed

| Supported speed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| reserved | reserved | reserved | reserved | reserved | reserved | reserved | reserved |
| reserved | reserved | reserved | reserved | reserved | reserved | 40 | 20 |

The supported speed field defines a bit vector of the the transfer raties supported by the port.  A bit shall be set for each speed that the node supports.

Table x defines the current speed field.

Table x - current speed

| current speed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| reserved | reserved | reserved | reserved | reserved | reserved | reserved | reserved |
| reserved | reserved | reserved | reserved | reserved | reserved | 40 | 20 |

The current speed field defines a bit vector that reports the node’s cuurent data transfer speed.  Only one speed shall be reported for the node.

The following is 9.5 of SSA-TL1 rev 6 modified to include speed negotion.

## Beginning communication

A port is in the disabled state and privileged mode after the POST, a Total/Absolute reset, a Local reset or when the link ERP exits unsuccessfully.  The port shall then use the following procedure to begin communication:

- Wait until the line fault detector is not active.

- Enter disabled state.

- Perform speed negotiation.

- If character synchronization is acheived enter the enabled state.

- When a flag character is decoded enter the ready state.

- Mark the port operational.

- Wait for at least 10 more character periods.

- If buffer space is available to receive a frame then send an RR Response and reset RR pending.  (See n.n.)

- Wait for  an RR response, then reset waiting for RR.

- For each QUERY NODE SMS received:

- a) if the DR bit is zero make an entry in the initiator table;

- b) return a QUERY NODE REPLY SMS.

- If the node is to be the master, (see n.n), then it should select normal mode internally.  Otherwise it should wait to receive a CONFIGURE PORT SMS specifying 'set normal mode'.

- Application and privileged frames can now be transmitted.

Speed negotiation

The transmitter sends characters at the highest speed supported.

The receiver attempts character synchronization.  Character synchronization is acheived when four consecutive comma characters are decoded and the speed negotiation is successfull.

If character synchronization is not acheived within 1 ms then the port switches the transmitter and reciever to the  next lower speed supported and attempts character synchronization again.

If character synchronization is not acheived at the lowest supported speed the speed negotion process has ended.  However the port remains in the disabled state and continues to transmit at the lowest speed.