Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a183R0
Date:	August 27, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 6a
Reply to:	Lawrence J. Lamers

To: 	   	X3T10.1 Membership
From:	Lawrence J. Lamers

Subject: Proposal to fix length of QUERY PROTOCOL REPLY

BACKGROUND

QUERY PROTOCOL REPLY is one of two SMSs that do not have a defined length.  This needlessly increases the difficulty for software and hardware.  The QUERY NODE REPLY SMS is corrected with the QUERY SWITCH SMS proposal

PROPOSAL

Table 1 - QUERY PROTOCOL REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (09h) |  |  |  |  |  |  |  |
| 1 | length |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | protocol list |  |  |  |  |  |  |  |
|  | protocol list |  |  |  |  |  |  |  |
| 15 | protocol list |  |  |  |  |  |  |  |

The length field indicates the number of valid bytes in protocol list and has a value in the range of 0 to 28.

The tag field is copied from the QUERY PROTOCOL SMS.  It identifies the QUERY PROTOCOL SMS for which this reply is being generated.

- The protocol list field contains one entry for each upper-level protocol that the node can receive.  Upper-level protocols are identified by ULP code values as defined in n.n.  Only values that may be returned by QUERY PROTOCOL REPLY may be used.