Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a179r1
Date:	November 1, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 7b 
Reply to:	Lawrence J. Lamers

To:  	X3T10.1 Membership
From:	Lawrence J. Lamers

Subject: Proposed Changes to SSA-TL2 for 40 MB/sec

The following beginning communication process has been modified to include speed negotion.

## Beginning communication

A port is in the Disabled state and Privileged mode after the POST, a Total or Absolute Reset frame, a Local Reset or when the Link ERP process exits with an alert code value.  The port shall then use the following procedure to begin communication.

- If the line fault detector indicates a line fault, then wait until there is no line fault.  A line fault indicates that the remote node is not powered-on or that the cable is disconnected.

- Transmit at least 200 DIS characters.

- Perform speed negotiation.

- Put the port into the Enabled state.

- Wait for the port to enter the Ready state, then set the port’s operational flag.

- Wait for at least 10 more character periods.

- Ensure that buffer space is available to receive at least 1 frame.  Then send an RR character pair and clear the rr pending flag (see Error: Reference source not found.)

- Wait to receive an RR character pair from the remote port.  Then clear the waiting for rr flag.

Privileged frames can now be received and transmitted.

Speed negotiation

The transmitter sends characters at the highest speed supported.

The receiver attempts character synchronization.  Character synchronization is acheived when four consecutive comma characters are decoded and the speed negotiation is successfull.

If character synchronization is not acheived within 1 ms then the port switches the transmitter and reciever to the  next lower speed supported and attempts character synchronization again.

If character synchronization is not acheived at the lowest supported speed the speed negotion process has ended.  However the port remains in the disabled state and continues to transmit at the lowest speed.