Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a182r5
Date:	November 8, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1
Reply to:	Larry Lamers

To: 	   	X3T10.1 Membership
From:	Larry Lamers

Subject: QUERY SWITCH SMS

BACKGROUND

This proposal corrects the existing limitations in the port mask allowing the architectural limit of 128 ports to be reported.

The November 1 1995 X3T10.1 plenary changed the total ports field to total other ports and added descriptive text.

PROPOSAL

Make the following changes

- Modify 11.2.6 (QUERY NODE REPLY SMS) as defined in SSA-TL!-rev 7

- Add after 11.2.10 the following two clauses, 11.2.11   QUERY SWITCH SMS and 11.2.12   QUERY SWITCH REPLY SMS as defined in the following pages.

Sincerely,

Lawrence J. Lamers
ljlamers@aol.com

- 11.2.6   QUERY NODE REPLY SMS

The QUERY NODE REPLY SMS as defined in Table 1 is returned when a QUERY NODE SMS is received.  The QUERY NODE REPLY SMS is returned on the same port that received the corresponding QUERY NODE SMS.

Table 1 - QUERY NODE REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (01h) |  |  |  |  |  |  |  |
| 1 | port |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | upper level protocol |  |  |  |  |  |  |  |
| 5 | itf | master priority |  |  | reserved |  |  |  |
| 6 | total other ports |  |  |  |  |  |  |  |
| 7 | ssa-tl1 version (02h) |  |  |  |  |  |  |  |
| 8 | unique id |  |  |  |  |  |  |  |
| 9 | unique id |  |  |  |  |  |  |  |
| 10 | unique id |  |  |  |  |  |  |  |
| 11 | unique id |  |  |  |  |  |  |  |
| 12 | unique id |  |  |  |  |  |  |  |
| 13 | unique id |  |  |  |  |  |  |  |
| 14 | unique id |  |  |  |  |  |  |  |
| 15 | unique id |  |  |  |  |  |  |  |
| 16 | initiator id |  |  |  |  |  |  |  |
| 17 | initiator id |  |  |  |  |  |  |  |
| 18 | initiator id |  |  |  |  |  |  |  |
| 19 | initiator id |  |  |  |  |  |  |  |
| 20 | p1o | p2o | reserved |  |  |  |  |  |

The port field indicates the number of the port currently being used.

The tag field is copied from the QUERY NODE SMS.  It identifies the QUERY NODE SMS for which this reply is being generated.

The upper level protocol field identifies the upper-level protocol to communicate with the node.  It shall contain a value from Table 2 that may be returned by QUERY NODE REPLY SMS.

Table 2 - Upper-level protocol code values

| ULP code | Protocol | Notes |
| --- | --- | --- |
| 00h | Use the QUERY PROTOCOL SMS to list upper level protocols supported | 1 |
| 01h | No upper-level protocol supported | 1 |
| 02h | Vendor specific | 1,2 |
| 03h-7Fh | reserved |  |
| 80h | SSA-IA/95SP | 1,2 |
| 81h | reserved |  |
| 82h | SSA S2P | 1,2 |
| 83h | SSA S3P | 1,2 |
| 84h-FAh | reserved |  |
| FBh-FFh | Vendor specific | 2 |
| Notes 1) May be returned in QUERY NODE REPLY. 2) May be returned in QUERY PROTOCOL REPLY |  |  |

The Initiator table full (itf) bit is set when there is no space left in the Initiator table to make an entry for the initiator that sent QUERY NODE SMS.

The Master priority field defines the priority of the node for becoming the Web Master.  A value of zero indicates that the node is not capable of functioning as a Master.  Any non-zero value indicates the node’s priority for becoming the Master.  A value of one is the lowest priority and seven the highest

Implementer’s note - A initiator may optionally fix its Master priority at 4 (the default priority) or provide some mechanism outside of SSA to change its Master priority dynamically.

The total other ports field contains a value that is one less than the number of ports implemented.  If this value exceeds two, the QUERY SWITCH SMS should be used to retrieve a port mask for the switch.

Implementer’s note: The total other ports field value of must be 0 (single port), 1 (dual port) or an odd number greater than 1 (switch) as per clause 8.2.

The ssa-tl1 field defined in Table xx identifies the version of SSA-TL1 being used by the sender.

The unique id field contains the node’s Unique ID.

The initiator id contains a value created by the target and returned to the initiator.  This field shall be used by the initiator in any future application SMSs which are utilizing the same path as used by the QUERY NODE SMS which caused this QUERY NODE REPLY SMS is be generated.

The p1o bit is only valid for single port and dual port nodes, Switch nodes shall clear the p1o bit.  For single port or dual port nodes, the p1o bit is set if port 1 is operational, and is cleared if port 1 is not operational.

The p2o bit is only valid for single port and dual port nodes, Switch nodes shall clear the p2o bit.  For single port or dual port nodes, the p2o bit is set if port 2 is operational, and is cleared if port 2 is not operational.

The Path component of the address field in a QUERY NODE REPLY SMS frame is copied from the return path field in the corresponding QUERY NODE SMS.  All padding bytes shall be discarded.

- 11.2.11   QUERY SWITCH SMS

The QUERY SWITCH SMS is sent from a Master Capable node to each switch operational node during the configuration process to determine which ports are operational.

The QUERY SWITCH SMS is defined in Table 3.

Table 3 - QUERY SWITCH SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (0Ch) |  |  |  |  |  |  |  |
| 1 | reserved |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | return path |  |  |  |  |  |  |  |
| 5 | return path |  |  |  |  |  |  |  |
| 6 | return path |  |  |  |  |  |  |  |
| 7 | return path |  |  |  |  |  |  |  |

The tag field is returned in the QUERY SWITCH REPLY SMS

The return path field specifies the Path component that shall be placed in the Path component of the address field of the resulting QUERY SWITCH REPLY SMS

The destination node normally returns a QUERY SWITCH REPLY SMS.  If a field is invalid then the destination node returns a RESPONSE SMS with a return code field of INVALID FIELD instead of QUERY SWITCH REPLY SMS.

- 11.2.12   QUERY SWITCH REPLY SMS

The QUERY SWITCH REPLY SMS as defined in Table 4 is returned when a QUERY SWITCH SMS is received.  The QUERY SWITCH REPLY SMS is returned on the same port that received the corresponding QUERY SWITCH SMS.

Table 4 - QUERY SWITCH REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (0Dh) |  |  |  |  |  |  |  |
| 1 | port |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | port mask |  |  |  |  |  |  |  |
|  | port mask |  |  |  |  |  |  |  |
| 23 | port mask |  |  |  |  |  |  |  |

The port field indicates the number of the port currently being used.

The tag field is copied from the QUERY SWITCH SMS.  It identifies the QUERY SWITCH SMS for which this reply is being generated.

The port mask field is a bit vector, where a bit that is set indicates that the port is operational.  Byte 4, bit 7 corresponds to port 1, byte 23, bit 0 corresponds to port 128.