Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a183r1
Date:	October 24, 1995
Project:	X3T10.1 / 0989D
Ref Doc.:	SSA-TL1 rev 7
Reply to:	Larry Lamers

To: 	   	X3T10.1 Membership
From:	Larry Lamers

Subject: Proposal to fix length of QUERY PROTOCOL REPLY SMS

BACKGROUND

QUERY PROTOCOL REPLY is one of two SMS that do not have a defined length.  This needlessly increased the difficuly of software and hardware.  The other variable length sms, the QUERY NODE REPLY SMS, is corrected with the QUERY SWITCH SMS proposal.

PROPOSAL

Change the text of the QUERY PROTOCOL REPLY SMS from SSA-TL1 Rev 7 to Rev 7b as shown on the next page.

Sincerely,

Lawence J. Lamers
ljlamers@aol.com

- 11.2.10   QUERY PROTOCOL REPLY SMS

The QUERY PROTOCOL REPLY SMS defined in ble 1 shall be returned a QUERY PROTOCOL SMS is received.  The QUERY PROTOCOL REPLY SMS shall be returned via the same port that received the corresponding QUERY PROTOCOL.  It indicates the particular upper-level protocols that the node can receive.

Table 1 - QUERY PROTOCOL REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (09h) |  |  |  |  |  |  |  |
| 1 | reserved |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | protocol list |  |  |  |  |  |  |  |
|  | protocol list |  |  |  |  |  |  |  |
| 15 | protocol list |  |  |  |  |  |  |  |

The tag field is copied from the QUERY PROTOCOL SMS.  It identifies the QUERY PROTOCOL SMS for which this reply is being generated.

The protocol list field contains one entry for each upper-level protocol that the node can receive.  Upper-level protocols are identified by ULP code values as defined in Table 34.  Only values that may be returned by QUERY PROTOCOL REPLY shall be used.  The protocol list field is always 11 bytes in length and is zero filled and left justified (first byte is in byte 4 of the  QUERY PROTOCOL REPLY SMS).

The entry for a standard protocol is a one-byte ULP code value.

The entry for a vendor-unique protocol consists of a one byte ULP code value followed by a 4-byte protocol  identifier.  The ULP code value for each protocol shall be unique.  The first 3 bytes of the protocol  identifier identify the original vendor of the protocol in the same way as in a Unique ID.  The remaining byte is assigned by the original vendor to the particular protocol.

For example, a target

which can receive SCSI-2 and two vendor-unique protocols may return the following 11 bytes in Protocol list:

82h				/* SSA-S2P */
FBh 444543h 01h		/* vendor A protocol*/
FCh 49424Dh 03h		/* vendor B protocol*/

The protocol identifier is based on the OUI (see 9.2.1). The first three bytes are an OUI, the last byte is administered by the OUI assignee. For example, ACDE.4801h might be the first protocol identified by the AC-DE-48 OUI assignee.