Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a194r2
Date:	October 31, 1995
Project:	X3T10.1 / 0989
Ref Doc.:	SSA-TL1 rev 7
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Assumptions made in SSA-TL1 rev 7x

BACKGROUND

During the editing of SSA-TL1, I came across several areas that needed clarification or needed to be consistent.  This proposal documents the decisions/actions, and raises some points for discussion.

The proposal is broken into two sections.  The first is clarifications that need discussion.  The second is clarification which just make sections consistent and do not need discussion (in my view).

PROPOSAL (Needs discussion)

- (10.4.10 in rev 7b) What happens when a Master receives an ASYNC ALERT SMS with a MASTER GENERATED ASYNC ALERT type field value?  I said (already in rev 7b)...
“The MASTER GENERATED ASYNC ALERT type field value shall only be generated by the Master, and therefore the Master should never receive an ASYNC ALERT SMS with a Master generated alert type field.  If the Master receives an ASYNC ALERT SMS with a Master generated alert type field, then the Master generates an ASYNC REPLY SMS with a return code field value of INVALID FIELD, logs the error, and does not send a MASTER ALERT SMS to the other initiators.”

- (12.5, bullet 6 in rev 7b) What so we do in an Master capable node does not respond to the Master Alert during Asynchronous Alert handling? (already in rev 7b)
I believe if there is no RESPONSE SMS to MASTER ALERT SMS then simply performs a Web Reset process.  This may seem drastic but in this case the problem is usually not in the initiator that is addressed by the MASTER ALERT SMS; it is more likely to be a communication block in the Web (e.g. another node may be hung and not processing its inbound buffers).  This is precisely the situation for which the Web Reset process was intended;  A total reset frame is unlikely to be effective because there is no way to determine which node is at fault.
I propose changing section 12.5 bullet 6 from...
“6)   Wait for RESPONSE SMS from each of the initiators for the MASTER ALERT SMS.”
to...
“6) Wait for a RESPONSE SMS from each of the Initiators for the MASTER ALERT SMS.  If any initiator fails to return a RESPONSE SMS associated with the MASTER ALERT SMS within 1 second, the Web Reset process shall be invoked (see 9.5.6).”

- (11.2.13 in rev 7c) Rename 3-party bit in the QUIESCE SMS to the rite bit (Remove Initiator table entry).  Replace the 3 party bit definition with the following definition and implemente’s note:
“When the rite bit (remove initiator table entry) is set, the node shall remove the Initiator table entry associated with the unique id field.  When the rite bit is cleared, the node shall delete all the return paths in the Initiator table entry associated with unique id field.”
	Implementer’s note: The rite bit should only be set by an initiator quiescing it’s own I/O processes, 	by the Master or by the resigning Master node.

- (11.1.3 in rev 7b) The return path field should not be validated (as registered) for the CONFIGURE PORT SMS.  The reason is that for loops, the alert path must be set for each port.  If the registration of the return path is checked, the initiator must register both paths.
The return path field should not be validated (as registered) for the QUERY PROTOCOL SMS.  The reason is that the initiator may wish to know if it can talk the same language as the node before registering.
The return path field should not be validated (as registered) for a QUIESCE SMS.  The initiator was using one path that has broken, so it issues a QUIESCE SMS down the alternate path that it may not be registered on.  By checking the registration, we are forcing a QUERY NODE SMS/QUERY NODE REPLY SMS pair just to quiesce.  It is cleaner to remove the restriction.
I propose that we do not check the return path field as being registered for any of the transport layer privileged messages.  The following transport layer messages use return path...
CONFIGURE PORT - Should not check for the reasons stated above
MASTER ALERT - Since we have a problem, perhaps we should not check.  An SMS from someone “strange” could cause extra errors logged or an extra configuration process.  Ho hum.
QUERY NODE - We do not check now since this is the SMS that registers.
QUERY PORT - The link erp counter could cleared by an unauthorized person.  Ho hum.
QUERY PROTOCOL - We should not check for the reasons above.
QUIESCE - We should not check for the reasons above.

I propose we change 11.1.3 first paragraph and remove number 1) from...
“The fields of an SMS shall be validated with the exception of  those in the QUERY NODE SMS, QUERY NODE REPLY SMS, CONFIGURE PORT SMS, and the response SMS associated with the CONFIGURE PORT SMS.  If an Asynchronous Alert is generated then the message is then terminated, without any other response
1)   If the return path or initiator id field is unknown, then generate an ASYNC ALERT SMS with an Unknown Return path or INITIATOR id meaning code, with two exceptions.  The QUERY NODE accepts any return path, since it builds the Initiator table.  The CONFIGURE PORT SMS responds with a RESPONSE SMS with a return code of UNKNOWN RETURN PATH for an unknown return path, since an alert path may not be accepted yet for the ASYNC ALERT SMS.”
to...
“The fields of an SMS shall be validated as described below.  If the Asynchronous Alert process is invoked then the message is terminated, without any other response.  The return code field of Privileged SSA-TL1 SMSs need not be registered in the Initiator table.”

- (Clause 6 (Web) in rev 7b) Clarify the definition of a node as follows: replace
”SSA-TL1 distinguishes three types of nodes, according to the connectivity: single port node; dual port node; switch node.”
with
“A node is identified in several ways.
     a)	The node is “Master Capable” or not.  All initiators must be Master Capable, while targets need 	not be.
     b)	The node may have one port (“Single ported”), two ports (“Dual ported”), or more that two 	ports (“Switch”).

- (global rev 7c)  QUERY NODE REPLY SMS - Change initiator id to return path id (same as SSA-S2P) since the terms Initiator ID and Unique ID were confusing (it is a path ID, not an Initiator ID).

- (9.1.3.2 in rev 7b) The Disabled state states that the rr pending flag and waiting for rr flag are set and the ack pending flag and waiting for ack flag are cleared in the Disabled state.  The Acknowledgments and Pacing sections state they are initialized in the Ready state.
I made them all initialized in the Disabled state. (already in rev 7b)

- (9.5.3 in rev 7b) In the Total Reset or Absolute Reset sections, the text said (already in 7c)...
        a)	A transmitting port shall send Total or Absolute resets while in the Ready states.  It 	should be restored to the Enabled or Ready states.
        b)	A receiving port shall recognize Total or Absolute Reset frames in “any state”.  Clearly 	impossible in the Inactive state and should be “any state but the Inactive state”.
        c)	If the output port is in the Disabled or Check states, a Total or Absolute Reset frame shall be 	held until it enters the Enabled or Ready states.  This is correct.

Rejected proposals

The following proposals are withdrawn due to lack of consensus.

- Does QUERY NODE SMS with the dr bit set remove the entries from the Initiator table?
I propose to change the third sentence under the dr bit definition paragraph from:
“appropriate If the dr bit is set, the Initiator table shall not be updated”. 
to
“If the dr bit is set, the appropriate elements are deleted from the port array, return path array, and initiator id array fields of the Initiator table entry associated with the unique id field value.”

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com