Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a195r2
Date:	November 8, 1995
Project:	x3t10.1/0989-D
Ref Doc.:	SSA-TL1 rev 7 
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Link Reset Handling process

BACKGROUND

The Link ERP process defines what happens when a link detects an error and sends a Link Reset frame to the remote port.  Nowhere is it described what happens when a Link Reset frame is received.  Although it may be extracted from the Link ERP process, this involves memory of which condition caused the entry point and ignores may of the steps.  Even though it duplicates information, I feel the following proposal makes it clearer.

At the direction of X3T10.1 (11/1/95), I have removed the Link Reset Handling process and restored the Link ERP handling and Link Reset Handling into a common process with the addition of a link reset received flag.

PROPOSAL

- Add the link reset received flag section under port attributes as defined on the following page.

- Add the following to the Disabled state description”
“The link reset received flag is cleared in the disabled state.”

- Replace the third paragraph under 10.1 with the following
“The port that detects the Link Error invokes the Link ERP process.  During the Link ERP process, a Link Reset frame will be sent to the remote node, which will trigger the remote node to invoke the Link ERP process (if it did not also detect an error and already invoked the Link ERP process).  The Link ERP process coordinates the actions of the two ports and remains synchronized.”

- Replace the Link ERP process definition section with the section on the following page.

- Remove the Link Reset Handling process section.

- 9.1.8½   link reset received flag

The link reset received flag is used during the Link ERP process to determine if the process was entered due to a link error (cleared) or due to receiving a Link Reset frame (set).  The link reset received flag is cleared whenever the port enters the disabled state.  The link reset received flag is set whenever the port receives a Link Reset frame and is not in the Disabled state.  The link reset received flag is tested during the Link ERP process.

- 10.1.3   Link ERP process definition

To facilitate cross-referencing, the Link ERP exits are identified by an alert code value that corresponds to the alert code field used by the Asynchronous Alert process.

The Link ERP process may not be invoked from within itself (i.e. it is non-recursive).  The implementation shall protect against the Link ERP process looping if there is a permanent error.

Implementer’s note - The following is an example of one method that can be used to protect against the Link ERP process from looping.  Each invocation of the Link ERP process increments a retry counter that is reset to zero periodically by a timer.  If the number of retries in one period of the timer exceeds a maximum value then the Link ERP process waits 25 ms to ensure that the remote port recognizes an unrecoverable error, it places the port in the Disabled state, and it exits with an alert code value of retry limit exceeded.  This example also would protect against excessive use of the ERP in the event of severe external noise.

The port that detects the error enters the Check state and invokes its Link ERP process.  Any Link ERP exit shall terminate the Link ERP process and invoke the Asynchronous Alert process as defined in 12.5.

- The Link Status Byte is built.  If the port detects a line fault that persists for more than 1 ms the port enters the Disabled state, and Link ERP process exits with an alert code value of Permanent line fault.

- If the port is not receiving characters, the port enters the Disabled state, and the Link ERP process exits with an alert code value of No characters received.

- If the port is detecting DIS characters the port enters the Disabled state and the Link ERP process exits with an alert code value of Remote port disabled.

- A Link Reset frame is transmitted.  If no acknowledgment is received the Link Reset frame is retransimitted once.  If no acknowledgment is received to the retransmitted Link Reset frame, then the Link ERP process waits 25 ms to ensure that the remote port recognizes an unrecoverable error, sets the port to the Disabled state, and exits with an alert code value of Link reset failed.

- If the link reset received flag is cleared and a Link Reset frame has not been received within 5 ms, the Link ERP process waits 25 ms to ensure that the remote port recognizes an unrecoverable error, places the port to the Disabled state, and exits with an alert code value of Link reset failed.

- If a hardware error is detected, the Link ERP process sets the port to the Disabled state and exits with an alert code value of Hardware error.

- If FRAME REJECT ERROR is indicated by the port’s Link Status Byte, the Link ERP process sets the port to the Disabled state, and exits with an alert code value of Frame reject.

- The number of outbound frames for which an acknowledgment is outstanding is calculated.  When using strict cyclic buffers this can be obtained as follows:

- Q=(transmit pointer - retry pointer) mod N

- where N is the number of transmit buffers provided.  Q should be 0 or 1 frames.

- The number of outbound frames that the local port is expecting an acknowledgment for but which have not been received by the remote port is also calculated, where local refers to the ports transmit sequence number and remote refers to the rsn field received in the Link Status Byte.

- P=(local transmit sequence number  - remote rsn)mod 4

- where P should be less than or equal to Q.

- If either Q > 1 or P > Q then the Link ERP process waits 25 ms to ensure that the remote port recognizes an unrecoverable error, sets the port to the Disabled state and it exits with an alert code value of Invalid retry status.

- The Link ERP process arranges to resend the number of frames indicated by subtracting P from its transmit pointer, modulo N.  Those outbound buffers that do not need to be retransmitted shall be discarded using the following algorithm.

- Do while retry pointer ¹ transmit pointer;
		Discard buffer at retry pointer;
		Increment retry pointer modulo N;
	End;

- The frame buffers and transmit pointer are set to retransmit the proper number of frames after the Link ERP re-establishes communication following the port being set to the Disabled state.

- If the port has received a frame containing any of the errors listed in Table 18, then the appropriate inbound buffer contents shall be discarded.  The ERP places the port in the Disabled state.  The ERP waits until the receiver detects DIS characters.  This is required to synchronize the local and remote ports Link ERP process's and prevent the transmitter sending RR while the remote port is still in the Check state.

- If the receiver does not detect DIS characters within 5 ms after the local port is disabled then the ERP exits with an alert code value of Time-out waiting for Disabled state.

- The port is set to the Enabled state.

- If the receiver detects a FLAG character within 5 ms of placing the port in the Enabled state, the port is set to the Ready state, otherwise the Link ERP process exits with an meaning code of Time-out waiting for Ready state.

- The ERP terminates successfully.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com