Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a198R2
Date:	July 28, 2026
Project:	0989-D
Ref Doc.:	X3T10.1/0989 rev 07
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible

Subject: Configuration process

BACKGROUND

A configuration process is needed to complete the transport layer document.

PROPOSAL

  - Replace clause 9.4 in SSA-TL1 rev 7 with the following page.

  - Make the following modifications to Beginning Communications (9.3)

    - Item 8) should be moved into the Configuration process.

    - Item 9) should be modified to read “ Privileged frames can now be received or transmitted.”

Sincerely,

Lawence J. Lamers
ljlamers@aol.com

- 9.4   Configuration

Each initiator shall invoke a Configuration process to determine the other nodes that are present and their path address(es).

- 9.4.1   Configuration Process

The Configuration Process involves the following steps, where a Master Capable node is a node (initiator or target) that is capable of being a Master.

- Each Master Capable node walks the web (see 9.4.2) to build a Topology table (see 9.2.3).

- Each Master Capable node examines the Topology table to determine the Master.  A Master is defined as the node with the largest value reported in the Master priority field.  In the event that more than one node has the same value for the Master priority field the node with the largest Unique ID is the Master.

- Each Master Capable node constructs a Configuration table from the information in the Topology table.

- Each Master Capable node registers with the nodes that it expects to perform I/O process with.  If a node supports multiple protocols a QUERY PROTOCOL SMS is issued to determine the protocols supported.

- If the node is to be the Master, (see xxx), then it should place the port in Normal mode internally.

- The Master sets its ports to Normal mode and transmits a CONFIGURE PORT SMS for each port to the nodes in the Topology table.  A CONFIGURE PORT SMS is required for each port whether it is operational or not.

- The Master sends a MASTER ALERT SMS with an alert code value of BOTH PORTS OF A LINK ARE IN NORMAL MODE for each link that has both ports in Normal mode.

- 9.4.2   Web Walking Process

The Web Walking process is defined in the following steps.

  - The Master Capable node selects a port on the base node.  The Web walking process is repeated for each operational port on the base node.

  - The Master Capable node sends a QUERY NODE SMS to the destination node incrementing the path address by one until a QUERY NODE REPLY is received reporting:

    - only one port operational indicating the end of a string has been reached;

    - a Unique ID that already exists in the Topology table indicating a loop has been established;

    - the total other ports field value exceeds two indicating that a switch has been discovered.

  - The Master Capable node sends a QUERY PORT SMS to each port reported operational to determine is speed.

  - If a switch is discovered the Master Capable node sends a QUERY SWITCH SMS to determine the number of operational ports on the switch.  The base node is set to that switch node and the Web Walking process is repeated.