Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a199r1
Date:	October 31, 1995
Project:	X3T10.1/0989D
Ref Doc.:	SSA TL1 rev 7
Reply to:	John Scheible

To: 	   	X3T10.1 Membership
From:	John Scheible and Mark DeWilde

Subject: Master Alive process

BACKGROUND

The August X3T10.1 Working group minutes (95a188R0) documented the following agreement:
“The recommended timer value is 10 seconds for an initiator to detect a QUERY NODE from the master and 5 seconds for a master to detect a response from the initiator, (this implies but does not mandate a maximum Node delay of 5 ms for processing a frame in a Node for a maximum number of  hops which is 505). In addition, it was decided to add a 'Master is Alive' bit to the Query Node SMS that will reset the timer in any Initiator when used in the Healthy Web process.”

I have attempted to take this somewhat inexact directive, and have implemented the following proposal.

PROPOSAL

I propose that the following changes be made to SSA-TL1 rev 7 in accordance to the imprecise directions at the X3T10.1 August 1995 plenary.

- Add a ma bit to bit 6 word 16 of the QUERY NODE SMS.  The description of the bit is as follows:
“The ma bit (Master Alive) is set during the Master Alive process (see x), and is cleared all other times.  When a node receives a QUERY NODE SMS with the ma bit set, it resets the Master Alive timer (see 9.2.7.2).”

- Modify “10.6   Target not responding” in rev 7 to “10.6   Master Alive process” in rev 7c as shown in the following page.

- 10.6   Master Alive process

- The Master sends a QUERY NODE SMS with the ma bit set to Master Capable node with Master Capable node with the next lower Unique ID every 3 seconds.  If the Master is the lowest Unique ID, it sends a QUERY NODE SMS with the ma bit set to the Master Capable node with the highest Unique ID.

- When a Master Capable node, which is not the Master, receives a QUERY NODE SMS with the ma bit set, it sends a QUERY NODE SMS with the ma bit set to the Master Capable node with the next lowest Unique ID.  If no lower Unique ID exists, it sends a QUERY NODE SMS with the ma bit set to the Master Capable node with the highest Unique ID.

- If 5 seconds elapses between receipt of QUERY NODE SMSs with the ma bit set, the Master Capable node sends a QUERY NODE SMS with the ma bit set to the Master.

- If neither the QUERY NODE REPLY SMS to the QUERY NODE SMS with the ma bit set is sent to the Master nor the QUERY NODE SMS with the ma bit set is received within 5 seconds the Master Capable node invokes the Configuration process.

Sincerely,

John Scheible
Voice:	(512) 823-8208
FAX: 	(512) 823-0758
Email:	Scheible@vnet.ibm.com

Mark DeWilde
Email:	mark@pathlight.com