Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a201R0
Date:	October 17, 1995
Project:	
Ref Doc.:	
Reply to:	Charles Monia

To: 	   	X3T10.1 Membership
From:	Charles Monia

Subject: S2P Queue Full Extension Proposal

BACKGROUND

The Queue Full interlock was added to prevent out of order commands when the target had no space to store an incoming SCSI Command SMS. In addition to commands terminated with QUEUE FULL status, this mechanism must also handle all other transient command failures. In SCSI-2, this includes commands terminated with RESERVATION CONFLICT and BUSY status.

PROPOSAL

Extend the Queue Full interlock to handle commands terminated with RESERVATION CONFLICT or BUSY status. Rename the interlock to “Queue Blocked” and modify the S2P draft standard to reflect the name change. Add the following description as subclause 7.8:

Begin new subclause 7.8

====================================

7.8  Queue Blocked Condition

A queue shall enter the blocked condition with respect to an I_T_L nexus when an SCSI COMMAND SMS is received for that nexus and one of the following occurs:

- A condition exists which would cause the device server to complete the command with a status of BUSY, queue full  or RESERVATION CONFLICT.

- The target’s S2P layer has no room to receive the incoming SMS.

Upon setting the Queue Blocked condition, the node shall discard the SCSI COMMAND SMS. and generate an SCSI STATUS SMS containing the appropriate status.  If termination was caused by no room to receive the SMS, a status of QUEUE FULL shall be returned. Subsequent SCSI COMMAND SMSs from that initiator shall be discarded if the command is directed to the blocked queue and the resume bit is cleared (see 7.6).

The node shall clear the Queue Blocked condition upon executing:

- A Total or Absolute Reset;

- A DEVICE RESET SMS;

- An SCSI Command SMS with the resume bit set (see 7.6), if directed to the blocked queue and sent by the initiator causing the Queue Blocked condition .

After acting upon the resume bit as specified in (c), the node shall reestablish the Queue Blocked  condition if one of the blocking conditions described above is detected.

When the Initiator’s S2P layer receives an SCSI STATUS SMS indicating QUEUE FULL, RESERVATION CONFLICT or BUSY status, the Initiator shall mark the command identified by the SMS and any subsequent commands directed to the queue as having been discarded.  The Initiator’s S2P layer shall send the next SCSI COMMAND SMS directed to that queue with the resume bit set (see 9.3.1).”

================================

End new subclause 7.8

Sincerely,

Charles Monia
Voice:	(508) 841-6757
FAX: 	(508) 841-6100
Email:	monia@shr.dec.com

Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a201R0
Date:	October 17, 1995
Project:	
Ref Doc.:	
Reply to:	Charles Monia

To: 	   	X3T10.1 Membership
From:	Charles Monia

Subject: S2P Queue Full Extension Proposal

BACKGROUND

The SCSI-2 logical model as well as existing applications assume that any device may act both as initiator and target. In SSA-S2P, the protocol for determining the roles supported by an SSA initiator is not fully defined.

An initiator determines if a device implements SCSI target functions by evaluating the response to a required SCSI command, such as INQUIRY, directed to LUN 0. An SSA initiator that is capable of executing SCSI commands will respond by returning the expected data and SCSI status. The response of an SSA initiator that does not support target functions is unspecified.

PROPOSAL

Modify the S2P protocol to specify the reply to an SCSI COMMAND or task management SMS received by a device that does not support SCSI target operations. Such a device shall return an SCSI Response SMS with return code set to SCSI TARGET OPERATIONS NOT IMPLEMENTED.

Specific changes:

- Modify table 15 in subclause 7.14 to add a new return code value of 21h (SCSI TARGET OPERATIONS NOT IMPLEMENTED).

- Add the following text to subclause 7.2:

“For each scsi application sms in table 4 sent by an scsi initiator to an scsi  target, the S2P layer of a receiving node that does not implement scsi command and task management functions shall return an scsi response sms with a return code of scsi target operations not IMPLEMENTED (see 7.14).”

Sincerely,

Charles Monia
Voice:	(508) 841-6757
FAX: 	(508) 841-6100
Email:	monia@shr.dec.com

Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a201R0
Date:	October 17, 1995
Project:	
Ref Doc.:	
Reply to:	Charles Monia

To: 	   	X3T10.1 Membership
From:	Charles Monia

Subject: S2P Queue Full Extension Proposal

BACKGROUND

Existing host implementations have certain dependancies on characteristics of an SCSI interlocked protocol which don’t exist for a non-interlocked bus, such as SSA. The main difference is that, in SSA, the states of the initiator and target are no longer always synchronized. For example, the SMS propagation delay from the target to the initiator causes a time skew between the target’s detection of a BUSY condition and the initiator’s awareness of that condition. As a result, the initiator may have several commands in transit when the condition is detected.

For compatibility with SCSI-2 host implementations, the S2P layer must conceal the effects of these residual commands from higher layers.

PROPOSAL

- Add the subclause below and related editorial changes, specifying the rules for interlocked protocol emulation.

- Modify the SCSI STATUS SMS to contain a copy of the qerr flag from mode page 0Ah in bit 2 of SMS byte 7. As described below, this flag determines how the logical unit handles pending commands when a Contingent Allegiance condition occurs.

Begin new subclause

=========================

## Interlocked Protocol Emulation

To be compatible with existing SCSI-2 application layers, the protocol must provide some way for the initiator’s S2P layer to emulate the behavior of an interlocked bus. For a bus of this type:

- No more than one unit of information can be in transit from the initiator to the target at any time and,

- Since the physical bus protocol interlocks all data transfers, including those associated with a state change, the interlock is used to synchronize the states of the initiator and target.

In parallel SCSI, for example,  transfers are interlocked through bus arbitration and the req-ack handshake used to transfer data. When a check condition status is returned, the target and initiator can use the interlock to establish the Contingent Allegiance condition in lock step.

In SSA, as with other “loosely coupled” interconnects, the time for information to propagate through the network precludes the use of an interlocked physical bus protocol. In this case, if the bus is to be used efficiently, several frames must be allowed in transit simultaneously. In such protocols, state synchronization is lost whenever an event occurs at the target, such as a check condition, which alters device state before the initiator is aware of it. Without intervention, the resulting disruption in the order of command execution may cause data corruption and loss of sense data. With an interlocked protocol, such intervention is the responsibility of the initiator. Since the transfer of information over an SSA bus is not interlocked, such intervention must be the target’s responsibility. This is accomplished in SSA using mechanisms by which the target can:

- Notify the initiator of the exception,

- Resynchronize with the initiator by flushing commands for the I_T_L nexus that were in-transit when the error occurred and,

- When necessary, preserve sense information until the faulting initiator has had a chance to retrieve the data.

To emulate the behavior of an interlocked bus, the S2P layer must conceal the process of purging the command pipeline and resynchronizing the initiator and target. These procedures are described in 9.3.1 and 9.3.2.

### Emulating the Queue Full, Busy and Reservation Conflict conditions

An SCSI-2 application client whose design is based on the interlocked I/O bus characteristics described in 9.3 expects the following behavior:

- Once a command has been rejected due to a queue full, reservation conflict or busy status, no other queue full, reservation conflict or busy status shall be returned for any pending command for that I_T_L nexus. A pending command may be completed with a check condition status however.

- Depending on the implementation, commands for the I_T_L nexus that were sent after a command that terminated with queue full, busy or reservation conflict are either held unsent or deleted from the initiator’s S2P protocol layer. (In either case, to permit recovery, the initiator’s upper layer is expected to maintain it’s own ordered list of pending commands.)

On detecting a Queue Blocked condition, the initiator’s protocol layer shall:

- Return the appropriate status to the application client.

- On receiving the next SCSI command for that nexus from the application client, set the restore  flag in the corresponding scsi command sms and send the SMS to the target. The target will automatically flush previous commands still in-transit, since the reset bit will be clear in their scsi command smss.

- If pending commands for that I_T_L nexus were retained by the initiator’s S2P protocol layer, the protocol layer may resend all those that were issued after the command for which the Queue Blocked condition was reported. The mechanism for requesting the protocol layer to resend such commands is implementation-specific.

- If commands issued after the Queue Blocked command were not retained by the initiator’s S2P layer, the upper layer must resend such commands by referencing it’s own ordered list of pending commands.

### Contingent Allegiance Emulation

An SCSI-2 application client whose design is based on the interlocked I/O bus characteristics described in 9.3 expects the following behavior when a Contingent Allegiance occurs:

- Once a check condition status has been returned to the application client, no status for any pending command shall be returned for that I_T_L nexus.

- The status of commands for the I_T_L nexus sent after the command that terminated with check condition status is dependant on the state of the qerr  flag in mode page 0Ah when the condition occurred. If this bit was set, all such commands are aborted, if clear, the commands shall remain pending in the logical unit’s command queue.

On detecting a check condition status, the initiator’s S2P layer shall:

- Return the check condition status to the application client.

- Wait to receive an scsi status sms for each command sent to the I_T_L nexus after the command that terminated with check condition status. The status field shall have a value of sca active. As each status is received, the corresponding entry in the Outstanding Commands Table shall be marked. The status shall not be returned to the application client.

- If the qerr field in the scsi status sms  of step a) was set to one, the initiator’s S2P layer shall discard all pending commands for that I_T_L nexus, including those marked in b). All discarded commands, including those for which an abort tag, abort or clear queue were issued, shall be removed from the Oustanding Commands Table.

- The initiator’s S2P layer shall send the next command from the application client to the I_T_L nexus with the queue control field set to a value of serial contingent allegiance (see Table 6) followed by a clear sca sms.

- The initiator’s S2P layer shall resend all commands remaining in the Outstanding Commands Table that were marked in step b). All such marks shall then be cleared.

==========================================

End of new subclause

Sincerely,

Charles Monia
Voice:	(508) 841-6757
FAX: 	(508) 841-6100
Email:	monia@shr.dec.com

Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a201R0
Date:	October 17, 1995
Project:	
Ref Doc.:	
Reply to:	Charles Monia

To: 	   	X3T10.1 Membership
From:	Charles Monia

Subject: S2P Queue Full Extension Proposal

BACKGROUND

The specification incompletely defines the use of multiple paths for a given command. In order to stabilize document technical content, the use of multiple paths should be prohibited and all references to multi-path command processing deleted until the technical issues are resolved.

PROPOSAL

Modify the S2P draft standard to restrict all transfers between an initiator and target to a single path. The draft standard can be extended at a later date to support multiple transfer paths for a single command once a technical proposal has been reviewed and approved.

Specific Changes:

Add the following new clause:

=========================

6   Paths

In SSA-S2P, a path is a conduit for the ordered delivery of frames between an initiator-target pair (an I_T nexus) which is free of undetected errors or frame loss. A major advantage of SSA is the ability to improve throughput by configuring a web having multiple paths which can be used to transport frames without contention. As described in clause 7, a path is identified by the return path id parameter returned by a target in the QUERY NODE REPLY sms and includes the ports, links and intermediate nodes used to transfer a frame from the source to the destination.

Ordered frame delivery means that, for a given path, frames are received in the order they were sent. SSA, however, does not guarantee that frame order is preserved between paths. If, for example, the initiator sends a frame to the target  on path 1 followed by another frame to the same target on path 2, the frames may arrive in any order. For certain SCSI operations, in-order delivery is essential for data integrity. For instance, to insure that the final state of the recorded data is correct, the sequence in which a series of ordered commands is presented to a disk drive must be identical to the sequence in which they were issued by the application. For that reason, this version of the SSA-S2P standard uses a single path for all communications between an I_T nexus. This restriction may be eliminated in subsequent versions. Nevertheless, in a system with several initiators or targets, multiple paths may still be used to advantage.

==================================

End new clause

Add the following new subclause.

Begin new subclause

==================================

9.5  Alternative paths not allowed

Although the dual port and switch functions of SSA-TL allow for alternative paths between the initiator and target, all S2P SMSs transferred between a given I_T nexus shall be confined to a single path.  Different paths may, of course, be used for different I_T nexuses.

==================================

End new subclause

Sincerely,

Charles Monia
Voice:	(508) 841-6757
FAX: 	(508) 841-6100
Email:	monia@shr.dec.com

Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a201R0
Date:	October 17, 1995
Project:	
Ref Doc.:	
Reply to:	Charles Monia

To: 	   	X3T10.1 Membership
From:	Charles Monia

Subject: S2P Queue Full Extension Proposal

BACKGROUND

The present S2P definition for a path between an I_T nexus is incomplete from the viewpoint of the sender because it excludes the sending port.

PROPOSAL

Change the definition of “path” as indicated below:

Current definition:

=========================================

- path:   The links and intermediate nodes used to transfer a frame from the source to the destination.

=========================================

Proposed definition:

=========================================

- path:   The port, links and intermediate nodes used to transfer a frame from the source to the destination.

=========================================

Sincerely,

Charles Monia
Voice:	(508) 841-6757
FAX: 	(508) 841-6100
Email:	monia@shr.dec.com