Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a208R1
Date:	November 6, 1995	
Project:	0989D
Ref Doc.:	X3T10.1/0989D
Reply to:	Mark DeWilde

To: 	X3T10.1 Membership
From:	Mark DeWilde

Subject:   Unique ID, ULP code, SMS code in Master Alert

BACKGROUND

For the October SSA Week, two proposals were submitted and approved for inclusion into TL1.  Since the resolution of both affected the structure of the Master_Alert SMS, they have been combined into this single proposal.  The first, entitled “Unique ID in Master Alert”, document X3T10.1/95a208R0, dropped the alert path from the Master_Alert SMS and replaced it with Unique_ID.  The second, entitled “Use of S2P SCSI Status vs. Async Alert”, document X3T10.1/95a213R0, pointed out that the current use of async alert by ULP’s made Master Alerts virtually unusable by the transport layer.  Two solutions were suggested. The first prohibited ULP’s in general from using Async_Alert SMS’s for anything but transport layer problems. The second solution added the information to the Master_Alert SMS necessary to allow the initiator to determine which layer of the node protocol should receive the Master_Alert, and what command caused it.  The committee accepted solution 2.  This revised proposal summarizes the changes to the Master_Alert  SMS.

PROPOSAL

Alter Section 11.2.4 to read as follows:

11.2.4   MASTER ALERT SMS

The MASTER ALERT SMS defined in Table 30 is sent from the Master to each other initiator for the following three reasons.

- To forward an ASYNC ALERT SMS or an equivalent alert detected within the Master node itself.  If the alert is for a Link ERP process exit then the initiator quiesces any I/O processes that were using the affected port before returning a RESPONSE SMS to the Master.  The initiator invalidates the entries in the Configuration table for the affected nodes (see 11.2.1).

- To indicate that both ports of a link have been put into Normal mode (e.g., at power-on or following error recovery).  The initiator returns a RESPONSE SMS to the Master immediately.

- To inform other initiators that they need to invoke the Configuration process to reconfigure the Web.

Table 30 - MASTER ALERT SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code = 05h |  |  |  |  |  |  |  |
| 1 | reserved | port |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | return path |  |  |  |  |  |  |  |
| 5 | return path |  |  |  |  |  |  |  |
| 6 | return path |  |  |  |  |  |  |  |
| 7 | return path |  |  |  |  |  |  |  |
| 8 | Alert Unique_ID |  |  |  |  |  |  |  |
| 9 | Alert Unique_ID |  |  |  |  |  |  |  |
| 10 | Alert Unique_ID |  |  |  |  |  |  |  |
| 11 | Alert Unique_ID |  |  |  |  |  |  |  |
| 12 | Alert Unique_ID |  |  |  |  |  |  |  |
| 13 | Alert Unique_ID |  |  |  |  |  |  |  |
| 14 | Alert Unique_ID |  |  |  |  |  |  |  |
| 15 | Alert Unique_ID |  |  |  |  |  |  |  |
| 16 | type |  |  |  |  |  |  |  |
| 17 | subtype |  |  |  |  |  |  |  |
| 18 | type information |  |  |  |  |  |  |  |
| 19 | Reserved |  |  |  |  |  |  |  |
| 20 | Control |  |  |  |  |  |  |  |
| 21 | Channel |  |  |  |  |  |  |  |
| 22 | Channel |  |  |  |  |  |  |  |
| 23 | Frame Data |  |  |  |  |  |  |  |
| 24 | Frame Data |  |  |  |  |  |  |  |

The port field identifies the port that invoked the Asynchronous Alert.

The tag field is assigned by the Master and returned in the response from the destination initiator.  It shall be unique among all the tags currently active from the Master.

The return path field specifies the path component that shall be placed in the address field of the RESPONSE SMS.

The alert Unique_ID field identifies the node that invoked the Asynchronous Alert.  It specifies the Unique_ID of the node that generated the Async_Alert SMS.

The type field contains the value copied from the corresponding field in the ASYNC ALERT SMS if the Master is forwarding an Asynchronous Alert.  The Master may also generate certain type codes itself (see 10.5).

The subtype field indicates more detailed information and is type dependent.

The type information field contains additional information that is type and subtype dependent.

The Control , Channel, and Frame Data Fields are copied from the Async_Alert SMS to provide the initiator receiving the Master_Alert the information necessary to route the SMS to the appropriate layer of the protocol.

The acknowledgment for the MASTER ALERT SMS is the receipt of a RESPONSE SMS with the same tag value as was used for the MASTER ALERT SMS.  The return code field of the RESPONSE SMS shall indicate whether or not the MASTER ALERT was processed successfully (see  11.2.14).

Sincerely,

Mark A. DeWilde

Principal System Architect

Pathlight Technologies

Voice:	(607)266-4000 X-403
FAX: 	(607)266-0352

Email:	mark@pathlight.com