Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/
Date:	27 October, 1995	
Project:	SSA-TL1
Ref Doc.:	X3T10.1/
Reply to:	Mark DeWilde

To: 	X3T10.1 Membership
From:	Mark DeWilde

Subject:   Unique ID in Master_Alert

BACKGROUND

Before all nodes had Unique_ID’s, the chosen method for the Master to indicate which node was causing the Async_Alert was to provide the alert path to each initiator.  This path is different for each initiator and must be calculated by the master. For large webs, this could be a large task.  The initiators must walk their network topology tables to determine the Unique_ID of the node generating the async alert, since the path provided by the master may not be in use by the initiator.

PROPOSAL

Simplify the task of the master by changing the alert path field in the Master_Alert to a Unique_ID field containing the Unique_ID of the node generating the async alert.

Sincerely,

Mark A. DeWilde

Principal System Architect

Pathlight Technologies

Voice:	(607)266-4000 X-403
FAX: 	(607)266-0352

Email:	mark@pathlight.com