Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a210R1
Date:	November 6, 1995	
Project:	0989D
Ref Doc.:	X3T10.1/0989D
Reply to:	Mark DeWilde

To: 	X3T10.1 Membership
From:	Mark DeWilde

Subject:   Interlocked Election of Master Process

BACKGROUND

At the October SSA Week meetings, Revision 0 of this proposal entitled “loss of initiator table space” was discussed.   With further discussion it became apparent that the loss of table space and several other problems were being caused by the common problem of multiple Masters existing on a web for a transient period.  In order to eliminate this transient period, I proposed an interlocked protocol verbally.  I took the action item to produce a written proposal and forward to the committee for comment and consideration. That proposal (which also cures the problem described in r0) follows.

PROPOSAL

Add master priority to the QUERY NODE SMS so that a current master receives immediate notification that it must resign.  Add a “Current Master” bit to the QUERY NODE REPLY so that the pending master “knows” to delay the assumption of mastership.  Use the QUERY NODE with the “MA” bit set to indicate to the pending master that the resigning master has resigned.  This set of changes provides an interlock mechanism that prevents multiple masters for any interval.  Add a timer to detect loss of resigning master in case the resigning master fails to activate the pending master with it’s QUERY NODE with MA set.

The following sections are changed as indicated:

12.8  Master Negotiation Process

During the configuration process, the initiator with the highest Master priority is elected to be the Master (see 11.2.6).  If more than one initiator is set at the same highest Master priority, then the initiator with the highest Unique ID among them is elected to be the Master.

Each initiator compares its own Master priority with the Master priority it receives from each other initiator in the QUERY NODE REPLY SMS.  Each initiator in the group with the highest Master priority then compares its own Unique ID with those of the other initiators in the group.  The initiator within the group with the highest Unique ID becomes the Master.  If two Webs that both contain a Master are joined by a new link, then each Master invokes the Configuration process for the new link. Since they may invoke the Configuration Process at different times, an interlock mechanism is necessary to allow them to complete outstanding Async Alert actions on their portions of the web prior to one of them assuming mastership of the entire web.  This is accomplished by the use of the “CM” bit in the QUERY NODE REPLY SMS, and the “My Master Priority” field in the QUERY NODE SMS.  The process is as follows, assuming the master of the first sub-web begins walking before the master of the second sub-web:

1)	The Master of the first sub web walks the second sub-web looking for Master Capable Nodes.  In doing so, it informs all of the Master Capable Nodes in the second sub-web of it’s own Master Priority.

2)	Each Master Capable Node in the second sub-web returns it’s master priority and CM bit in the query node reply to the master of the first sub-web.

3)	If the master of the first sub-web determines that it must resign, then it stops accepting new ASYNC ALERT SMS’s and continues processing the remaining pending ones.  If it receives a QUERY NODE SMS after it has completed  processing all pending ASYNC ALERT SMS’s, then it returns a QUERY NODE REPLY with the CM bit clear.  If it receives a QUERY NODE SMS while it is still processing pending ASYNC ALERT SMS’s, then it returns a QUERY NODE REPLY with the CM bit set.  When it has completed processing the pending ASYNC ALERT SMS’s, it sends a QUERY NODE SMS with the “MA” bit set to indicate it has finished it’s activities and has resigned. Note that it is not necessary to send the QUERY NODE SMS with the MA bit set if it earlier returned the QUERY NODE REPLY with the CM bit clear.

4)	If the master of the first sub-web determines it has won mastership, and has received a QUERY NODE REPLY with the CM bit clear, then it assumes mastership of the entire web, configuring the new ports and sending MASTER ALERT SMS’S to the initiators indicating reconfiguration required.

5)	If the master of the first sub-web determines it has won mastership, and has received a QUERY NODE REPLY with the CM bit set, then it delays assuming mastership of the entire web.  The pending master starts a 5 second timer and  continues as master of the portion of the web over which it already has mastership.  If the pending master receives a QUERY NODE SMS with the MA bit set from the resigning master, then the pending master stops the timer, sends a QUERY NODE REPLY SMS to the resigning master, and assumes mastership of the entire web. It then begins configuring the new ports and sending MASTER ALERT SMS’S to the initiators indicating reconfiguration required.  If  the timer times out, then the pending master sends a QUERY NODE SMS to the resigning master.  If a query node reply is received with the CM bit set, the timer is re-started.  If no query node reply is received within 5 seconds, then the pending master assumes mastership of the entire web and resets the nodes on the second sub-web. If a query node reply is received with the CM bit clear, then the pending master assumes mastership of the entire web and completes re-configuration.

When the current Master changes its Master priority the following actions are taken.

- If the current Master determines that it should resign then it finishes processing any ASYNC ALERT SMSs previously received and it does not send an ASYNC REPLY SMS to any future ASYNC ALERT SMS.

- The current Master forwards its new Master priority to each other initiator in a MASTER ALERT SMS.

- Each other initiator enters the new Master priority into its Configuration table and returns a Response SMS.

- If another initiator determines that it should become the new Master then it issues a CONFIGURE PORT SMS to each port in the Web.  Each port returns a Response SMS.  The initiator is then the new Master.

When an initiator (other than the current Master) changes its Master priority:

- The initiator sends the current Master an ASYNC ALERT SMS containing its new Master priority.

- The current Master enters the new Master priority into its Configuration table and forwards it to each other initiator in a MASTER ALERT SMS.

- Each other initiator enters the new Master priority into its Configuration table and returns a Response SMS.

- If the current Master determines that it should resign then it finishes processing any ASYNC ALERT SMSs previously received and it does not send an ASYNC REPLY SMS  to any future ASYNC ALERT SMSs.

If the initiator that changed its Master priority determines that it should become the new Master, then it waits to receive the ASYNC REPLY SMS  before issuing a CONFIGURE PORT SMS to each port in the Web.  It is then the new Master.

11.2.5 QUERY NODE SMS

This SMS is sent from a initiator to every other Operational node during the Configuration process.  QUERY NODE can also be used as a remote wrap test to verify the integrity of the Path.

The QUERY NODE SMS is defined in table 31.

Table 31 - QUERY NODE SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (00h) |  |  |  |  |  |  |  |
| 1 | ssa-tl1 version (02h) |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | return path |  |  |  |  |  |  |  |
| 5 | return path |  |  |  |  |  |  |  |
| 6 | return path |  |  |  |  |  |  |  |
| 7 | return path |  |  |  |  |  |  |  |
| 8 | unique id |  |  |  |  |  |  |  |
| 9 | unique id |  |  |  |  |  |  |  |
| 10 | unique id |  |  |  |  |  |  |  |
| 11 | unique id |  |  |  |  |  |  |  |
| 12 | unique id |  |  |  |  |  |  |  |
| 13 | unique id |  |  |  |  |  |  |  |
| 14 | unique id |  |  |  |  |  |  |  |
| 15 | unique id |  |  |  |  |  |  |  |
| 16 | dr | ma | My Master Priority |  |  | reserved |  |  |

The ssa-tl1 version field defined in Table 32 identifies the version of SSA-TL1 being used by the sender.

Table 32 - QUERY NODE SMS

| Version | Description |
| --- | --- |
| 00h-01h | SSA-TL1 implementations before standardization |
| 02h | This standard |
| 03h-FFh | reserved |

The tag field is returned in the QUERY NODE REPLY SMS.  The tag is assigned by the initiator and it shall be unique among the tag values that are currently active from that initiator.

The return path field specifies the path component that shall be placed in the Initiator table entry created in response to this QUERY NODE SMS, if an entry is created.  This value is used for the address field of the resulting QUERY NODE REPLY SMS, and is used for the address field of any future application SMS that utilizes this Initiator table entry.

The unique id field contains the Unique ID of the initiator that issued QUERY NODE SMS.

The dr bit (Disable Registration) controls the updating of the Initiator table in the target.  If the dr bit is cleared and the destination node is a target, then the target shall enter the specified return path and unique id into its Initiator table.  If an initiator intends to use several alternative paths to the same target node then it shall issue QUERY NODE SMS with the dr bit is cleared once over each path.

The ma bit (Master Alive) is set when initiators are using the QUERY NODE SMS to detect the Loss of Master.

The “My Master Priority” field is valid only for Master Capable Nodes, and contains the Master Priority of the node that issued the QUERY NODE SMS.  It is used by the node being queried to decide if it needs to resign mastership.

The destination node normally returns a QUERY NODE REPLY SMS.  If a field is invalid then the destination node returns a RESPONSE SMS instead of QUERY NODE REPLY SMS.

11.2.6 QUERY NODE REPLY SMS

The QUERY NODE REPLY SMS as defined in table 33 is returned when a QUERY NODE SMS is received.  The QUERY NODE REPLY SMS is returned on the same port that received the corresponding QUERY NODE SMS.

Table 33 - QUERY NODE REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (01h) |  |  |  |  |  |  |  |
| 1 | reserved | port |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | upper level protocol |  |  |  |  |  |  |  |
| 5 | itf | Master priority |  |  | reserved |  |  |  |
| 6 | total ports |  |  |  |  |  |  |  |
| 7 | ssa-tl1 version (02h) |  |  |  |  |  |  |  |
| 8 | unique id |  |  |  |  |  |  |  |
| 9 | unique id |  |  |  |  |  |  |  |
| 10 | unique id |  |  |  |  |  |  |  |
| 11 | unique id |  |  |  |  |  |  |  |
| 12 | unique id |  |  |  |  |  |  |  |
| 13 | unique id |  |  |  |  |  |  |  |
| 14 | unique id |  |  |  |  |  |  |  |
| 15 | unique id |  |  |  |  |  |  |  |
| 16 | initiator id |  |  |  |  |  |  |  |
| 17 | initiator id |  |  |  |  |  |  |  |
| 18 | initiator id |  |  |  |  |  |  |  |
| 19 | initiator id |  |  |  |  |  |  |  |
| 20 | p1o | p2o | CM | reserved |  |  |  |  |

The port field indicates the number of the port currently being used.

The tag field is copied from the QUERY NODE SMS.  It identifies the QUERY NODE SMS for which this reply is being generated.

The upper level protocol field identifies the upper-level protocol to communicate with the node.  It shall contain a value from Table 34 that may be returned by QUERY NODE REPLY SMS.

Table 34- Upper-level protocol code values

| ULP code | Protocol | Notes |
| --- | --- | --- |
| 00h | Use the QUERY PROTOCOL SMS to list upper level protocols supported | 1 |
| 01h | No upper-level protocol supported | 1 |
| 02h | Vendor specific | 1,2 |
| 03h-7Fh | reserved |  |
| 80h | SSA-IA 95SP | 1,2 |
| 81h | reserved |  |
| 82h | SSA S2P | 1,2 |
| 83h | SSA S3P | 1,2 |
| 84h-FAh | reserved |  |
| FBh-FFh | Vendor specific | 2 |
| Notes 1) May be returned in QUERY NODE REPLY. 2) May be returned in QUERY PROTOCOL REPLY |  |  |

The Initiator table full (itf) bit is set when there is no space left in the Initiator table to make an entry for the initiator that sent QUERY NODE SMS.

The Master priority field defines the priority of the node for becoming the Web Master.  A value of zero indicates that the node is not capable of functioning as a Master.  Any non-zero value indicates the node’s priority for becoming the Master.  A value of one is the lowest priority and seven the highest

Implementer’s note - A initiator may optionally fix its Master priority at 4 (the default priority) or provide some mechanism outside of SSA to change its Master priority dynamically.

The total ports field contains a value that is one less than the number of ports implemented.  If this value exceeds two, the QUERY SWITCH SMS should be used to retrieve a port mask for the switch.

The unique id field contains the node’s Unique ID.

The initiator id contains a value created by the target and returned to the initiator.  This field shall be used by the initiator in any future application SMSs which are utilizing the same path as used by the QUERY NODE SMS which caused this QUERY NODE REPLY SMS is be generated.

The p1o bit is only valid for single port and dual port nodes, Switch nodes shall clear the p1o bit.  For single port or dual port nodes, the p1o bit is set if port 1 is operational, and is cleared if port 1 is not operational.

The p2o bit is only valid for single port and dual port nodes, Switch nodes shall clear the p2o bit.  For single port or dual port nodes, the p2o bit is set if port 2 is operational, and is cleared if port 2 is not operational.

The CM bit is only valid for Master Capable Nodes, and indicates that the node sending the QUERY NODE REPLY is the Current Master.  If this bit is set and there is a Master Election that would cause this node to resign and another to become master, the node that will assume mastership will wait to do so until it is queried by this node, indicating this node’s master duties have been completed.

The Path component of the address field in a QUERY NODE REPLY SMS frame is copied from the return path field in the corresponding QUERY NODE SMS.  All padding bytes shall be discarded.

Sincerely,

Mark A. DeWilde

Principal System Architect

Pathlight Technologies

Voice:	(607)266-4000 X-403
FAX: 	(607)266-0352

Email:	mark@pathlight.com