Accredited Standards Committee

X3, Information Processing Systems

Doc:   X3T10.1/95a210R2

Date:  November 9, 1995

Project:  0989D

Ref Doc.: X3T10.1/0989D

Reply to: Mark DeWilde

To:  X3T10.1 Membership

From:     Mark DeWilde

Subject:   Interlocked Election of Master Process, Rev 2

BACKGROUND

At the October SSA Week meetings, Revision 0 of this proposal entitled

"loss of initiator table space" was discussed.   With further

discussion it became apparent that the loss of table space and several

other problems were being caused by the common problem of multiple

Masters existing on a web for a transient period.  In order to

eliminate this transient period, I proposed an interlocked protocol

verbally.  I took the action item to produce a written proposal and

forward to the committee for comment and consideration. That proposal

(which also cures the problem described in r0) follows. Since Rev 1 of

this proposal, the methods have been generalized to take more of the

possible synarios into consideration, while maintaining the basic

process originally described.

PROPOSAL

Add master priority to the QUERY NODE SMS so that a current master

receives immediate notification that it must resign.  Add a "Current

Master" bit to the QUERY NODE REPLY so that the pending master "knows"

to delay the assumption of mastership.  Use the QUERY NODE with the

"MA" bit set to indicate to the pending master that the resigning

master has resigned.  This set of changes provides an interlock

mechanism that prevents multiple masters for any interval.  Add a

timer to detect loss of resigning master in case the resigning master

fails to activate the pending master with it’s QUERY NODE with MA set.

The following sections are changed as indicated:

12.8  Master Negotiation Process

During the configuration process, the initiator with the highest

Master priority is elected to be the Master (see 11.2.6).  If more

than one initiator is set at the same highest Master priority, then

the initiator with the highest Unique ID among them is elected to be

the Master.

Each initiator compares its own Master priority with the Master

priority it receives from each other initiator in the QUERY NODE REPLY

SMS.  Each initiator in the group with the highest Master priority

then compares its own Unique ID with those of the other initiators in

the group.  The initiator within the group with the highest Unique ID

becomes the Master.  If two Webs that both contain a Master are joined

by a new link, then each Master invokes the Configuration process for

the new link. Since they may invoke the Configuration Process at

different times, an interlock mechanism is necessary to allow them to

complete outstanding Async Alert actions on their portions of the web

prior to one of them assuming mastership of the entire web.  This is

accomplished by the use of the "CM" bit in the QUERY NODE REPLY SMS,

and the "My Master Priority" field in the QUERY NODE SMS. The

following sections describe the usage of the fields and messages.

Definitions of terms used in the description:

Sub-Web:

A string of nodes with target functions, initiator functions, or

a combination of both that are added to another web as a unit.

Resignation Request:

Received by a Master or Resigning master of a sub-web, this is a

a QUERY NODE SMS that contains either a higher master priority

than the receiving node’s, or the same Master priority and a

higher Unique ID.  A resignation request is answered with a QUERY

NODE REPLY with the CM bit set.

Pending Resignation:

A Resignation is Pending whenever a QUERY NODE REPLY SMS is sent

with the CM bit set.  The Pending Resignation is satisfied when

the Formal Resignation is performed.

Resigning Master:

The (temporary) Master node of a Sub-Web that has received  one

or more Resignation Requests.  It no longer accepts ASYNC ALERT

SMS’s, but has not yet finished processing those received prior

to the QUERY NODE SMS.

Formal Resignation:

A Resigning Master Formally Resigns to a node with which it has

completed a Resignation Request transaction at an earlier time

when it sends a QUERY NODE SMS with the MA bit set to that node.

Pending Master:

The current Master of a sub-web that, after walking a newly

attached sub web containing one or more  active Masters,  has

discovered that it has won mastership of that new sub web.  It

waits to assume mastership of the new sub-web until all of the

resigning masters formally resign.

Assuming Master:

A Pending Master that has received all of the Formal Resignations

from the Resigning Masters is the Assuming Master until it has

completed the web configuration and is then the web Master. The

Assuming Master may receive a Resignation Request and not become

the web Master.

Rules for Interlocked Master Election

1.   If the Master of a sub-web receives a Resignation Request, it

becomes a Resigning Master. It immediately stops accepting new

ASYNC ALERT SMSs, sends a QUERY NODE REPLY SMS with the CM bit

set, logs a Pending Resignation, and continues finishing the

actions required by the remaining pending ASYNC ALERTs.

2.   A Resigning Master shall reply to all  subsequent Resignation

Requests by returning QUERY NODE REPLY SMSs with the CM bit set

and logging additional Pending Resignations until it has

completed the pending ASYNC ALERT SMS actions.  After Completing

the actions required by the pending ASYNC ALERT SMSs, the

resigning master shall reply to  QUERY NODE SMSs by returning a

QUERY NODE REPLY SMS with the CM bit clear.  Any QUERY NODE SMSs

that would not require resignation are always answered with QUERY

NODE REPLY SMSs with the CM bit clear.

3.   When a resigning master has completed all actions required by

pending ASYNC ALERT SMSs, it performs Formal Resignations for all

Pending Resignations.

4.   A Pending Master will wait until all Resigning Masters have

Formally Resigned prior to configuring the ports on the new Sub-

Web. When it has received all Formal Resignations from all

Resigning Masters, it becomes the Assuming Master.

5.   If a Pending Master receives one or more  Resignation Requests,

it becomes a Resigning Master. It stops accepting ASYNC ALERT

SMSs, completes any actions required by pending ASYNC ALERT SMSs,

and then formally resigns to all nodes with which it has a

Pending Resignation.

6.   If  the  Assuming Master receives an ASYNC ALERT SMS indicating

"PORT NOW OPERATIONAL", from a port during configuration, it must

check it’s web topology map to see if there has been a new sub-

web added to the web.  If not, then the Assuming Master continues

it’s configuration of the ports in the web. If there has been a

new sub-web added, then the Assuming Master walks the new sub-

web.

If the web walk indicates that the Assuming Master will

be the master of the new sub-web, and there are no resigning

masters in the new sub-web, then the Assuming Master

performs a sub-web reset on the new sub web and configures

it.  The sub-web reset is required since the sub-web had no

master when attached, and there could be outstanding

commands and stale initiator table entries in the nodes.

If the web walk indicates that the Assuming Master will

be the master of the new sub-web, and there are Resigning

Masters in the new sub-web, then the Assuming Master waits

for the Resigning Master(s) to Formally Resign, and then

configures the ports on the new sub-web.

If the web walk discovers a node that would win the

Master Election Process, but has not yet begun walking the

web, the Assuming Master completes the configuration

process.  When the node with the higher priority begins

walking the web, the assuming master will become a Resigning

Master, and the transition of Mastership will proceed as

outlined.

If the Assuming Master receives a Resignation Request

from a node in the new web after the ASYNC ALERT SMS,  it

sends a QUERY NODE REPLY with the CM bit set, and becomes a

Resigning Master.  Since the ASYNC ALERT SMS Processing

includes walking the new web, this will be completed prior

to resignation.  If configuration of ports was in progress,

this operation ceases since the new master will re-configure

all of the web ports.

7.   If an Assuming Master receives an ASYNC ALERT SMS during the

configuration process indicating that a portion of the sub- web

has been disconnected, then the Assuming Master must modify it’s

web topology map and perform third-party quiesces on behalf of

any missing Initiators.  If the portion of the web removed is not

in the Assuming Master’s topology map, then the Assuming Master

must perform a sub-web reset on the nodes in the sub-web being

annexed, since the Unique ID’s of the missing initiators are

unknown.

8.   The Assuming Master, after configuring all the ports in the web

must issue MASTER ALERT SMSs to the initiators in the sub-webs to

cause them to walk the portions of the web new to them.  This

could be done with the "PORT NOW OPERATIONAL" ALERT CODE,  or by

the "RECONFIGURATION REQUIRED" ALERT CODE.  If there are multiple

sub webs added, the latter may require fewer MASTER ALERT SMSs.

9.   Each Pending Master shall start a timer when it receives a reply

confirming the receipt of a Resignation Request.  If the

Resigning Master has not issued a Formal Resignation in 5

seconds, then the Pending Master shall send a QUERY NODE SMS to

the Resigning Master.  If the Resigning Master returns a QUERY

NODE REPLY with the CM bit set, the Pending Master restarts the

Timer.  If the Resigning Master returns a QUERY NODE REPLY SMS

with the CM bit clear, then the Pending Master accepts this reply

as the Formal Resignation, but still replies to any subsequent

Formal Resignations from that node.  If no reply is received

within another 5 seconds, the Pending Master issues a TOTAL RESET

to the node, followed by re-configuration of the web.

10.  It is possible for the Assuming Master to receive a Resignation

Request during configuration before configuring the port with the

associated pending ASYNC ALERT SMS indicating "PORT NOW

OPERATIONAL",  This situation indicates that the Resignation

Request comes from a node within a sub web that was attached to a

sub-web containing a resigning master after the Assuming master

had walked the sub-web containing the Resigning Master. In this

case, the Assuming Master becomes a Resigning Master, ceases

configuration, completes the pending ASYNC ALERT SMS processing

and then Resigns Formally. The new Assuming Master will send a

MASTER ALERT indicating "PORT NOW OPERATIONAL" to cause the old

Assuming Master to walk the new sub-web.

When the current Master changes its Master priority the following

actions are taken.

1)   If the current Master determines that it should resign then it

finishes processing any ASYNC ALERT SMSs previously received and it

does not send an ASYNC REPLY SMS to any future ASYNC ALERT SMS.

2)   The current Master forwards its new Master priority to each other

initiator in a MASTER ALERT SMS.

3)   Each other initiator enters the new Master priority into its

Configuration table and returns a RESPONSE SMS.

4)   If another initiator determines that it should become the new

Master then it issues a CONFIGURE PORT SMS to each port in the Web.

Each port returns a RESPONSE SMS.  The initiator is then the new

Master.

When an initiator (other than the current Master) changes its Master

priority:

1)   The initiator sends the current Master an ASYNC ALERT SMS

containing its new Master priority.

2)   The current Master enters the new Master priority into its

Configuration table and forwards it to each other initiator in a

MASTER ALERT SMS.

3)   Each other initiator enters the new Master priority into its

Configuration table and returns a RESPONSE SMS.

4)   If the current Master determines that it should resign then it

finishes processing any ASYNC ALERT SMSs previously received and it

does not send an ASYNC REPLY SMS  to any future ASYNC ALERT SMSs.

If the initiator that changed its Master priority determines that it

should become the new Master, then it waits to receive the ASYNC REPLY

SMS  before issuing a CONFIGURE PORT SMS to each port in the Web.  It

is then the new Master.

11.2.5 QUERY NODE SMS

This SMS is sent from a initiator to every other Operational node

during the Configuration process.  QUERY NODE can also be used as a

remote wrap test to verify the integrity of the Path.

The QUERY NODE SMS is defined in table 31.

Table 31 - QUERY NODE SMS

Byte Bit 7   6     5     4     3     2     1   Bit 0

0                   sms code (00h)

1               ssa-tl1 version (02h)

2                        tag

3                        tag

4                    return path

5                    return path

6                    return path

7                    return path

8                     unique id

9                     unique id

10                     unique id

11                     unique id

12                     unique id

13                     unique id

14                     unique id

15                     unique id

16    dr    ma  My Master Priority     reserved

The ssa-tl1 version field defined in Table 32 identifies the version

of SSA-TL1 being used by the sender.

Table 32 - QUERY NODE SMS

Version   Description

00h-01h   SSA-TL1 implementations before

standardization

02h     This standard

03h-FFh   reserved

The tag field is returned in the QUERY NODE REPLY SMS.  The tag is

assigned by the initiator and it shall be unique among the tag values

that are currently active from that initiator.

The return path field specifies the path component that shall be

placed in the Initiator table entry created in response to this QUERY

NODE SMS, if an entry is created.  This value is used for the address

field of the resulting QUERY NODE REPLY SMS, and is used for the

address field of any future application SMS that utilizes this

Initiator table entry.

The unique id field contains the Unique ID of the initiator that

issued QUERY NODE SMS.

The dr bit (Disable Registration) controls the updating of the

Initiator table in the target.  If the dr bit is cleared and the

destination node is a target, then the target shall enter the

specified return path and unique id into its Initiator table.  If an

initiator intends to use several alternative paths to the same target

node then it shall issue QUERY NODE SMS with the dr bit is cleared

once over each path.

The MA bit (Master Alive) is set when initiators are using the QUERY

NODE SMS to detect the Loss of Master.

The "My Master Priority" field is valid only for Master Capable Nodes,

and contains the Master Priority of the node that issued the QUERY

NODE SMS.  It is used by the node being queried to decide if it needs

to resign mastership.

The destination node normally returns a QUERY NODE REPLY SMS.  If a

field is invalid then the destination node returns a RESPONSE SMS

instead of QUERY NODE REPLY SMS.

11.2.6 QUERY NODE REPLY SMS

The QUERY NODE REPLY SMS as defined in table 33 is returned when a

QUERY NODE SMS is received.  The QUERY NODE REPLY SMS is returned on

the same port that received the corresponding QUERY NODE SMS.

Table 33 - QUERY NODE REPLY SMS

Byte  Bit7   6     5     4     3     2     1     Bit 0

0                   sms code (01h)

1     resv |             port

2                        tag

3                        tag

4                upper level protocol

5      itf | Master priority |        reserved

6                    total ports

7               ssa-tl1 version (02h)

8                     unique id

9                     unique id

10                    unique id

11                    unique id

12                    unique id

13                    unique id

14                    unique id

15                    unique id

16                   initiator id

17                   initiator id

18                   initiator id

19                   initiator id

20  p1o   p2o    CM             reserved

The port field indicates the number of the port currently being used.

The tag field is copied from the QUERY NODE SMS.  It identifies the

QUERY NODE SMS for which this reply is being generated.

The upper level protocol field identifies the upper-level protocol to

communicate with the node.  It shall contain a value from Table 34

that may be returned by QUERY NODE REPLY SMS.

Table 34- Upper-level protocol code values

ULP   Protocol                                            Notes

code

00h   USE THE QUERY PROTOCOL SMS TO LIST UPPER LEVEL       1

PROTOCOLS SUPPORTED

01h   NO UPPER-LEVEL PROTOCOL SUPPORTED                    1

02h   Vendor specific                                     1,2

03h-7Fh reserved

80h   SSA-IA 95SP                                         1,2

81h   reserved

82h   SSA S2P                                             1,2

83h   SSA S3P                                             1,2

84h-FAh reserved

FBh-FFh Vendor specific                                      2

Notes

1) May be returned in QUERY NODE REPLY.

2) May be returned in QUERY PROTOCOL REPLY

The Initiator table full (itf) bit is set when there is no space left

in the Initiator table to make an entry for the initiator that sent

QUERY NODE SMS.

The Master priority field defines the priority of the node for

becoming the Web Master.  A value of zero indicates that the node is

not capable of functioning as a Master.  Any non-zero value indicates

the node's priority for becoming the Master.  A value of one is the

lowest priority and seven the highest

Implementer's note - A initiator may optionally fix its

Master priority at 4 (the default priority) or provide some

mechanism outside of SSA to change its Master priority

dynamically.

The total ports field contains a value that is one less than the

number of ports implemented.  If this value exceeds two, the QUERY

SWITCH SMS should be used to retrieve a port mask for the switch.

The unique id field contains the node's Unique ID.

The initiator id contains a value created by the target and returned

to the initiator.  This field shall be used by the initiator in any

future application SMSs which are utilizing the same path as used by

the QUERY NODE SMS which caused this QUERY NODE REPLY SMS is be

generated.

The p1o bit is only valid for single port and dual port nodes, Switch

nodes shall clear the p1o bit.  For single port or dual port nodes,

the p1o bit is set if port 1 is operational, and is cleared if port 1

is not operational.

The p2o bit is only valid for single port and dual port nodes, Switch

nodes shall clear the p2o bit.  For single port or dual port nodes,

the p2o bit is set if port 2 is operational, and is cleared if port 2

is not operational.

The CM bit is only valid for Master Capable Nodes, and indicates that

the node sending the QUERY NODE REPLY is the Current Master.  If this

bit is set and there is a Master Election that would cause this node

to resign and another to become master, the node that will assume

mastership will wait to do so until it is queried by this node,

indicating this node's master duties have been completed.

The Path component of the address field in a QUERY NODE REPLY SMS

frame is copied from the return path field in the corresponding QUERY

NODE SMS.  All padding bytes shall be discarded.

Sincerely,

Mark A. DeWilde

Principal System Architect

Pathlight Technologies

Voice:    (607)266-4000 X-403

FAX:      (607)266-0352

Email:    mark@pathlight.com