Title:	Undesirable or Unacceptable Recovery from Hot Swap of SSA Devices

Problem:	When a device is removed from the path between an initiator and target, the outstanding commands that use the broken path must be quiesced and re-tried.  If the target device is a tape drive or optical storage device, the unexpected break in the data flow can cause long delays an rewinding and re-starting as well as consuming excessive time retrying the commands as a best case synario.  With some optical devices where a “warm-up” write to reach steady state is required, quantities of one-time write media will be destructively consumed.  Placing a large buffering requirement on these devices to work around this problem makes the SSA version of these devices prohibitively expensive. While good for SSA competing technologies, it becomes a serious consideration for users who would consider using SSA devices for data backup.

Proposed Solution:	Currently, when the command/data path between initiator and target is broken, the protocol requires that the commands from the initiator be quiesced either by the initiator along an alternate path or by the Master through a third-party Quiesce.  The commands are aborted and re-issued by the upper level protocol if possible.  It would be more desirable from the hot-plugging standpoint if the transfers in progress could be automatically re-routed to an alternate path, when available, and allowed to complete without cancellation and re-try.  The mechanisms  needed in order to do this transparently to the protocol layer are:

1.	A means by which the data set represented by a Data_ready/Data_Reply set of SMS’s or a Data_request SMS is acknowledged by the receiver of the data prior to the sender discarding the buffer, so that it could be re-sent along an alternate path if transmission on the main (command) path were lost.

2.	A means to detect how much of the data set has been sent and to back up and re-send the remainder of the set to a new path and channel.

3.	A means for the receiver of the data to request a retransmit of the data set remaining in the transfer to a new path/channel,.

4.	A means for the receiver of the data to provide the sender with the required new path and channel data.

One method of implementing these mechanisms would be to define  a “re-transmit” bit in an additional byte of the Data_Reply and the Data_Request SMS’s, and to use special values of the other fields to indicate special purposes.  In the case of an initiator requesting data from a target, the following sequence could be implemented:

1.	Initiator sends SCSI_CMD SMS specifying tag and initiator_ID

2.	Target sends Data_Ready specifying tag, offset, byte count and transfer ID

3.	Initiator sends Data_Reply SMS specifying initiator_ID, byte count, 	receiver channel and transfer ID.

4.	Target begins sending data frames

5.	When all the data frames are received, the initiator notifies the target by sending a Data_Reply SMS with tag, initiator_ID, receiver channel and transfer ID as before, but with byte count and retransmit  = 0.  The target treats this as an acknowledgment of  the data transfer.

If the link breaks before the transfer has completed, then:

1.	Async Alerts cause Master Alert, notifying initiator of  loss of path.  The Initiator then sets up to use an alternate path.

2.	The Initiator sends a Data_reply SMS with tag, and transfer ID as before, but with the new initiator ID that corresponds to the new path, a new channel number, and a byte count equal to the number of bytes to be re-transmitted .  The initiator also sets the (new) re-transmit bit in the SMS.  The target treats this as a request to re-try the last transfer of  the data transfer, and to continue with the transfer along the new path/channel. The SMS may be received on either port at the target. The presence of the retransmit bit differentiates the SMS from a request for more data.

3.	The target re-transmits the lost data along the new path to the initiator.

4.	When all the data frames are received, the initiator notifies the target by sending a Data_Reply SMS with tag, initiator_ID, receiver channel and transfer ID as before, but with byte count and retransmit = 0.  The target treats this as an acknowledgment of  the data transfer.

If no alternate path exists, then the Master will perform a third-party quiesce as before on the outstanding commands.

In the case of an Initiator sending data to a target, the sequence is as follows:

1.	Initiator sends SCSI_CMD SMS specifying tag and initiator_ID

2.	Target sends Data_Request specifying tag, offset, byte count and tchannel.

3.	Initiator begins sending data frames to target.

4.	When all the data frames are received, the target notifies the initiator by sending a Data_Request SMS with tag and  Tchannel as before, but with byte count = 0. The retransmit bit is not set.   The Initiator treats this as an acknowledgment of  the data transfer.

If the link breaks before the transfer has completed, then:

1.	Async Alerts cause Master Alert, notifying initiator of  loss of path.  The Initiator then sets up to send data over an alternate path.

2.	The initiator uses the alternate path to send a Data_Reply SMS containing the Tag, and the new initiator ID with the byte count, Ichannel and transferID fields set to zero. The retransmit bit is set.

3.	The Target sends a Data_request  SMS with old tag, new byte offset and byte count equal to the number of bytes to be re-transmitted .  The target provides a new Tchannel, and uses the initiator ID from the Data_reply to determine over which path to send the new Data_request.

4.	The Initiator treats the Data_request  as a request to re-try the remainder of the last data transfer, and to continue with the transfer along the new path/channel. The SMS may be received on either port at the target. The presence of the retransmit bit differentiates the SMS from a request for more data.

5.	The initiator  re-transmits the lost data along the new path to the target.

6.	When all the data frames are received, the target notifies the target by sending a Data_Request SMS with tag as before, but with byte count = 0.  The initiator treats this as an acknowledgment of  the data transfer.

This method (or one similar) could provide the necessary mechanisms to allow commands to complete with minimal recovery overhead rather than be canceled and re-tried at a later time over an alternate path.  The receiver of the data always notifies the sender of how much to re-send, so there is no possibility of missing data.  If the web configuration is such that the alternate path is set up and established before all of the data frames can “drain” out of the pipeline, it would be possible for the old channel to be re-assigned to a different transfer. If the direction of the transfer were the same, the data frames from the old transfer could arrive and be accepted as those from the new transfer.  In order to prevent this race condition, it is possible to delay the re-allocation of the receiver channel for some time interval to allow the pipeline to empty. The received old frames would be discarded.

An additional SMS change required to support the proposal is to add a transfer_ID field to the Data_Request SMS, so that replies to multiple outstanding Data_Requests can be differentiated.