Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/
Date:	27 October, 1995	
Project:	
Ref Doc.:	X3T10.1/
Reply to:	Mark DeWilde

To: 	X3T10.1 Membership
From:	Mark DeWilde

Subject:   Use of S2P SCSI Status vs. Async Alert

BACKGROUND

S2P presently generates async alerts for SMS’s that have not caused link ERP’s, have valid TL1 ULP codes, but fail to satisfy checks within the S2P layer.  As an example, async alerts with a code of “unknown SMS” are generated for unknown S2P Codes.  The transport layer cannot perform this check, the upper level protocol must.  When the async alert is sent to the master, the master will send a Master_Alert to all the initiators indicating a node has received an unknown SMS from some initiator’s transport or upper level protocol layers.  No information is provided as to which layer has sent the original SMS.  In reality, if the S2P layer has received the SMS in order to perform the checking, we at least know it is an S2P SMS.  Initiators receiving the Master_Alert would need to provide the alert node’s unique_id to all upper level protocols if they were able to use this information. Since any protocol or the TL1 layer could have SMS’s outstanding to that node, no action can be taken by any of the layers until a command times out.  Not only is the information useless to the ULP, but permitting the Master_Alert to apply to ULP conditions means that it cannot be used by the transport layer to uniquely mean a transport layer problem.

PROPOSAL

Extend the number of S2P SCSI_Status SMS Status codes to cover the SMS code validation errors discovered at the S2P Layer.  It is still acceptable to use Async_alert to report transport layer errors  on application SMS’s and errors such as unknown ULP which must be parsed by the transport layer in order to route to the appropriate ULP.  Where the return path id is unknown, we could either discard the SMS, or add a field to the Master_Alert SMS that would contain the ULP code of the SMS that generated the Async_alert.  I prefer the latter.

Sincerely,

Mark A. DeWilde

Principal System Architect

Pathlight Technologies

Voice:	(607)266-4000 X-403
FAX: 	(607)266-0352

Email:	mark@pathlight.com