Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/95a214r0
Date:	November 14, 1995
Project:	X3T10.1/xxxxx
Ref. Doc.:	SSA-PH2
Reply to:	Ian Judd

To: 	   	X3T10.1 Membership
From:	Ian Judd

Subject: SSA Speed Negotiation

### BACKGROUND

For compatibility with earlier products it is expected that most 40 MB/s ports will be capable of dual-speed operation, i.e. they can negotiate with the remote node and operate at either 20 or 40 MB/s.

### PROPOSAL

Each link in the SSA Web negotiates its speed independently. This allows the Web to be heterogeneous; it is another aspect of spatial reuse. For example, a 40 MB/s adapter could be connected to a mixture of 40 MB/s and 20 MB/s disk drives. (For best performance the 40 MB/s drives should be nearer to the adapter than the 20 MB/s drives.)

The link speed is negotiated automatically at power-on. This is essential to allow communication to be established, which is a pre-requisite for configuring or resetting the Web.

Each port performs the negotiation algorithm in Figure 1 during the Disabled state, but only if the port is not operational. Negotiation is performed at power-on and after an unrecoverable link error or Total/Absolute_reset; it is not performed for recoverable link errors. Each port must complete speed negotiation before entering the Enabled state (otherwise an RR may be lost). The operating speed is locked in the Enabled, Ready and Check states.

Notes:

- The Receiver must be able to decode either of the 2 valid comma characters (DIS or FLAG) during speed negotiation, since the remote port may have already entered the Enabled state.

- The Transmitter starts at the maximum speed supported and it does not step down until the Receiver has determined the incoming speed from the remote node. This avoids any possibility of a live lock scenario where both ports are changing their Transmit speeds out of phase with the remote Receiver.

- The Receiver cycles through the supported speeds indefinitely until speed negotiation is successful. (There is no value in terminating the negotiation process without establishing communication.)

- There is no requirement for all future implementations to support operation at 20MB/s. It is sufficient for the two ports on each link to have one operating speed in common. For example, a 100/40MB/s node would ‘connect’ to a 40/20MB/s node at 40MB/s.

- The time-out for decoding 4 consecutive commas has been set to 200 characters for consistency with the maximum time allowed by SSA-PH1 to acquire character synchronisation.

- SSA-TL1 specifies that the minimum duration of the Disabled state must be 200 characters at 20 MB/s. During speed negotiation this needs to be interpreted as a minimum of 10 µs, since the character period varies.

- A Total_reset frame cannot be transmitted or received until speed negotiation has completed. To minimise the possibility that a node cannot be reset the negotiation process should preferably be implemented in hardware rather than firmware.

- If two 20/40 MB/s ports erroneously agreed to operate at 20 MB/s they would continue in this state indefinitely and performance would be degraded. This situation is very unlikely but it could be detected by the Master using the information in the Query_port_reply SMS; the Master could then force the speed to be negotiated again by issuing a Total_reset frame to one of the nodes.

The algorithm can support an implementation option whereby a port which is experiencing a high number of link errors forces the link to operate at a lower speed. To achieve this the port would simply cause an unrecoverable link error by entering the Disabled state, and during speed negotiation it would then start its Transmitter and Receiver at a lower speed.

Enter Disabled state
                         (Start sending DIS)
                                |
                                |            Y
                         Port Operational? -------------------*
                                |N                            |
                                |                             |
                         Set Tx speed = Maximum supported     |
                         Set Rx speed = Maximum supported     |
                                |                             |
 *----------------------------->|                             |
 |                              |                             |
 |                       Start 200 character time-out         |
 |                       (at current Rx speed)                |
 |                              |                             |
 |        *-------------------->|                             |
 |        |N                    |                             |
 |    Time-out? <------- 4 consecutive                        |
 |        |          N   commas decoded?                      |
 |        |Y                    |                             |
 |        |                     |Y                            |
 |    Cycle Rx speed            |                             |
 |    to next value      Set Tx speed = Rx speed              |
 |        |                     |                             |
 *--------*                     |<----------------------------*
                                |
                         Wait at least 10µS
                                |
                         Enter Enabled state
                         (Start sending FLAG)

Figure 1. Speed negotiation algorithm

- In a heterogeneous environment the available inter-connections may not be designed to operate at the highest speed supported by the nodes. For example, the user may wish to plug some new 20/40 MB/s disk drives into an existing 20 MB/s backplane. Therefore a multi-speed port should provide a configuration facility (eg. jumpers or a software utility) for manually selecting the maximum operating speed during installation.

Ian Judd
IBM, Havant
14 November, 1995