Accredited Standards Committee

X3, Information Processing Systems

Doc:   X3T10.1/95a218r0

Date:  November 14, 1995

Project:  0989D

Ref Doc.: X3T10.1/0989D

Reply to: Aaron Dailey

To:  X3T10.1 Membership

From:      Aaron Dailey

Subject:   Corrections for Configuration Process

BACKGROUND

The configuration process introduced in revision 7c is incomplete.

PROPOSAL

Change section 9.4 as follows:

Each configurator node shall invoke a configuration process to determine the other nodes that are present and their path address(es).

9.4.1 Configuration Process

The configuration process involves the following steps.

1) Each configurator node walks the web (see 9.4.2) to build a topology table (see 9.2.3)

2) Each configurator node examines the topology table to determine the master. A master is defined as the configuator node with the largest value reported in the master priority field.  In the event that more than one node has the same value for the master priority field the node with the largest unique id is the master.

3) Each configurator node constructs a configuration table from the information in the topology table.

4) Each configurator node registers with the nodes with which it expects to perform I/O.  The master node must register with every node to be able to later send CONFIGURE PORT SMS.

5) If the node is to be the master, (see 12.1) then it should internally set its operational ports to normal mode.

6) The master sends a CONFIGURE PORT SMS for each port to the nodes in the topology table.  Operational ports are set to normal mode, and inoperative ports are left unchanged.

7) The master sends a MASTER ALERT SMS with an alert code value of BOTH PORTS OF A LINK ARE IN NORMAL MODE for each link that has both ports in normal mode.  After configuring every port on a switch, the master generates a MASTER ALERT SMS with alert code value of ALL OPERATIONAL PORTS ON THE SPECIFIED NODE ARE IN NORMAL MODE.

8) Upon completion of step 7, the master sends a MASTER ALERT SMS with an alert value of ALL OPERATIONAL PORTS ON ALL NODES IN THE WEB ARE IN NORMAL MODE.

9.4.2 Web Walking Process

The configurator node is assigned as the base node.  All ports on the configurator node are marked as unexplored.

The following steps are then performed until all nodes have been encountered:

1) The configurator node selects a port on the base node, and marks it as explored.  The web walking proces is repeated for each unexplored, operational port on the base node.

2) The master capable node sends a QUERY NODE SMS to the destination node incrementing the path address by one until a query node reply is received reporting:

a) only one port operational indicating the end of a string has been reached

b) a Unique Id that already exists in the topology table indicating a loop has been established.

c) the TOTAL OTHER PORTS field exceeds two indicating that a switch has been discovered

3) The configurator node may optionally send QUERY PROTOCOL SMS and QUERY PORT SMS as desired.

3) If a switch is discovered, the configurator node sends a QUERY SWITCH SMS to determine the operational ports on the switch.  The base node is set to that switch node and the web walking process is recursively called.  The port indicated in the QUERY NODE REPLY is marked as explored.

Aaron Dailey

adaptec

(303) 516 4701

aarond@btc.adaptec.com