Accredited Standards Committee

Accredited Standards Committee

X3, Information Processing Systems

Doc:	X3T10.1

Date:	October 27, 1995

Project:	SSA-TL1

Ref Doc:	X3T10.1

Reply to:	Aaron Dailey

To:		X3T10.1 Membership

From:		Aaron Dailey

Subject:	Determining target status of initiators

# BACKGROUND

Currently, using a query node SMS, it is simple for an initiator to detect an initiator or a target.  An initiator will have master priority greater than zero; a target will have a zero priority and an upper level protocol.  However, it should be possible to have an initiator which also accepts target commands.  The spec does not clearly state how an initator would determine node’s dual target/initiator status.

# PROPOSAL

The upper level protocol field (byte 4) of the query node SMS is redefined to be a target upper level protocol field.  The same change is made the query protocol SMS - all protocols supported are target protocols.  An initiator which does not support any target upper level protocols will return 0x1, indicating ‘no target upper level protocol supported.’

An initiator is allowed to send any SMS code upper level protocol that it desires, regardless of the reply upper level protocol field returned in query node reply SMS.  Other initiators will not not be able to determine which upper level protocols a node supports as initiator.

If an application SMS is sent to a node which does that does not support a given upper level protocol, a transport layer error is sent.  This is detailed in section 11.1.1.2 of the current spec.

Aaron Dailey

adaptec

(303) 516 4701

aarond@btc.adaptec.com