Doc:  X3T10.1/95a222r0

Date: 27 November 1995

Project:

Ref Doc.:

Reply to: Aaron Dailey

To:       X3T10.1 Membership

From:     Aaron Dailey

Subject:  X3T10.1/95a222r0 Non Master Capable Configutors

Background:

It would useful to have nodes which, although they do not have the hardware

and software resources to become master, need to have knowledge of web

topology.  Currently, if a node is to receive master alerts, a necessary part

of keeping an accurate topology, the node must return a master priority

greater than zero. This implies that the node must be master capable.

Proposal:

Make the desire to receive master alerts independent of master priority.

Change bit 3 of byte 5 of query node reply to be a master alert subscription

bit.  Any node which expects to receive master alerts must set this bit.

Setting the master alert subscription bit does not include the node in the

master alive process.

Aaron Dailey

adaptec

(303) 516 4701

aarond@btc.adaptec.com