Accredited Standards Committee

X3, Information Processing Systems

Doc: 	X3T10.1/95-229R0

Date:	July 29, 2026

Project:

Ref Doc:	X3T10.1/0989D Rev 8a

Reply to:	Ed Carmona

To: 	X3T10.1 Membership

From: 	Ed Carmona

Subject: Additional Query Port information

BACKGROUND

Currently, a Configutor cannot receive key configuration parameters based on a Configuration Port SMS being issued to a node. The following proposal requests that changes be made to retrieve all configuration parameters including A Quota, B Quota, EUDC bit, Reflect bit, & Mode bit information.

PROPOSAL

The following changes are requested to the SSA-TL1 specifcations:

Change Table 36 as follows:

Table 36 - QUERY PORT REPLY SMS

| Byte | Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | sms code (0Bh) |  |  |  |  |  |  |  |
| 1 | reserved |  |  |  |  |  |  |  |
| 2 | tag |  |  |  |  |  |  |  |
| 3 | tag |  |  |  |  |  |  |  |
| 4 | link erp error count |  |  |  |  |  |  |  |
| 5 | link erp error count |  |  |  |  |  |  |  |
| 6 | a quota |  |  |  |  |  |  |  |
| 7 | b quota |  |  |  |  |  |  |  |
| 8 | eudc | reflect | mode |  | reserved |  |  |  |
| 9 | reserved |  |  |  |  |  |  |  |
| 10 | alarm threshold |  |  |  |  |  |  |  |
| 11 | alarm threshold |  |  |  |  |  |  |  |
| 12 | supported speed |  |  |  |  |  |  |  |
| 13 | supported speed |  |  |  |  |  |  |  |
| 14 | current speed |  |  |  |  |  |  |  |
| 15 | current speed |  |  |  |  |  |  |  |

Add the A Qutoa, B Quota, EUDC bit, REFLECT bit, & MODE bit  descriptions in clause 11.2.8,

The a quota field specifies the current frame A quota for the port set by the CONFIGURE PORT SMS for the selected port.

The B quota field specifies the current frame B quota for the port set by the CONFIGURE PORT SMS for the selected port.

The eudc field specifies the current setting of the eudc bit for the port set by the CONFIGURE PORT SMS for the selected port.

The reflect field specifies the current setting of the reflect flag for the node.

The mode field specifies the current port mode as defined in Table 37.

Table 37 - Current Mode field values

| Value | Description |
| --- | --- |
| 01b | Wrap mode. |
| 10b | Normal mode. |
| 11b | Privileged mode. |

Ed Carmona

Conner Peripherals, Inc.

Voice: (408) 456-4082

Email: ed.carmona@conner.com