Accredited Standards Committee
X3, Information Processing Systems

Doc: 	X3T10.1/96a126r0
Date:	February 29, 1996
Project:	X3T10.1 / 1145
Ref Doc.:	SSA-PH1 rev 8
Reply to:	Bill Ham, DEC

To: 	   	X3T10.1 Membership
From:	Bill Ham, DEC

Subject: Proposal to modify clause G.1 (wiring board design)

PROPOSAL

Replace Annex G clause G.1 with the folloing proposal:

Sincerely,

Bill Ham
EXT PH:	508 841-2629

- G.1  Guidelines for implementing printed circuit board design.

The terms used in this clause are defined as follows:

- Line path: The electrical conductor between the port connector and the termination circuitry, commonly the termination resistor (see Error: Reference source not found).

- Line + path: The line path for the + signal.

- Line - path: The line path for the - signal.

Stub: Any electrical path connecting to the line path but not part of the line path (See Figure G.2).

Figure G.1 - Line path definitions

The following guidelines should be followed when desiging printed circuit boards using SSA.

Figure G.2- Stub examples

- Isolate other active signals from the line path on the printed circuit board.

- Stubs should be minimized and not exceed 0,5 inch in length.

- Capacitance to ground on stubs should be minimized.

- Line + paths and line - paths should be laid out as 75 ohm single ended traces on one signal layer, preferably on top or bottom layers.

- Isolate SSA ports from each other.

Refer to Montrose, Mark I.,Printed Circuit Board Design Techniques for EMC compliance,  IEEE press 1996 (ISDN 0-7803-1131-0) for more details